Addendum to the

October 2022 Initial Environmental Examination

Project Number: 54373-001

August 2025

Maldives: Strengthening Gender Inclusive Initiatives Project

DV/GBV Shelter in North Male atoll, Hulhumale

Prepared by the Ministry of Social and Family Development of the Republic of Maldives for the Asian Development Bank.

ABBREVIATIONS

| ADB DDR | - - | Asian Development Bank Due Diligence Report |
| --- | --- | --- |
| GOM HDC | - - | Government of the Republic of Maldives Housing Development Corporation |
| IA | - | Implementing Agency |
| MOCHI | - | Ministry of Construction, Housing and Infrastructure |
| PMU MOFP MSFD FPA MBS MOE MOTE WDC | - - - - - - - - | Project Management Unit Ministry of Finance and Planning Ministry of Social and Family Development Family Protection Agency Maldives Bureau of Statistics Ministry of Education Ministry of Tourism and Environment Women’s Development Committee |

CONTENTS

I.	Introduction	4

II.	Location of the new DV/GBV site in North Male atoll, Hulhumale	4

III.	Description of Environment, Assessment of Potential Environmental Impacts and Environmental Management Plan	7

IV.	Meaningful Consultations for the new DV/GBV Shelter site	8

V.	Conclusion and Recommendations	8

APPENDICES

Appendix 1: Endorsement of allocation of site for DV/GBV Shelter by HDC	9

Appendix 2: Site Minutes and Site Visit Photos	12

Appendix 3: Rapid Environmental Assessment (REA) checklist	14

FIGURES

Figure 1 (a): Proposed Original location of the site of DV/GBV shelter at Hulhumale at Phase 1 development	5

Figure 1 (b):  Proposed New location of the site of DV/GBV Shelter at Hulhumale at  Phase 2 development	5

Figure 2: Proposed New Location with surrounding land use	6

Figure 3: Front Access Road Hithigas and some highrise buildings 300 m from the proposed new site	7

## Introduction

- During the project preparation in 2022, as the implementing agency of Output 5 (Climate-resilient shelters for DV/GBV survivors established) of the Strengthening Gender Inclusive Initiatives Project, the Ministry of Construction, Housing and Infrastructure, MOCHI, (formerly Ministry of National Planning, Housing and Infrastructure, MNPHI) of the Republic of Maldives prepared and submitted to the Asian Development Bank (ADB) a draft initial environmental examination (IEE) for the proposed domestic and gender-based violence (DV/GBV) shelter in North Male atoll, Hulhumale. After the ADB review mission in October 2024, it is agreed to implement Output 5 by the Project Management Unit (PMU) under Ministry of Social and Family Development (MSFD) as the lead implementing agency of the project. In order to support the implementation of construction of DV/GBV shelters under output 5 by PMU, it is decided to recruit Project Engineer, Construction Field Engineer and National (Environmental and Social) Safeguards Consultant under the PMU. Therefore, this addendum is prepared by PMU, MSFD.

- The Housing Development Corporation (HDC) identified and allocated a site for the DV/GBV shelter around the center of the island, for which the draft IEE was prepared. The size of the proposed site was 10,000 square feet and was located along the Banafsaa Hingun road, close to a stadium to the southern part.

- However, due to unavoidable circumstances, HDC changed the site for the construction of DV/GBV shelter at Hulhumale. On 14 October 2024, HDC allocated a new site for the proposed construction of the shelter. The new site is almost of the same area but slightly different in orientation. The proposed construction will maintain the same area and facilities as initially planned, with the only variation being adjustments to ventilation for improved air circulation.  A addendum to the concept design is prepared and subsequently, the bid document is revised accordingly. This addendum to the IEE is prepared to reflect the due diligence for the new site.

## Location of the new DV/GBV site in North Male atoll, Hulhumale

- The Hulhumalé DV/GBV Shelter will be constructed on currently unused and vacant land, owned by the Government of Maldives. The land plot has been transferred by the HDC to the MSFD. The land handover letter from HDC for the plot Lot 22241 for the Hulhumalé DV/GBV shelter is attached in the Appendix 1. No private land acquisition is therefore envisaged for the implementation of the proposed project components. The photographs of the location and site visit minutes are given in Appendix 2.

- Figure 1 (a) shows the location of the original proposed site and Figure 1 (b) shows the proposed new site and the surrounding area. The proposed new site is about 1.8 km away from the original proposed site and is situated in a newly developing area. The area is proposed to be developed mostly as a commercial land use area, where still no development has started yet within 300 m radius of the proposed site. The proposed plot of area is currently completely empty having approx. 10,011.6 square feet. At the front of the proposed site is the access road Hithiga, while the three adjoining sites are empty but are earmarked for construction of institutional building on the north side, office building on south side, and parking on the back side (Figure 2). Some high-rise buildings are located around 300 m from the proposed site (Figure 3).

### Figure 1 (a): Proposed Original location of the site of DV/GBV shelter at Hulhumale at Phase 1 development

### Figure 1 (b):  Proposed New location of the site of DV/GBV Shelter at Hulhumale at  Phase 2 development

### Figure 2: Proposed New Location with surrounding land use

### Figure 3: Front Access Road Hithigas and some highrise buildings 300 m from the proposed new site

| - Hithigas magu in front and some highrise building 300 m from the proposed new site (Google earth street view) | - Hithigas magu in front and some highrise building 300 m from the proposed new site |
| --- | --- |

## Description of Environment, Assessment of Potential Environmental Impacts and Environmental Management Plan

- The proposed new location of Hulhumalé DV/GBV Shelter was screened using ADB’s Rapid Environmental Assessment (REA) Checklist, Preliminary Climate Risk Screening Checklist and the Asbestos Screening Tool (Appendix 3). The proposed new site is located in the same island, North Male atoll. There is no vegetation at the site. The new site has similar environmental features as the original site and is not within nor adjacent to any environmentally sensitive areas such as nationally or locally protected areas. There is also no significant change in the project footprint (land requirement) and design. Based on this, no significant adverse environmental impact that is irreversible, diverse, or unprecedented is envisaged with the change in the DV/GBV site. Potential environmental impacts as identified in the October 2022 Draft IEE are the same for the new site, and the EMP remains applicable and sufficient.

## Meaningful Consultations for the new DV/GBV Shelter site

- Officials from HDC and the Project Management Unit (PMU) under the Ministry of Social and Family Development (MSFD), and ADB Mission visited the site on 13 October 2024. Prior to this, site visit was carried out by the International Project Management Specialist (ADB Consultant) along with HDC and PMU, MSFD on 1 October 2024, after finalization of the proposed new site.

- Consultations took place between ADB International Project Management Specialist (ADB Consultant), representatives of HDC, and PMU, MSFD during the site visit. During the ADB review mission in October 2024, ADB mission members, representatives of HDC, and PMU, MSFD consulted about site and construction of the shelter at the site. These consultations enabled the ADB, HDC, and MSFD to understand the condition of the site, and the willingness and enthusiasm of the HDC to establish the DV/GBV Shelter.

- The proposed new site is situated in a developing area and there is with no settlement within 300 m radius of the site. As the proposed new site is close to the previous site at Banafsaa Hingun, which is about 1.8 km distance, the previous stakeholder consultations conducted in June 2022 are considered sufficient at this stage. The IEE including this addendum will be updated based on the final detailed design of the contractor. Additional consultation will be ensured and conducted by the contractor, with the support of the PMU and national safeguards consultant, and the same will be reported to ADB through the QPR and/or updated IEE.

## Conclusion and Recommendations

- The proposed DV/GBV shelter in Hulhumale remains Category B for environment. The proposed new site is located in the same island, North Male atoll. There is no vegetation at the site. The new site has similar environmental features as the original site and is not within nor adjacent to any environmentally sensitive areas such as nationally or locally protected areas. There is also no significant change in the project footprint (land requirement) and design. Based on this, no significant adverse environmental impact that is irreversible, diverse, or unprecedented is envisaged with the change in the DV/GBV site. Potential environmental impacts as identified in the October 2022 Draft IEE are the same for the new site, and the EMP remains applicable and sufficient. The Draft IEE and EMP prepared in October 2022, and this Addendum shall be included in the bid and contract documents, updated based on the final detailed design of the contractor, and submitted to ADB for review and clearance prior to the start of construction.

- Additional consultation will be ensured and conducted by the contractor, with the support of the PIU and national safeguards consultant, and the same will be reported to ADB through the QPR and/or updated IEE.

### Appendix 1: Endorsement of allocation of site for DV/GBV Shelter by HDC

### Appendix 2: Site Minutes and Site Visit Photos

Site Visit Photos

| Proposed new site: unused land | Discussion during site visit ADB Consultant, PMU and HDC |
| --- | --- |
| Site visit during ADB mission | Site visit during ADB mission |

### Appendix 3: Rapid Environmental Assessment (REA) checklist

Instructions:

- The project team completes this checklist to support the environmental classification of a project. It is to be attached to the environmental categorization form and submitted to the Environment and Safeguards Division (RSES) for endorsement by Director, RSES and for approval by the Chief Compliance Officer.

- This checklist focuses on environmental issues and concerns. To ensure that social dimensions are adequately considered, refer also to ADB's (a) checklists on involuntary resettlement and Indigenous Peoples; (b) poverty reduction handbook; (c) staff guide to consultation and participation; and (d) gender checklists.

- Answer the questions assuming the “without mitigation” case. The purpose is to identify potential impacts. Use the “remarks” section to discuss any anticipated mitigation measures.

Instructions:

- The project team completes this checklist to support the environmental classification of a project. It is to be attached to the environmental categorization form and submitted to the Environment and Safeguards Division (RSES) for endorsement by Director, RSES and for approval by the Chief Compliance Officer.

- This checklist focuses on environmental issues and concerns. To ensure that social dimensions are adequately considered, refer also to ADB's (a) checklists on involuntary resettlement and Indigenous Peoples; (b) poverty reduction handbook; (c) staff guide to consultation and participation; and (d) gender checklists.

- Answer the questions assuming the “without mitigation” case. The purpose is to identify potential impacts. Use the “remarks” section to discuss any anticipated mitigation measures.

Strengthening Gender Inclusive Initiatives Project in the Maldives

Strengthening Gender Inclusive Initiatives Project in the Maldives

SARD/SAUW

SARD/SAUW

Country/Project Title:

Sector Division:

| Screening Questions | Yes | No |  | Remarks |
| --- | --- | --- | --- | --- |
| A. PROJECT SITING IS THE PROJECT AREA ADJACENT TO OR WITHIN ANY OF THE FOLLOWING AREAS: |  |  |  | The subproject site is situated in a newly developing area. The area is proposed to be developed as a commercial area, where still no development has yet started within 300 m radius. The proposed plot of area is currently completely empty having approx. 10,000 square feet. |
| UNDERGROUND UTILITIES | X |  |  | There are no underground utilities network within the subproject site; these are however, available in the immediate vicinity. The utility connections will run underneath the front road of the area. The road is just in front of the area. Utility network connections (such as for electricity, water supply, waste collection and sewerage) will be provided to the subproject site by the area Service Provider – Malè Water and Sewerage Company (MWSC) and State Electric Company Limited (STELCO), state owned enterprises and Waste Management Corporation Limited (WAMCO), a service provider. |
| CULTURAL HERITAGE SITE |  | X |  | Ocular inspection of the proposed subproject area showed no physical cultural heritage sites within or adjacent to the proposed subproject site. |
| PROTECTED AREA |  | X |  | There are no legally protected areas within the near vicinity of the proposed subproject site. |
| WETLAND |  |  | X |  |
| MANGROVE |  |  | X |  |
| ESTUARINE |  |  | X |  |
| BUFFER ZONE OF PROTECTED AREA |  |  | X |  |
| SPECIAL AREA FOR PROTECTING BIODIVERSITY |  |  | X |  |
| BAY |  |  | X | The subproject site is located inland; ~450m closest shoreline to the East. |
| B. POTENTIAL ENVIRONMENTAL IMPACTS WILL THE PROJECT CAUSE… |  |  |  |  |
| Encroachment on historical/cultural areas? |  |  | X | Ocular inspection of the proposed subproject area showed no physical cultural heritage sites within or adjacent to the proposed subproject site. EMP will specify Chance Find Procedures. |
| Encroachment on precious ecology (e.g., sensitive, or protected areas)? |  |  | X | No encroachment on precious ecology on or near the subproject site |
| Impacts on the sustainability of associated sanitation and solid waste disposal systems? | X |  |  | The construction and O&M of DV/GBV shelter will result in minor impact on the existing sewerage, sanitation and solid waste disposal systems since the shelter will service up to approx. 22 occupants at any given time. During construction stage, temporary sanitation (portable toilets), electricity connection to the nearest commercial meter, and waste collection area will be established by the contractor within the work site. The Contractor in coordination with area service provider – WAMCO and under the purview of Housing Development Corporation (HDC), Maldives will be responsible for waste collection, transportation and final disposal during construction stage. During O&M, the DV/GBV shelter will be serviced by the existing utility network (sewerage, water supply and electricity) in the wider area established by MWSC and STELCO. During O&M, the service provider - WAMCO will be responsible for waste collection, transport and final disposal; disposal will be in an existing waste management area located in Hulhumalè, approx. 1727 m distance west from the proposed subproject site, and later transported to Thilafushi on daily basis. These provisions will be specified in the EMP. The Project Implementation Unit (PIU) will obtain official acknowledgement letters from the service providers for provision of services with implementable timeline to the subproject site. |
| Dislocation or involuntary resettlement of people? |  |  | X |  |
| Disproportionate impacts on the poor, women and children, Indigenous Peoples or other vulnerable groups? |  |  | X |  |
| Increased noise and air pollution resulting from increased traffic volume? |  |  | X | There will be short term localized impacts on air, noise and dust during the construction stage; these can be effectively managed through good engineering construction practices and mitigation measures specified in the EMP. |
| Occupational and community health and safety risks? | X |  |  | Occupational health and safety will be a concern in particular: working at heights (G+2.5 building), work with construction materials, utilities, fuels, during construction. Appropriate safety measures will be specificized in the EMP including only trained workers assigned to such works and provision of PPE. Residential / working communities residing in and around the subproject site will be consulted throughout the project cycle and informed on community health and safety risks through community awareness programs, information disclosure / disclosure processes. The Community health and safety plan will be developed and implemented by the contractor and this will be specified in the EMP. |
| Risks and vulnerabilities related to occupational health and safety due to physical, chemical, biological, and radiological hazards during project construction and operation? | X |  |  | Occupational health and safety will be a concern in particular: working at heights (G+2.5 building), work with construction materials, work on utilities, fuels during construction phase. Appropriate safety measures will be specificized in the EMP including only trained workers assigned to such works and provision of PPE. At the DV/GBV shelter during O&M stage, for fire accidents, a fire extinguisher and dry sand buckets will be provided. This provision will be specified in the EMP. |
| Generation of dust in sensitive areas during construction? | X |  |  | There will be short term localized impacts on air and increase in dust during construction phase; these can be effectively managed through good engineering construction practices and mitigation measures specified in the EMP. |
| Requirements for disposal of fill, excavation, and/or spoil materials? | X |  |  | The water table in the area is approx. 2 m below Natural Ground Level (N.G.L). obtained at medium tide. Site may be backfilled / levelled to increase site elevation as well as the building elevation. Disposal of spoils will be undertaken as per Site specific EMP for Spoils Disposal developed and implemented by the contractor in coordination with the HDC and this provision will be specified in the EMP. |
| Noise and vibration due to blasting and other civil works? | X |  |  | Blasting is not anticipated. There will be short-term and site-specific noise and vibration impacts from construction activities. Construction activities will be prohibited at night and appropriate noise monitoring will be implemented. All nearby properties will be inspected (structural check) and site condition report will be developed by the contractor before civil works and based on the analysis/ observations temporary |
| Long-term impacts on groundwater flows as result of needing to drain the project site prior to construction? |  |  | X | Drainage of project site prior to construction is not anticipated. |
| Long-term impacts on local hydrology as a result of building hard surfaces in or near the building? | X |  |  | As per local stakeholder consultations including HDC, there are no issues with flooding in the area. The HDC is implementing stormwater drainage network in the area. Natural drainage pattern and site slope will be defined and maintained. The contractor / facility operator will also identify available water resources for construction and O&M of the DV/GBV shelter and usage will be estimated, approvals obtained from the relevant agencies, as required. During O&M stage, water usage will be minimal |
| Large population influx during project construction and operation that causes increased burden on social infrastructure and services (such as water supply and sanitation systems)? |  |  | X | The construction workforce is likely to come from the local area and local hiring will be given preference. The contractor will ensure adequate temporary infrastructure in the construction (workers) camp such that there is no burden on existing social infrastructure and services in the subproject area. These provisions will be specified in the |
| Social conflicts if workers from other regions or countries are hired? | X |  |  | Social conflicts are possible but unlikely as there will be a limited number of workers from other regions. Moreover, proper planning and management can avoid such social conflicts; worker staff will be sensitized with the local cultural |
| Risks to community safety caused by fire, electric shock, or failure of the building’s safety features during operation? | X |  |  | At the DV/GBV shelter during O&M stage, for fire accidents, a fire extinguisher, dry sand buckets and appropriate electrical signage will be provided. These provisions will be specified in the EMP |
| Risks to community health and safety caused by management and disposal of waste? |  |  | X | During construction stage, there will be provision of temporary waste collection area within the work site that will be developed and implemented by the contractor. The Contractor in coordination with the area service provider and under the purview of the HDC will be responsible for waste collection, transportation and final disposal during construction stage. During O&M, the service provider - WAMCO will be responsible for waste collection, transport and disposal. Waste disposal will be in an existing waste management area located approx. 1727 m distance from the proposed subproject site and this will be further transported to This is later transported to Thilafushi on daily basis. These provisions will be specified in the EMP. Sewerage waste will be discharged via a lateral connection from the new DV/GBV collection chamber to a main sewerage pipeline operated and maintained by service provider – MWSC. The Project Implementation Unit (PIU) will obtain official acknowledgement letters from the service providers for provision of services with implementable timeline to the subproject site. |
| Community safety risks due to both accidental and natural hazards, especially where the structural elements or components of the project are accessible to members of the affected community or where their failure could result in injury to the community throughout project construction, operation and decommissioning? | X |  |  | Residential communities residing close to the subproject area will be consulted throughout the project cycle and informed on community health and safety risks. The Community health and safety plan will be developed and implemented by the contractor and this provision will be specified in the EMP. |

Country/Project Title: Strengthening Gender Inclusive Initiatives Project in the Maldives

Sector: Public Sector Management

Subsector: Social Protection Initiatives

Division/Department: SARD/SAUW

Country/Project Title: Strengthening Gender Inclusive Initiatives Project in the Maldives

Sector: Public Sector Management

Subsector: Social Protection Initiatives

Division/Department: SARD/SAUW

A Checklist for Preliminary Climate Risk Screening

| Screening Questions |  | Score | Remarks |
| --- | --- | --- | --- |
| Location and Design of project | Is siting and/or routing of the project (or its components) likely to be affected by climate conditions including extreme weather-related events such as floods, droughts, storms, landslides? | 1 | Medium risk of potential for damage to building and harm to its users due to heavy precipitation and potential flooding. Detailed design to ensure proper drainage and site slope |
|  | Would the project design (e.g., the clearance for bridges) need to consider any hydro-meteorological parameters (e.g., sea-level, peak river flow, reliable water level, peak wind speed etc.,)? | 1 | Heavy precipitation has to be taken into account |
| Materials and Maintenance | Would weather, current and likely future climate conditions (e.g., prevailing humidity level, temperature contrast between hot summer days and cold winter days, exposure to wind and humidity hydro-meteorological parameters likely affect the selection of project inputs over the life of project outputs (e.g., construction material)? | 1 | Sea Level Rise SLR and flood mitigation has to be considered and design has to be adjusted to this increasing temperature and humidity will affect the living conditions in the proposed building. Recommended mitigation measures should take into consideration insulation of roof and external walls, energy efficient windows, reflective tiles for roof, or roof painted white, preserve existing trees, plant additional ones and vegetation for shading and cooling of site area energy efficient windows, reflective tiles for roof, or roof painted white, preserve existing trees, plant additional ones and vegetation for shading and cooling of site area |
|  | Would weather, current and likely future climate conditions, and related extreme events likely affect the maintenance (scheduling and cost) of project output(s)? | 0 | Flexibility in scheduling is proposed |
| Performance of project outputs | Would weather/climate conditions, and related extreme events likely affect the performance (e.g., annual power production) of project output(s) (e.g., hydro-power generation facilities) throughout their design life time? | 0 | Not relevant |

Options for answers and corresponding score are provided below:

| Response | Score |
| --- | --- |
| Not Likely | 0 |
| Likely | 1 |
| Very Likely | 2 |

- Responses when added that provide a score of 0 will be considered low risk project.

- If adding all responses will result to a score of 1-4 and that no score of 2 was given to any single response, the project will be assigned a medium risk category.

- A total score of 5 or more (which include providing a score of 1 in all responses) or a 2 in any single response, will be categorized as high-risk project.

- Result of Initial Screening (Low, Medium, High): Medium

Other Comments:

-----------------------------------------------------------------------------------------------------------------

Prepared by:  Hassan Nizdhy and Dr S M Atiqul Islam

Position: Architectural Consultant, PMU and International Project Management Specialist, ADB Consultant

ASBESTOS SCREENING TOOL

| Screening Questions | Yes* | Maybe* | No | Remarks *For those with answers of YES and MAYBE, document the potential likelihood of asbestos being encountered. |
| --- | --- | --- | --- | --- |
| Does the proposed project involve, or potentially involve, any of the following activities that are commonly associated with asbestos use: |  |  |  |  |
| • Construction/commissioning of a new asset? | X |  |  | The proposed project involves the construction of a Domestic Violence / Gender - Based Violence (DV/GBV) shelter in Hulhumalè. This new asset will require construction activities, including site preparation, foundation work, and building erection. Potential likelihood of asbestos is low, as the project is situated on a newly reclaimed land with no historical land use that would indicate the presence of asbestos. |
| • Refurbishment / demolition of an existing asset? |  |  | X | The project does not involve any refurbishment or demolition activities, as it is focused solely on the construction of a new facility. |
| • Post-disaster response, involving reconstruction, repair, or removal of damaged assets? |  |  | X | The project is not related to post-disaster activities, as it does not involve the reconstruction or repair of existing damaged assets. |
| • Maritime activities? |  |  | X | The project does not include any maritime activities, as it is located inland and does not interact with marine environments. |
| • Water supply, sanitation, wastewater, sewerage, or water hygiene initiatives? |  | X |  | The project will involve connections to local water supply and sewerage systems, which are essential for the operation of the DV/GBV shelter. This includes the installation of plumbing and sanitation facilities. The likelihood of Asbestosis low, as modern water and sewerage systems typically do not utilize asbestos materials, especially in newly developed areas. |
| • Earthworks, remedial activities, or solid waste management? | X |  |  | The project is likely to include earthworks as part of the construction phase, which will involve site preparation, excavation, and grading activities. The likelihood of Asbestos is low, given that the land is a new reclaimed area with no historical land use that would suggest the presence of asbestos. |
| • Power, telecommunications, or energy supply infrastructure? |  |  | X | The project does not involve the construction or commissioning of power or telecommunications infrastructure, as it is focused solely on the shelter. |
| • Maintenance, demolition, transportation, or disposal of wastes associated with the above activities? |  | X |  | There will be maintenance and waste disposal activities associated with the construction and operation of the shelter, particularly during the construction phase when waste materials will be generated. The potential likelihood of Asbestos: Low, if any waste materials from existing structures are inadvertently included; however, given the new reclaimed nature of the site, the likelihood remains low. Usually, modern water and sewerage systems typically do not utilize asbestos materials, especially in newly developed areas. |

Note: If you answered YES or MAYBE to the above questions, assume that the project is likely to encounter asbestos as a direct or indirect result of project-related activities and proceed to the toolkit for screening asbestos risks in new ADB-supported projects Part B – Screening Tools and Checklist

Strengthening Gender Inclusive Initiatives Project, Hulhumale Site, Draft IEE (October 2022).

Construction of a Domestic Violence / Gender Based Violence (DV/GBV) shelter under project output 5, Hulhumalè.

If possible, provide details on the sensitivity of project components to climate conditions, such as how climate parameters are considered in design standards for infrastructure components, how changes in key climate parameters and sea level might affect the siting/routing of project, the selection of construction material and/or scheduling, performances and/or the maintenance cost/scheduling of project outputs.