ADDENDUM FOR THE DESIGN OF SHELTERS

SUBJECT: Clarifications to DV/GBV Shelter Conceptual Design Documents

DATE: 08th December 2025

PURPOSE:

This document clarifies specific design features outlined in the conceptual design documents that must be considered when preparing price quotations. When submitting bids, it is important to note that the proposed changes aim to achieve two primary objectives:

    - Reducing overall construction costs and ensuring that the design aligns more closely with local cultural values. Adjustments should focus on incorporating local and sustainable practices and materials, which not only lower ongoing operations and maintenance expenses but also enhance the aesthetic appeal of the DV/GBV shelters.

    - Ensuring that the design reflects Maldivian architectural styles and community preferences, supporting integration with the local community and cultural context.

All proposed changes will be subject to employer’s approval as part of detailed design milestone.

CLARIFICATION POINTS:

- Contextual Adaptation:

- Modifications will be made to the conceptual design to ensure that the final design is reflective of the local island context. This includes considering local architectural styles, local materials or building methods, cultural practices, passive building design methodologies and environmental conditions that influence shelter design in the Maldives.

- For interior space management, the Contractor shall maintain the overall uses and facilities specified in the Specific Requirements of the DV shelter in the bid documents and concept design. However, the Contractor may redesign individual components, considering Maldivian practices, to create a more localized design approach. Specifications of materials shall be scaled down to moderate standards, rather than luxurious ones, to ensure cost-effectiveness. This is subject to prior approval of the Employer.

- The design shall be climate-resilient; where necessary, buildings shall be elevated above ground level.

- All designs shall comply with the Building Act and relevant Maldivian standards and regulations.

- Material Selection:

All materials used in the construction of the shelter must be commonly available and widely used in the Maldives, ensuring suitability for the local climate and ease of future maintenance or replacement.

- Privacy Screening Solutions:

The proposed privacy screen component will be reimagined in the final design stage to ensure alignment with project goals. Key considerations for the redesign include:

- Cost-Effectiveness: Solutions should be economical to build and maintain, ensuring sustainability.

- Aesthetic Appeal: Designs must be visually pleasing and appropriate for the local context of the Maldives, blending with the surrounding environment.

- Cultural Integration: Incorporate traditional design elements that reflect local architecture and cultural values.

- Use of Local Vegetation: Utilize native plants to provide privacy while enhancing the shelter's visual appeal and ecological compatibility.

- Alignment with Project Objectives: Ensure that changes support the overall goals of the DV/GBV shelter, maintaining or enhancing the functionality of privacy screens for survivors, thereby creating a safe and welcoming environment.

- Courtyards - (Reading Court, Green Spill Over Area, Connecting Court, Interactive Court)

The courtyard concepts in the design package will be adapted to reflect Maldivian traditions while ensuring practicality, safety, and comfort. Key considerations include:

- Ground Finishes: Use white coral sand as the primary surface, with paving stones, clay tiles, or bricks in circulation areas. Avoid excessive use of plain concrete.

- Seating: Provide a mix of standard benches, joali fathi (traditional rope chairs), and roped swings, positioned in shaded or semi-shaded areas to improve comfort and usability.

- Shading Structures: Pergolas and awnings should be simplified or redesigned with locally available materials. Omit where not required to minimize maintenance.

- Integration with Surroundings: Courtyards must connect clearly with adjacent buildings, allowing natural light, ventilation, safe movement, and fostering social interaction and security.

- Boundary Wall

The boundary wall for the DV/GBV shelters will be constructed to a minimum height of 2 meters above ground level, providing essential privacy and security for residents. The design should balance protective function with cultural and community-friendly elements.

Key features include:

- Height: Minimum of 2 meters to ensure privacy and deter unauthorized access.

- Design Elements: Incorporation of local artwork and decorative motifs to reflect Maldivian culture and add visual appeal.

- Landscaping: Use of native plants around the wall to soften its appearance and promote a community-friendly environment.

- Materials: Constructed as a reinforced concrete structural system with solid block masonry infill.

- Additional Considerations

The following requirements shall be considered and incorporated into the detailed design and during the construction phase:

- Provide separate toilets, bathrooms and shower facilities for women and men, ensuring privacy and dignity in line with DV/GBV shelter requirements.

- Incorporate adequate thermal insulation and suitable finishes to roofs and ceilings to minimize heat gain and improve indoor comfort.

- Ensure the roof form and slope are designed considering prevailing wind direction and wind loads to enhance structural stability and durability.

- Include provisions for future installation of solar photovoltaic panels, such as reserved roof area, structural load capacity, cable routes and space for inverters and control equipment.

PREPARED BY:

PMU, MSFD, Maldives