ANNEX1: Minimum requirement for the Maldives National Health Laboratory

- General Information

  - Location: Maldives, Kaafu Atoll, Hulhumale’ Phase 2

  - Land Area: 1263.11 sqm

  - Site Coverage: 0.7

  - Foot Print: 994.18 sqm

  - Proposed floors: 9 floors and basement

- The document outlines the minimum requirements for the development of the Maldives National Health Laboratory. During the design process, the consultancy firm must incorporate best practices, adhere to recent guidelines for design and construction of the laboratory, increase or decrease the size of rooms, add or remove necessary services/rooms, equipment and furniture, and make any essential changes to the building to fit the available space. The building should be designed with ample space to facilitate the smooth movement of people and goods, ensuring efficient operations and comfort for all occupants.

- The building shall be constructed using the Reinforced Concrete (RC) and permanent masonry walls, ensuring durability, strength, and compliance with modern construction standards.

- The HVAC system must be designed to maintain appropriate air quality, temperature, humidity, and containment. Certain areas should have negative air pressure, airlocks, and HEPA systems, as well as differential zoning and filtration systems, to guarantee safe and efficient operations. The consultant firm should provide guidance and implement any required adjustments to ensure that the HVAC system is designed and installed in accordance with international laboratory standards.

- Electric sockets, Internet connection ports, phone line connection ports should be installed in all rooms.

- The design process should be according to the Terms of Reference (ToR) for the Consultancy for the Architectural, Structural, Services, Drawings, BoQs, Technical Specification and Supervision of the construction of the National Health Laboratory.

- Proposed NHL Building Structure

| Floors | Terrace | Catering area, Recreational area, HAVC system outdoor units, Water tanks and Solar panels | Facilities |
| --- | --- | --- | --- |
|  | Ninth Floor | Virology - Grade 3 Virus Laboratory and Staff |  |
|  | Eight Floor | Virology - Grade 2 Virus Laboratory and Staff |  |
|  | Seventh Floor | Clinical Microbiology Laboratory and Staff |  |
|  | Sixth Floor | Food and Pharmaceutical Microbiology Laboratory and Microbiology Section Staff |  |
|  | Fifth Floor | Food and Pharmaceutical Chemistry Laboratory and Chemistry Section Staff |  |
|  | Fourth Floor | National Health Laboratory Division and Space for Development |  |
|  | Third Floor | Maldives Therapeutic Goods Division and Inspectorate Division |  |
|  | Second Floor | Administration and Support Service Section and Food Control Division |  |
|  | First Floor | Multipurpose hall, Gym, Recreational area and Childcare Center |  |
|  | Ground Floor | Receptions, Storage and Parking |  |
|  | Basement | Parking |  |

8.Basement

| # | Room/Space | Estimated Size (sqft) | Room Type | Function | Non-Medical Items | Medical Items | IT Items | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Car Parking | 2500 |  | To park 10 cars |  |  | - CCTV camera |  |
| 2 | Cycle Parking | 4600 |  | To park 300 cycle |  |  | - CCTV camera |  |
| 3 | Ramp |  |  | For vehicle movement |  |  |  |  |

9.Ground Floor

| # | Room/Space | Estimated Size (sqft) | Room Type | Function | Non-Medical Items | Medical Items | IT Items | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Portico |  |  |  |  |  |  |  |
| 2 | Main reception and waiting area | 500 | Open | Operate as the main entry to the building and to provide customer services | - Main Counter - 05 Staff chairs for reception staff - 05 Customer chairs for the reception - 20 Customer waiting chairs - Racks - 04 Dustbin - 01 Clock - Decorative items/Plants |  | - 05 Computer systems - 05 Phones - 03 Printers - 01 Queue system with display - 75inch TV to show information |  |
| 3 | Call center | 200 | Closed | Call center for the MFDA and NHL | - Call center work stations - 02 Chairs - 01 Dustbin - 01 Clock - Decorative items/Plants |  | - 02 Computer system - 02 Phones - 01 Printer |  |
| 4 | Space for reception and call center head | 120 | Half Closed | Oversee reception and call center works | - 01 Table - 01 Chair for the staff - 02 Chairs in front of the table - 01 Dustbin - 01 Clock |  | - 01 Computer system - 01 Phone - 01 Printer |  |
| 5 | Staff working area | 240 | Open | Space for the staff to work - 3 Staff | - 05 Table (open concept) - 05 Chair - 01 Table for printer |  |  |  |
| 6 | Meeting room | 250 | Closed | To conduct small meetings | - 01 Meeting Table - 10 Chairs - 01 Table for printer - 01 Tea setup table |  | - 01 75inch TV - 01Computer system - 01 Three in one printer - 01 Phone |  |
| 7 | Food and pharmaceutical sample receiving reception and customer waiting area | 300 | Closed | Specifically focused on food and pharmaceutical sample receiving | - 01 Counter - 02 Staff chairs - 02 Customer chairs for the reception - 10 Customer waiting chairs - Racks - 02 Dustbin - 01 Clock - Decorative items/Plants |  | - 02 Computer system - 02 Printers and - 02 Phones - 01 Queue system with display - 75inch TV to show information |  |
| 8 | Sample sorting and masking room | 160 | Closed | Space for sample sorting and masking | - Work desk with drawers - Racks |  | - Barcode printer (sample labeling) |  |
| 9 | Sample storage room for food and pharmaceuticals | 120 | Closed | To store receiving samples until it gets delivered to the specific lab | - Work desk with drawers - Racks | - 01 Refrigerator | - 01 Computer system - 01 Printer |  |
| 10 | Infectious disease reception and customer waiting area | 300 | Closed | Specifically focused on infectious sample receiving | - 01 Counter - 02 Staff chairs - 02 Customer chairs for the reception - 10 Customer waiting chairs - Racks - 02 Dustbin - 01 Clock - Decorative items/plants |  | - 02 Computer system - 02 Printers and - 02 Phones - 01 Queue system with display - 75inch TV to show information |  |
| 11 | Sample collection area | 250 | Closed | Space to collect samples | - 02 Chairs for the staff - 02 Chairs for the customers | - 01 Hospital Bed with bed curtains - 02 Blood collection counters - 01 Refrigerator | - 02 Computer system - 02 Printers - 01 Phone |  |
| 12 | Sample sorting and masking room | 160 | Closed | Space for sample sorting and masking | - Work desk with drawers - Racks |  | - Barcode printer (sample labeling) |  |
| 13 | Sample storage room for infectious disease samples | 160 | Closed | To store receiving samples until it gets delivered to the specific lab | - Work desk with drawers - Racks | - 01 Refrigerator | - 01 Computer system - 01 Printer |  |
| 14 | Chemical/Consumable store 1: Ambient | 250 | Closed | To store chemicals that should be kept in ambient temperature | - Racks - Work desk |  |  | Should be accessible to an open area |
| 15 | Chemical/Consumable store 2: Cold | 250 | Closed | To store chemicals that should be kept in low temperature |  | - 01 Freezer (2-8 Degree Celsius - 01 Freezer (-20 Degree Celsius) - 01 Freezer (-80 Degree Celsius) |  |  |
| 16 | General Store: Stock Items | 600 | Closed | To store the large bulk of goods received | - Racks - Work desk - 01 Chair |  | - 01 Computer system - 01 Printer - 01 Phone |  |
| 17 | General Store 02: Damaged and Auction Goods | 600 | Closed | To keep damaged goods until it gets auctioned or thrown | - Racks - Work desk - 01 Chair |  |  |  |
| 18 | Non-medical items repair & maintenance room with a small store inside | 400 | Closed | To conduct repair works of non-medical items | - Racks - Work bench - 02 Chairs |  |  |  |
| 19 | Bio-medical repair workshop with a small store inside | 400 | Closed | To conduct repair works of medical items | - Racks - Work bench - 01 Table for printer - 02 Chairs |  | - 02 Computer system - 01 Three in one printer - 01 Phone |  |
| 20 | Waste Management (General waste) | 150 | Closed | To keep general waste till it get discarded and thrown | - Waste bins - Racks |  | - 01 Phone |  |
| 21 | Waste Management (Bio-hazard) | 150 | Closed | To keep bio waste till it get discarded and thrown | - Waste bins - Racks | - 01 Incinerator or shredder | - 01 Phone |  |
| 22 | Parking space for waste vehicles |  | Open | Space for the vehicle to load waste |  |  |  |  |
| 23 | Car Parking | 480 |  | 2 Car parking |  |  |  |  |
| 24 | Support staff area | 160 | Closed | Area for the support staff to rest and work | - 01 Table - 02 Chairs - 01 Rack |  | - 01 Phone |  |
| 25 | Pantry | 180 | Closed | Area for the staff to eat and drink | - 01 Dining table - 06 Chairs - Kitchen cabinets - 01 Sink - 01Refrigerator |  | - 01 Phone |  |
| 26 | Guard room | 160 | Closed | To supervise and monitor the building security | - 01 Work desk with drawers - 02 chairs - Storage Cabinets |  | - 02 55inch TVs for CCTV footage - 01 Computer system - 01 Phone - Fire calling system - Public calling system |  |
| 27 | Drivers Room | 160 | Closed | Drivers workspace | - 01 Work desk with drawers - 02 Chairs - Storage Cabinets |  | - 01 55inch TV, - 01 Computer system - 01 Phone |  |
| 28 | Generator Room | 250 | Closed | To keep the backup generator | - 01 Generator - Fuel storage - Racks |  |  |  |
| 29 | Panel Board Room | 150 | Closed | To keep the panel boards |  |  |  |  |
| 30 | Toilet sink area for male | 45 |  |  | - 01 Mirror - 02 Sink with tap - 01 Tissue dispenser - 01 Hand dryer - 01 Dustbin |  |  |  |
| 31 | Toilet 1 (Staff and Customer use) - male use | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder |  |  |  |
| 32 | Toilet 2 (Staff and Customer use) - male use | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder |  |  |  |
| 33 | Toilet sink area for female | 45 |  |  | - 01 Mirror - 02 Sink with tap - 01 Tissue dispenser - 01 Hand dryer - 01 Dustbin |  |  |  |
| 34 | Toilet 3 (Staff and Customer use) - female | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder |  |  |  |
| 35 | Toilet 4 (Staff and Customer use) - female | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder |  |  |  |
| 36 | Lift 1 (Employees and customers) | 60 |  | Customer and staff use (access controlled for upper floors) |  |  |  | 14 Pax |
| 37 | Lift 2 (Employees and customers) | 60 |  | Customer and staff use (access controlled for upper floors) |  |  |  | 14 Pax |
| 38 | Lift 3 (Food and Pharmaceutical) | 45 |  | Only for food and pharmaceutical lab staffs and to transfer samples (access controlled) |  |  |  | 11 Pax |
| 39 | Lift 4 (Infectious Disease) | 45 |  | Only for Infectious disease lab staffs (access controlled) |  |  |  | 11 Pax |
| 40 | Lift 5 (Dumbwaiter lift) | 20 |  | To transfer samples to infectious disease labs (access to only bacteriology & virology labs) |  |  |  | 100 Kg lift weight |
| 41 | Lift 6 (Heavy Duty Lift) | 80 |  | Only for heavy machineries and for garbage (access controlled) |  |  |  | 2000 Kg lift weight |
| 42 | Corridor with emergency shower |  |  | Walkways and Showers for emergency use | - 02 Shower sets - 01 Rack | - 03 Eye wash kit - 01 First aid kit - 02 Spill kit | - 03 Phones |  |
| 43 | Staircase |  |  | Normal use - Provide an access from one floor to another |  |  |  |  |
| 44 | Fire-exit with fire exit doors |  |  | Emergency use: access to go out of the building in case of fire/emergency |  |  |  |  |

10. First Floor

| # | Room/Space | Estimated Size (sqft) | Room Type | Function | Non-Medical Items | Medical Items | IT Items | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Multipurpose hall | 2500 | Closed | To host a wide range of events and meetings | - 60 Chairs - 20 foldable Tables - 01 Podium |  | - 01Led screen - Sound system and Microphones - Lighting System - Video Conferencing system - 01 Laptop - 01 Phone |  |
| 2 | Multipurpose hall - Control room | 200 | Half Closed | Control the multipurpose hall IT setup | - Work bench - Racks |  | - Control systems of sound, light and screen |  |
| 3 | Multipurpose hall - Store room | 200 | Closed | To store Multipurpose hall items (chairs and tables) | - Racks |  |  |  |
| 4 | Multipurpose hall - catering area - | 200 | Closed | To prepare the catering setup and washing area for plates, etc. | - 02 Sink - Kitchen cabinets with a table top. |  |  |  |
| 5 | Small Conference room - Multipurpose hall | 250 | Closed | To conduct discussions | - Conference table - 10 Conference Chairs |  |  |  |
| 6 | Toilet sink area for male - Multipurpose hall | 45 | Closed |  | - 01 Mirror - 01 Sink with tap - 01 Tissue dispenser - 01 Hand dryer - 01 Dustbin |  |  |  |
| 7 | Toilet 1 (Staff and Customer use) - male use - Multipurpose Hall | 35 | Closed |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder |  |  |  |
| 8 | Toilet sink area for female - Multipurpose Hall | 45 | Closed |  | - 01 Mirror - 01 Sink with tap - 01 Tissue dispenser - 01 Hand dryer - 01 Dustbin |  |  |  |
| 9 | Toilet 2 (Staff and Customer use) - male use - Multipurpose Hall | 35 | Closed |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder |  |  |  |
| 10 | Child care center | 200 | Closed | Designed to provide a safe, nurturing environment offering educational, social, and physical activities for young children | - Child-sized tables and chairs - Storage bins and shelves - Books - Educational toys - Art supplies - Play mats |  | - Security Cameras - Phone |  |
| 11 | Lounges for waiting | 250 | Closed | Visitors to comfortably wait for appointments/ meetings. | - Comfortable chairs or sofas - 01 Coffee table and side tables - Rug - 01 Mini-fridge - Decorative items |  |  |  |
| 12 | Recreational area | 2000 | Closed | Space where people can engage in various activities that promote physical well-being, mental relaxation, and social engagement. | - Shock-absorbing futsal flooring (synthetic futsal turf). (3 on 3 futsal ground). - Mesh netting around the perimeter. - Players' benches & officials’ area - Spectator seating (if space allows) |  |  |  |
| 13 | Recreational area store | 180 | Closed | To store items related to Recreational activities | - Racks |  |  |  |
| 14 | Recreational office | 200 | Closed | To manage office recreational activities | - 02 Table (open concept) - 02 Chair - 01 Table for printer |  | - Phone |  |
| 15 | Gym | 500 | Closed | Designed to support physical exercise and fitness training | - 01 Counter - 02 chairs - Dumbbells - Barbells - Kettlebells - Leg Press Machines - Chest Press Machines - Squat Racks - Treadmills - Stationary Bikes - Yoga Mats - Foam Rollers - Exercise Balls - Boxing Equipment - Lockers - Mirrors |  | - Sound System - 01 Computer system |  |
| 16 | Change room – male | 35 | Closed | To change into workout/regular clothes | - Mirrors - Cloth hangers |  |  |  |
| 17 | Change room – female | 35 | Closed | To change into workout/regular clothes | - Mirrors - Cloth hangers |  |  |  |
| 18 | Toilet sink area for male - gym | 45 | Closed |  | - 01 Mirror - 01 Sink with tap - 01 Tissue dispenser - 01 Hand dryer - 01 Dustbin |  |  |  |
| 19 | Gym toilet – male | 35 | Closed |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder |  |  |  |
| 20 | Gym toilet – male shower area | 35 | Closed | To take showers after the workout | - Shower set - Cloth hanger |  |  |  |
| 21 | Toilet sink area for female - gym | 45 | Closed |  | - 01 Mirror - 01 Sink with tap - 01 Tissue dispenser - 01 Hand dryer - 01 Dustbin |  |  |  |
| 22 | Gym toilet – female | 35 | Closed |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder |  |  |  |
| 23 | Gym toilet – female shower area | 35 | Closed | To take showers after the workout | - Shower set - Cloth hanger |  |  |  |
| 24 | Accommodation room 1 with attached toilet | 250 | Closed | To provide inhouse accommodation for officials and staff | - 01 Single Bed - 01 Closet - 01 Dresser with mirror - 01 Desk and Chair - Mini-Fridge - 01 Mirror - 01 Sink with tap - 01 Tissue dispenser - 01 Hand dryer - 01 Dustbin - 01 Toilet set - Shower set - 01 Hand shower - 01 Toilet paper holder |  |  |  |
| 25 | Accommodation room 2 with attached toilet | 250 | Closed | To provide inhouse accommodation for officials and staff | - 01 Single Bed - 01 Closet - 01 Dresser with mirror - 01 Desk and Chair - Mini-Fridge - 01 Mirror - 01 Sink with tap - 01 Tissue dispenser - 01 Hand dryer - 01 Dustbin - 01 Toilet set - Shower set - 01 Hand shower - 01 Toilet paper holder |  |  |  |
| 26 | Utility Room | 150 | Closed | To keep cleaning items | - Racks |  |  |  |
| 27 | Prayer room with ablution area (men) | 350 | Closed | Dedicated space where staff can go to pray | - Racks |  |  |  |
| 28 | Prayer room with ablution area (women) | 350 | Closed | Dedicated space where staff can go to pray | - Racks |  |  |  |
| 29 | Support staff area | 160 | Closed concept | Area for the support staff to rest and work | - 01 Table - 02 chairs - 01 Rack |  | - 01 Phone |  |
| 30 | Pantry | 180 | Closed concept | Area for the staff to eat and drink | - 01 Dining table - 06 chairs - Kitchen cabinets - 01 Sink - 01 Refrigerator |  | - 01 Phone |  |
| 31 | Network rack space | 50 |  | To mount the network switches of each floor |  |  |  |  |
| 32 | Lift 1 (Employees and customers) | 60 |  | Customer and staff use (access controlled for upper floors) |  |  |  | 14 Pax |
| 33 | Lift 2 (Employees and customers) | 60 |  | Customer and staff use (access controlled for upper floors) |  |  |  | 14 Pax |
| 34 | Lift 3 (Food and Pharmaceutical) | 45 |  | Only for food and pharmaceutical lab staffs (access controlled) |  |  |  | 11 Pax |
| 35 | Lift 4 (Infectious Disease) | 45 |  | Only for Infectious Disease lab staffs (access controlled) |  |  |  | 11 Pax |
| 36 | Lift 6 (Heavy Duty Lift) | 80 |  | Only for heavy machineries and for garbage (access controlled) |  |  |  | 2000 Kg lift weight |
| 37 | Corridor |  |  | Walkways and Showers for emergency use |  |  |  |  |
| 38 | Staircase |  |  | Normal use - provide an access from one floor to another |  |  |  |  |
| 39 | Fire-exit with fire exit doors |  |  | Emergency use: access to go out of the building in case of fire |  |  |  |  |

11.Second Floor

| # | Room/Space | Estimated Size (sqft) | Room Type | Function | Non-Medical Items | Medical Items | IT Items | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | MFDA Political Appointee Room | 180 | Closed | Space for the Political appointee of MFDA | - 01 Desk - 01 Chair for the executive - 02 Chairs in front of the table - 01 Filing rack - 01 Dustbin - 01 Sofa set - 01 Coffee table - 01 Rug - Artwork, Plant or other decorative items to create a pleasant environment |  | - 01 Computer System - 01 Phone - 01 Three in one printer |  |
| 2 | MFDA Director General Room | 180 | Closed | Space for the Director General of MFDA | - 01 Desk - 01 Chair for the executive - 02 Chairs in front of the table - 01 Filing rack - 01 Dustbin - 01 Sofa set - 01 Coffee table - 01 Rug - Artwork, Plant or other decorative items to create a pleasant environment |  | - 01 Computer System - 01 Phone - 01 Three in one printer |  |
| 3 | Administration and Support Service Section |  |  |  |  |  |  |  |
| 4 | Administration and Support Service Section - Director | 150 | Half closed | 01 Staff (EX1) | - 01 Desk - 01 Chair for the executive - 02 Chairs in front of the table - 01 Filing rack - 01 Dustbin - Artwork, Plant or other decorative items to create a pleasant environment |  | - 01 Phone |  |
| 5 | Administration and Support Service Unit - Assistant Director | 120 | Open | 01 Staff (MS3) | - 01 Desk - 01 Chair for the executive - 02 chair in front of the table - 01 Filing cupboard - 01 Dustbin |  | - 01 Phone |  |
| 6 | Administration and Support Service Unit - Staff | 720 | Open | 09 Staff (GS2 to MS2 level) | - 09 Desks with dividers (open concept) - 09 Chairs - 03 Filing rack - 01 Desk for printer setup - 04 Dustbin - 01 Clock |  | - 03 Phone - 01 Three in one Printer |  |
| 7 | Human Resource Unit |  |  |  |  |  |  |  |
| 8 | Human Resource Unit - Senior Human Resource Officer | 120 | Open | 01 Staff (MS3) | - 01 Desk - 01 Chair for the executive - 02 Chair in front of the table - 01 Filing cupboard - 01 Dustbin |  | - 01 Phone |  |
| 9 | Human Resource Unit - Staff | 160 | Open | 02 Staff (MS2) | - 02 Desks with dividers (open concept) - 02 Chairs - 01 Filing rack - 01 Desk for printer setup - 01 Dustbin - 01 Clock |  | - 02 Phone |  |
| 10 | Legal Unit (Closed Room) |  |  |  |  |  |  |  |
| 11 | Legal Unit - Senior Legal Officer | 120 | Open | 01 Staff (CS11) | - 01 Desk - 01 Chair for the executive - 02 Chair in front of the table - 01 Filing cupboard - 01 Dustbin |  |  |  |
| 12 | Legal Unit - Staff | 160 | Open | 02 Staff (CS9 to CS7) | - 02 Desks with dividers (open concept) - 02 Chairs - 01 Filing rack - 01 Desk for printer setup - 01 Dustbin - 01 Clock |  |  |  |
| 13 | Budget and Finance Unit |  |  |  |  |  |  |  |
| 14 | Budget and Finance Unit - Senior Accounts Officer | 120 | Open | 01 Staff (MS3) | - 01 Desk - 01 Chair for the executive - 02 Chair in front of the table - 01 Filing cupboard - 01 Dustbin |  |  |  |
| 15 | Budget and Finance Unit - Staff | 160 | Open | 02 Staff (MS2) | - 02 Desks with dividers (open concept) - 02 Chairs - 01 Filing rack - 01 Desk for printer setup - 01 Dustbin - 01 Clock |  |  |  |
| 16 | Procurement and Logistics Unit |  |  |  |  |  |  |  |
| 17 | Procurement and Logistics Unit - Assistant Director | 120 | Open | 01 Staff (MS3) | - 01 Desk - 01 Chair for the executive - 02 Chair in front of the table - 01 Filing cupboard - 01 Dustbin |  |  |  |
| 18 | Procurement and Logistics Unit - Staff | 160 | Open | 02 Staff (MS2) | - 02 Desks with dividers (open concept) - 02 Chairs - 01 Filing rack - 01 Desk for printer setup - 01 Dustbin - 01 Clock |  |  |  |
| 19 | Procurement and Logistics Unit - Small storage for bids | 180 | Closed | To store bid files (ongoing and recent works) | - Filing racks |  |  |  |
| 20 | Information Technology Unit |  |  |  |  |  |  |  |
| 21 | Information Technology Unit - Senior Computer Programmer | 120 | Open | 01 Staff (MS3) | - 01 Desk - 01 Chair for the executive - 02 Chair in front of the table - 01 Filing cupboard - 01 Dustbin |  |  |  |
| 22 | Information Technology Unit - Staff | 160 |  | 2 Staff (MS2) | - 02 Desks with dividers (open concept) - 02 Chairs - 01 Filing rack - 01 Desk for printer setup - 01 Dustbin |  |  |  |
| 23 | Information Technology Unit - Workshop with Storage | 200 | Closed | To repair IT hardware and store IT hardware (small quantity) | - Work desk with electric sockets to repair/test PCs - Racks for storage |  |  |  |
| 24 | Server Room | 250 | Closed | To keep Servers, UPS etc. | - Racks |  | - 01 UPS rack - 01 Network rack (Including phone) - 01 Server rack - 04 20 core dual processes 256 GB RAM. - 02 50 TB useable (02) - 01 30 KVA UPA. - 01 Network rack (fiber switch layer 03, 24 port.) SFP+ - 02 Network core switch layer 02 48 port. |  |
| 25 | Small Meeting space for Administration and Support Service Section | 180 | Open | To conduct small urgent discussions. Space 6 people | - 01 Meeting table - 06 chairs |  |  |  |
| 26 | Food Control Division |  |  |  |  |  |  |  |
| 27 | Food Control Division - Lead Food Quality Assurance Officer | 160 | Closed | 01 Staff (PSR9) | - 01 Desk - 01 Chair for the executive - 02 Chairs in front of the table - 01 Filing rack - 01 Dustbin - Artwork, Plant or other decorative items to create a pleasant |  | - 01 Phone |  |
| 28 | Food Regulatory Programs Section |  |  |  |  |  |  |  |
| 29 | Food Regulatory Programs Section - Food Quality Assurance Coordinator | 150 | Half closed | 01 Staff (PSR8) | - 01 Desk - 01 Chair for the executive - 02 Chairs in front of the table - 01 Filing rack - 01 Dustbin - Artwork, Plant or other decorative items to create a pleasant |  | - 01 Phone |  |
| 30 | Regulatory Programs Unit |  |  |  |  |  |  |  |
| 31 | Regulatory Programs Unit - Food Quality Assurance Coordinator | 120 | Open | 01 Staff (PSR8) | - 01 Desk - 01 Chair for the executive and - 02 Chair in front of the table - 01 Filing cupboard - 01 Dustbin |  | - 01 Phone |  |
| 32 | Regulatory Programs Unit - Staff | 480 | Open | 06 Staff (PSR7) | - 06 Desks with dividers (open concept) - 06 Chairs - 01 Filing rack - 01 Desk for printer setup - 03 Dustbins - 01 Clock |  | - 03 Phone - 01 Three in one Printer |  |
| 33 | Risk Analysis and Product Registration Unit |  |  |  |  |  |  |  |
| 34 | Risk Analysis and Product Registration Unit - Food Quality Assurance Coordinator | 120 | Open | 01 Staff (PSR8) | - 01 Desk - 01 Chair for the executive and - 02 chair in front of the table - 01 Filing cupboard - 01 Dustbin |  | - 01 Phone |  |
| 35 | Risk Analysis and Product Registration Unit - Staff | 560 | Open | 07 Staff (PSR4 to PSR7) | - 07 Desks with dividers (open concept) - 07 Chairs - 02 Filing rack - 01 Desk for printer setup - 04 Dustbins - 01 Clock |  | - 04 Phone - 01 Three in one Printer |  |
| 36 | Quality Assurance Section |  |  |  |  |  |  |  |
| 37 | Quality Assurance Section - Food Quality Assurance Coordinator | 150 | Half closed | 01 Staff (PSR8) | - 01 Desk - 01 Chair for the executive - 02 Chairs in front of the table - 01 Filing rack - 01 Dustbin - Artwork, Plant or other decorative items to create a pleasant |  | - 01 Phone |  |
| 38 | Official Control Unit |  |  |  |  |  |  |  |
| 39 | Official Control Unit - Food Quality Assurance Coordinator | 120 | Open | 01 Staff (PSR8) | - 01 Desk - 01 Chair for the executive and - 02 Chair in front of the table - 01 Filing cupboard - 01 Dustbin |  | - 01 Phone |  |
| 40 | Official Control Unit - Staff | 1040 | Open | 13 Staff (PSR 4 to PSR7) | - 13 Desks with dividers (open concept) - 13 Chairs - 03 Filing rack - 01 Desk for printer setup - 06 Dustbins - 01 Clock |  | - 06 Phone - 01 Three in one Printer |  |
| 41 | Health Certification Unit |  |  |  |  |  |  |  |
| 42 | Health Certification Unit - Food Quality Assurance Coordinator | 120 | Open | 01 Staff (PSR8) | - 01 Desk - 01 Chair for the executive and - 02 Chair in front of the table - 01 Filing cupboard - 01 Dustbin - 01 Clock |  | - 01 Phone |  |
| 43 | Health Certification Unit - Staff | 560 | Open | 07 Staff (PSR4 to PSR8) | - 07 Desks with dividers (open concept) - 07 Chairs - 02 Filing rack - 01 Desk for printer setup - 06 Dustbins - 01 Clock |  | - 06 Phone - 01 Three in one Printer |  |
| 44 | AMR Coordination Unit |  |  |  |  |  |  |  |
| 45 | AMR Coordination Unit - Assistant Director | 120 | Open | 01 Staff (MS3) | - 01 Desk - 01 Chair for the executive and - 02 Chair in front of the table - 01 Filing cupboard - 01 Dustbin |  |  |  |
| 46 | AMR Coordination Unit - Staff | 160 | Open | 02 Staff (PSR7) | - 02 Desks with dividers (open concept) - 02 Chairs - 01 Filing rack - 01 Desk for printer setup - 01 Dustbin - 01 Clock |  |  |  |
| 47 | Small Meeting space for Food Control Division | 180 | Open | To conduct small urgent discussions. Space for 6 people. | - 01 Meeting table - 06 chairs |  |  |  |
| 48 | Second floor Meeting Hall | 350 | Closed | To conduct large meetings. Space for 25 people. | - 01 Conference table - 25 Chairs - 01 Side table for computer setup - 01 Tea setup table - 01 Dustbin - 01 Clock - Decor: Plants, artwork, or other decorative items to create a pleasant environment |  | - 01 75inch TV - 01 Computer system - 01 Phone - 01 Video Conferencing Equipment |  |
| 49 | Support staff area | 160 | Closed concept | Area for the support staff to rest and work | - 01 Table - 02 Chairs - 01 Rack |  | - 01 Phone |  |
| 50 | Pantry | 180 | Closed concept | Area for the staff to eat and drink | - 01 Dining table - 06 chairs - Kitchen cabinets - 01 Sink - 01 Refrigerator |  | - 01 Phone |  |
| 51 | Network rack space | 50 |  | To mount the network switches of each floor |  |  |  |  |
| 52 | Toilet sink area for male | 45 |  |  | - 01 Mirror - 02 Sink with tap - 01 Tissue dispenser - 01 Hand dryer - 01 Dustbin |  |  |  |
| 53 | Toilet 1 (Staff use) - male use | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder - 01 Dustbin |  |  |  |
| 54 | Toilet 2 (Staff use) - male use | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder - 01 Dustbin |  |  |  |
| 55 | Toilet sink area for female | 45 |  |  | - 01 Mirror - 02 Sink with tap - 01 Tissue dispenser - 01 Hand dryer - 01 Dustbin |  |  |  |
| 56 | Toilet 3 (Staff use) - female | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder - 01 Dustbin |  |  |  |
| 57 | Toilet 4 (Staff use) - female | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder - 01 Dustbin |  |  |  |
| 58 | Lift 1 (Employees and customers) | 60 |  | Customer and staff use (access controlled for upper floors) |  |  |  | 14 Pax |
| 59 | Lift 2 (Employees and customers) | 60 |  | Customer and staff use (access controlled for upper floors) |  |  |  | 14 Pax |
| 60 | Lift 3 (Food and Pharmaceutical) | 45 |  | Only for food and pharmaceutical lab staffs (access controlled) |  |  |  | 11 Pax |
| 61 | Lift 4 (Infectious Disease) | 45 |  | Only for Infectious Disease lab staffs (access controlled) |  |  |  | 11 Pax |
| 62 | Lift 6 (Heavy Duty Lift) | 80 |  | Only for heavy machineries and for garbage (access controlled) |  |  |  | 2000 Kg lift weight |
| 63 | Corridor |  |  | Walkways and Showers for emergency use |  |  |  |  |
| 64 | Staircase |  |  | Normal use - Provide an access from one floor to another |  |  |  |  |
| 65 | Fire-exit with fire exit doors |  |  | Emergency use: access to go out of the building in case of fire |  |  |  |  |

12.Third Floor

| # | Room/Space | Estimated Size (sqft) | Room Type | Function | Non-Medical Items | Medical Items | IT Items | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Maldives Therapeutic Goods Division |  |  |  |  |  |  |  |
| 2 | Maldives Therapeutic Goods Division - Pharmaceutical Specialist | 160 | Closed | 01 Staff (PSR9) | - 01 Desk - 01 Chair for the executive - 02 Chairs in front of the table - 01 Filing rack - 01 Dustbin Artwork, Plant or other decorative items to create a pleasant environment |  | - 01 Phone |  |
| 3 | Regulation Section |  |  |  |  |  |  |  |
| 4 | Regulation Section - Senior Pharmacist | 150 | Half closed | 01 Staff (PSR8) | - 01 Desk - 01 Chair for the executive - 02 Chairs in front of the table - 01 Filing rack - 01 Dustbin Artwork, Plant or other decorative items to create a pleasant environment |  | - 01 Phone |  |
| 5 | Registration and Licensing Unit |  |  |  |  |  |  |  |
| 6 | Registration and Licensing Unit - Senior Pharmacist | 120 | Open | 01 Staff (PSR8) | - 01 Desk - 01 Chair for the executive - 02 Chair in front of the table - 01 Filing cupboard - 01 Dustbin |  | - 01 Phone |  |
| 7 | Registration and Licensing Unit - Staff | 720 | Open | 09 Staff (PSR5) | - 09 Desks with dividers (open concept) - 09 Chairs - 02 Filing rack - 01 Desk for printer setup - 06 Dustbins - 01 Clock |  | - 06 Phone - 01 Three in one Printer |  |
| 8 | Therapeutic Goods Unit |  |  |  |  |  |  |  |
| 9 | Therapeutic Goods Unit - Senior Pharmacist | 120 | Open | 01 Staff (PSR8) | - 01 Desk - 01 Chair for the executive - 02 Chair in front of the table - 01 Filing cupboard - 01 Dustbin |  | - 01 Phone |  |
| 10 | Therapeutic Goods Unit - Staff | 720 | Open | 09 Staff (PSR5 to PSR7) | - 09 Desks with dividers (open concept) - 09 Chairs - 02 Filing rack - 01 Desk for printer setup - 03 Dustbins - 01 Clock |  | - 03 Phone - 01 Three in one Printer |  |
| 11 | Quality Assurance Section |  |  |  |  |  |  |  |
| 12 | Quality Assurance Section - Senior Pharmacist | 150 | Half closed | 01 Staff (PSR8) | - 01 Desk - 01 Chair for the executive - 02 Chairs in front of the table - 01 Filing rack - 01 Dustbin - Artwork, Plant or other decorative items to create a pleasant environment |  | - 01 Phone |  |
| 13 | Monitoring and Surveillance Unit |  |  |  |  |  |  |  |
| 14 | Monitoring and Surveillance Unit - Senior Pharmacist | 120 | Open | 01 Staff (PSR8) | - 01 Desk - 01 Chair for the executive - 02 Chair in front of the table - 01 Filing cupboard - 01 Dustbin |  | - 01 Phone |  |
| 15 | Monitoring and Surveillance Unit - Staff | 560 | Open | 07 Staff (PSR4 to PSR7) | - 07 Desks with dividers (open concept) - 07 Chairs - 02 Filing rack - 01 Desk for printer setup - 07 dustbins - 01 Clock |  | - 07 Phone - 01 Three in one Printer |  |
| 16 | Pharmacovigilance Unit |  |  |  |  |  |  |  |
| 17 | Pharmacovigilance - Senior Pharmacist | 120 | Open | 01 Staff (PSR8) | - 01 Desk - 01 Chair for the executive - 02 Chair in front of the table - 01 Filing cupboard - 01 Dustbin |  | - 01 Phone |  |
| 18 | Pharmacovigilance - Staff | 240 | Open | 03 Staff (PSR5 to PSR7) | - 01 Desk - 01 Chair for the executive and - 02 chair in front of the table - 01 Filing cupboard - 01 Dustbin - 01 Clock |  | - 02 Phone - 01 Three in one Printer |  |
| 19 | Small Meeting space for Maldives Therapeutic Goods Division | 180 | Open | To conduct small urgent discussions. Space 6 people. | - 01 Meeting table - 06 chairs |  |  |  |
| 20 | Inspectorate Division |  |  |  |  |  |  |  |
| 21 | Lead Food Quality Assurance Officer/Pharmaceutical specialist | 160 | Closed | 01 Staff (PSR 9) | - 01 Desk - 01 Chair for the executive - 02 Chairs in front of the table - 01 Filing rack - 01 Dustbin - Artwork, Plant or other decorative items to create a pleasant environment |  | - 01 Phone |  |
| 22 | Food Border inspection section |  |  |  |  |  |  |  |
| 23 | Food quality assurance coordinator | 150 | Half closed | 01 Staff (PSR 8) | - 01 Desk - 01 Chair for the executive - 02 Chairs in front of the table - 01 Filing rack - 01 Dustbin - Artwork, Plant or other decorative items to create a pleasant environment |  | - 01 Phone |  |
| 24 | Central Unit |  |  |  |  |  |  |  |
| 25 | Food quality assurance coordinator | 120 | Open | 01 Staff (PSR8) | - 01 Desk - 01 Chair for the executive and - 02 chair in front of the table - 01 Filing cupboard - 01 Dustbin |  | - 01 Phone |  |
| 26 | Food Safety Inspector | 320 | Open | 04 Staff (PSR 4 &5) | - 04 Desks with dividers (open concept) - 04 Chairs - 01 Filing rack - 01 Desk for printer setup - 03 Dustbin - 01 Clock |  | - 03 Phone - 01 Three in one Printer |  |
| 27 | Food Facility / Establishment Inspection section |  |  |  |  |  |  |  |
| 28 | Food quality assurance coordinator | 150 | Half closed | 01 Staff (PSR 8) | - 01 Desk - 01 Chair for the executive - 02 Chairs in front of the table - 01 Filing rack - 01 Dustbin - Artwork, Plant or other decorative items to create a pleasant environment |  | - 01 Phone |  |
| 29 | Central Unit |  |  |  |  |  |  |  |
| 30 | Food quality assurance coordinator | 120 | Open | 01 Staff (PSR 8) | - 01 Desk - 01 Chair for the executive and - 02 Chair in front of the table - 01 Filing cupboard - 01 Dustbin |  | - 01 Phone |  |
| 31 | Food quality assurance officer | 1600 | Open | 20 Staff (PSR 4 & 5) | - 20 Desks with dividers (open concept) - 20 Chairs - 04 Filing rack - 01 Desk for printer setup - 05 Dustbins - 01 Clock |  | - 05 Phone - 01 Three in one Printer |  |
| 32 | Pharmaceutical Facility / Establishment Inspection Section |  |  |  |  |  |  |  |
| 33 | Senior Pharmacist | 150 | Half closed | 01 Staff (PSR 8) | - 01 Desk - 01 Chair for the executive - 02 Chairs in front of the table - 01 Filing rack - 01 Dustbin Artwork, Plant or other decorative items to create a pleasant environment |  | - 01 Phone |  |
| 34 | Central Unit |  |  |  |  |  |  |  |
| 35 | Senior Pharmacist | 120 | Open | 01 Staff (PSR 8) | - 01 Desk - 01 Chair for the executive - 02 Chair in front of the table - 01 Filing cupboard - 01 Dustbin |  | - 01 Phone |  |
| 36 | Pharmaceutical officer | 1200 | Open | 15 Staff (PSR 4 & 5) | - 15 Desks with dividers (open concept) - 15 Chairs - 03 Filing rack - 01 Desk for printer setup - 05 Dustbins |  | - 05 Phone - 01 Three in one Printer |  |
| 37 | Pharmaceutical Boarder Inspection Section |  |  |  |  |  |  |  |
| 38 | Senior Pharmacist | 150 | Half closed | 01 Staff (PSR 8) | - 01 Desk - 01 Chair for the executive - 02 Chairs in front of the table - 01 Filing rack - 01 Dustbin - Artwork, Plant or other decorative items to create a pleasant environment |  | - 01 Phone |  |
| 39 | Central Unit |  |  |  |  |  |  |  |
| 40 | Senior Pharmacist | 120 | Open | 01 Staff (PSR 8) | - 01 Desk - 01 Chair for the executive - 02 Chair in front of the table - 01 Filing cupboard - 01 Dustbin |  | - 01 Phone |  |
| 41 | Pharmaceutical officer | 320 | Open | 04 Staff (PSR 5) | - 04 Desks with dividers (open concept) - 04 Chairs - 01 Filing rack - 01 Desk for printer setup - 02 Dustbins |  | - 04 Phone - 01 Three in one Printer |  |
| 42 | Small Meeting space for Inspectorate Division | 180 | Open | To conduct small urgent discussions. Space 6 people. | - 01 Meeting table - 06 Chairs |  |  |  |
| 43 | Third floor Meeting Hall | 350 | Closed | To conduct large meetings. Space for 25 people. | - 01 Conference table - 25 Chairs - 01 Side table for computer setup - 01 Tea setup table - 01 Dustbin - 01 Clock - Decor: Plants, artwork, or other decorative items to create a pleasant environment |  | - 01 75inch TV - 01 Computer system - 01 Phone - 01 Video Conferencing Equipment |  |
| 44 | Support staff area | 160 | Closed | Area for the support staff to rest and work | - 01 Table - 02 Chairs - 01 Rack |  | - 01 Phone |  |
| 45 | Pantry | 180 | Closed concept | Area for the staff to eat and drink | - 01 Dining table - 06 Chairs - Kitchen cabinets - 01 Sink - 01 Refrigerator |  | - 01 Phone |  |
| 46 | Network rack space | 50 |  | To mount the network switches of each floor |  |  |  |  |
| 47 | Toilet sink area for male | 45 |  |  | - 01 Mirror - 02 Sink with tap - 01 Tissue dispenser - 01 Hand dryer - 01 Dustbin |  |  |  |
| 48 | Toilet 1 (Staff use) - male use | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder - 01 Dustbin |  |  |  |
| 49 | Toilet 2 (Staff use) - male use | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder - 01 Dustbin |  |  |  |
| 50 | Toilet sink area for female | 45 |  |  | - 01 Mirror - 02 Sink with tap - 01 Tissue dispenser - 01 Hand dryer - 01 Dustbin |  |  |  |
| 51 | Toilet 3 (Staff use) - female | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder - 01 Dustbin |  |  |  |
| 52 | Toilet 4 (Staff use) - female | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder - 01 Dustbin |  |  |  |
| 53 | Lift 1 (Employees and customers) | 60 |  | Customer and staff use (access controlled for upper floors) |  |  |  | 14 Pax |
| 54 | Lift 2 (Employees and customers) | 60 |  | Customer and staff use (access controlled for upper floors) |  |  |  | 14 Pax |
| 55 | Lift 3 (Food and Pharmaceutical) | 45 |  | Only for food and pharmaceutical lab staffs (access controlled) |  |  |  | 11 Pax |
| 56 | Lift 4 (Infectious Disease) | 45 |  | Only for Infectious Disease lab staffs (access controlled) |  |  |  | 11 Pax |
| 57 | Lift 6 (Heavy Duty Lift) | 80 |  | Only for heavy machineries and for garbage (access controlled) |  |  |  | 2000 Kg lift weight |
| 58 | Corridor |  |  | Walkways and Showers for emergency use |  |  |  |  |
| 59 | Staircase |  |  | Normal use - Provide an access from one floor to another |  |  |  |  |
| 60 | Fire-exit with fire exit doors |  |  | Emergency use: access to go out of the building in case of fire |  |  |  |  |

13.Fourth Floor

| # | Room/Space | Estimated Size (sqft) | Room Type | Function | Non-Medical Items | Medical Items | IT Items | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | National Health laboratory Division |  |  |  |  |  |  |  |
| 2 | National Health laboratory Head - Health Laboratory Specialist | 160 | Closed | 01 Staff (PSR9) | - 01 Desk - 01 Chair for the executive - 02 Chairs in front of the table - 01 Filing rack - 01 Dustbin Artwork, Plant or other decorative items to create a pleasant environment |  | - 01 Computer System - 01 Phone |  |
| 3 | Quality Management Section |  |  |  |  |  |  |  |
| 4 | Quality Management Section Head - Senior Health Laboratory Technologist | 150 | Half closed | 01 Staff (PSR 8) | - 01 Desk - 01 Chair for the executive - 02 Chairs in front of the table - 01 Filing rack - 01 Dustbin - Artwork, Plant or other decorative items to create a pleasant environment |  | - 01 Computer System - 01 Phone |  |
| 5 | Calibration and Customer Service Unit |  |  |  |  |  |  |  |
| 6 | Calibration and Customer Service Unit Head - Senior Health Laboratory Technologist | 120 | Open | 01 Staff (PSR 8) | - 01 Desk - 01 Chair for the executive and - 02 chair in front of the table - 01 Filing cupboard - 01 Dustbin |  | - 01 Phone |  |
| 7 | Calibration and Customer Service Unit Staff | 320 | Open | 04 Staff (PSR 4 to 7) | - 04 Desks with dividers (open concept) - 04 Chairs - 01 Filing rack - 01 Desk for printer setup - 02 Dustbins - 01 Clock |  | - 03 Phone - 01 Three in one Printer |  |
| 8 | Documentation and Assessment Unit |  |  |  |  |  |  |  |
| 9 | Documentation and Assessment Unit Head - Senior Health Laboratory Technologist | 120 | Open | 01 Staff (PSR 8) | - 01 Desk - 01 Chair for the executive and - 02 chair in front of the table - 01 Filing cupboard - 01 Dustbin |  | - 01 Phone |  |
| 10 | Documentation and Assessment Unit Staff | 320 | Open | 03 Staff (PSR 4 to 7) | - 03 Desks with dividers (open concept) - 03 Chairs - 01 Filing rack - 01 Desk for printer setup - 01 Dustbin - 01 Clock |  | - 01 Phone - 01 Three in one Printer |  |
| 11 | Small Meeting space for National Health Laboratory | 180 | Open | To conduct small urgent discussions. Space 6 people. | - 01 Meeting table - 06 chairs |  |  |  |
| 12 | Fourth floor Meeting Hall | 350 | Closed | To conduct large meetings. Space for 25 people. | - 01 Conference table - 25 Chairs - 01 Side table for computer setup - 01 Tea setup table - 01 Dustbin - 01 Clock - Decor: Plants, artwork, or other decorative items to create a pleasant environment |  | - 01 75inch TV - 01 Computer system - 01 Phone - 01 Video Conferencing Equipment |  |
| 13 | Library | 300 | Closed | To provide access to a vast collection of resources, including books, magazines, research papers. | - Bookshelves - Reading Areas (02 Table and 04 chairs) |  | - 02 Computer system - 01 Phone - 01 Three in one Printer |  |
| 14 | Glassware Stock | 350 | Closed | To store all the glassware | - Racks - Work desk with drawers |  | - 01 Computer system - 01 Phone |  |
| 15 | NHL Store | 450 | Closed | To store Non-medical items of NHL | - Racks - Work desk with drawers |  | - 01 Computer system - 01 Phone |  |
| 16 | Support staff area | 160 | Closed concept | Area for the support staff to rest and work | - 01 Table - 02 chairs 01 Rack |  | - 01 Phone |  |
| 17 | Pantry | 180 | Closed concept | Area for the staff to eat and drink | - 01 Dining table - 06 chairs - Kitchen cabinets - 01 Sink 01 Refrigerator |  | - 01 Phone |  |
| 18 | Network rack space | 50 |  | To mount the network switches of each floor |  |  |  |  |
| 19 | Toilet sink area for male | 45 |  |  | - 01 Mirror - 02 Sink with tap - 01 Tissue dispenser - 01 Hand dryer - 01 Dustbin |  |  |  |
| 20 | Toilet 1 (Staff use) - male use | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder |  |  |  |
| 21 | Toilet 2 (Staff use) - male use | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder |  |  |  |
| 22 | Toilet sink area for female | 45 |  |  | - 01 Mirror - 02 Sink with tap - 01 Tissue dispenser - 01 Hand dryer - 01 Dustbin |  |  |  |
| 23 | Toilet 3 (Staff use) - female | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder |  |  |  |
| 24 | Toilet 4 (Staff use) - female | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder |  |  |  |
| 25 | Lift 1 (Employees and customers) | 60 |  | Customer and staff use (access controlled for upper floors) |  |  |  | 14 Pax |
| 26 | Lift 2 (Employees and customers) | 60 |  | Customer and staff use (access controlled for upper floors) |  |  |  | 14 Pax |
| 27 | Lift 3 (Food and Pharmaceutical) | 45 |  | Only for food and pharmaceutical lab staffs (access controlled) |  |  |  | 11 Pax |
| 28 | Lift 4 (Infectious Disease) | 45 |  | Only for Infectious Disease lab staffs (access controlled) |  |  |  | 11 Pax |
| 29 | Lift 6 (Heavy Duty Lift) | 80 |  | Only for heavy machineries and for garbage (access controlled) |  |  |  | 2000 Kg lift weight |
| 30 | Corridor with emergency shower |  |  | Walkways and Showers for emergency use | - 02 Shower sets - 01 Rack | - 02 Eye wash kit - 01 First aid kit - 02 Spill kit | - 03 Phones |  |
| 31 | Staircase |  |  | Normal use - Provide an access from one floor to another |  |  |  |  |
| 32 | Fire-exit with fire exit doors |  |  | Emergency use: access to go out of the building in case of fire |  |  |  |  |

14.Fifth Floor

| # | Room/Space | Estimated Size (sqft) | Room Type | Function | Non-Medical Items | Medical Items | IT Items | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Equipment Hall | 450 | Closed | To store lab's primary instruments and machines | - Work bench with drawers - 08 Stool chairs with back support - Racks | - HPLCMSMS - GCMSMS - ICPMSMS - AAS - HPLC - 02 FTIR - Mercury analyzer | - 05 Computer system - 05 Printer - 01 Phone | - Gas line for Argon Nitrogen, Helium and Hydrogen with control feature - Machines exhaust vents |
| 2 | Water Chemistry | 400 | Closed | To analyze safety and quality of water | - Central workbench with drawers - 02 Sinks - Rack on the workbench with power supply - Side workbenches with drawers - 04 Stool chairs with back support - Overhead cabinets | - 02 Fume hood - 02 Spectro photo meter - 03 Ph meter - 03 Econ meter - 01 Turbidity meter - 02 COD incubator - 02 BOD incubator - Chloride meter - Fridge refrigerator (2 to 8 Degree Celsius) | - 01 Phone |  |
| 3 | Pending samples - Water Chemistry | 180 | Closed | Designated space for the storage and management of water samples that are awaiting analysis. | - Racks | - 02 Fridge (2 to 8 degree Celsius) |  |  |
| 4 | Sample retention - Water chemistry | 190 | Closed | Designated space for the storage and management of water samples after the analysis. | - Racks | - 02 Fridge (2 to 8 degree Celsius) |  |  |
| 5 | Special chemical storage | 200 | Closed | Dedicated area to safely store and manage chemicals that are used in the analysis and treatment of water samples. | - Racks | - Acid cupboard flammable chemical cupboard |  |  |
| 6 | Food Chemistry | 400 | Closed | To analyze and study the chemical properties and reactions of food. | - Central workbench with drawers - 02 Sinks - Rack on the workbench with power supply - Side workbenches with drawers - 04 Stool chairs with back support - Overhead cabinets | - 02 Fluorometer with computer and printer - 01 Eliza Reader with printer - 01 Mercury Analyzer with gas connection and printer - 01 Water Bath - 01 Miller - 01 Freezer - 01 Fridge (Sample retention, pending samples and reagents) - 01 Kjeldahl protein digestor with water connection and drainage - 01 Kjeldahl protein analyzer with water connection and drainage - 01 Soxhlet fat extractor with water connection and drainage - 01 Turbovap (Automated Solvent Evaporation System) with gas connection - 01 Sonicator - 01 Refractometer with water bath - 01 Chiller with water connection - 01 mixer /grinder - 02 Fumehood - 01 Oven conventional - 01 Oven conventional ovens vacuum oven - 01 Muffle furnace - 01 Dessicator - 01 cryostat with gas connection - 01 hot plate with stirrer - 01 shaker - 01 Ph meter - 01 vacuum filter - 01 Dispensers - 01 burette stand - 02 Pipette stand and 1 impulse sealer | - 01 Computer system - 02 Printer - 01 Phone |  |
| 7 | Pending samples - Food Chemistry | 180 | Closed | Designated space for the storage and management of food samples that are awaiting analysis | - Racks | - 01 Freezer (-20 degree Celsius) - 01 Fridge (2 to 8 degree Celsius) |  |  |
| 8 | Sample retention - Food Chemistry | 180 | Closed | Designated space for the storage and management of food samples after the analysis | - Racks | - 01 Freezer (-20 degree Celsius) - 01 Fridge (2 to 8 degree Celsius) |  |  |
| 9 | Special Chemical storage | 200 | Closed | Designated area designed to store and manage chemicals used in food analysis and testing. |  | - Acid cupboard flammable chemical cupboard |  |  |
| 10 | Pharmaceutical Chemistry | 400 | Closed | To study and analyze drugs and pharmaceuticals through various chemical and analytical techniques. | - Central workbench with drawers - 02 Sinks - Rack on the workbench with power supply - Side workbenches with drawers - 04 Stool chairs with back support - Overhead cabinets | - Fume hood - Disintegration apparatus with water in and drainage - Melting point apparatus - 02 Spectro photo meter - Dissolution apparatus with water in and drain |  |  |
| 11 | Pending samples - Pharmaceutical Chemistry | 180 | Closed | Dedicated space for storing and managing pharmaceutical samples that are awaiting analysis | - Racks | - 01 Fridge (2 to 8 degree Celsius) - 01 Freezer (-20 degree Celsius) |  |  |
| 12 | Sample retention - Pharmaceutical Chemistry | 180 | Closed | Dedicated space for storing and managing pharmaceutical samples after the analysis | - Racks | - 01 Fridge (2 to 8 degree Celsius) - 01 Freezer (-20 degree Celsius) |  |  |
| 13 | Special Chemical storage | 180 | Closed | Dedicated area designed for the safe and efficient storage of chemicals used in the development, testing, and production of pharmaceuticals |  | - Acid cupboard flammable chemical cupboard |  |  |
| 14 | Sensory preparation | 200 | Closed | To prepare and present food or product samples for sensory testing | - Work bench with drawers - 02 Stool chairs with back support - Racks - 02 Sink | - Microwave, sieve shaker and a water bath |  |  |
| 15 | Sensory Evaluation | 200 | Closed | Area designed for assessing the sensory attributes of products, such as food, beverages | - Work bench with drawers - 05 Cubicles - 05 Stool chairs with back support |  |  | - Very good lighting |
| 16 | Changing Room with lockers | 200 | Closed | Dedicated area where personnel can change into appropriate work attire and securely store their personal belongings | - Lockers - Hangers - Bench to sit down. - 01 Shoe rack (for outdoor foot wear) - 01 Shoe rack (indoor foot wear) - 01 Sink - 01 Dryer - 01 Dustbin - Small Work bench with drawers. |  |  |  |
| 17 | Calibration / Weighing Room | 250 | Closed | Area for precise calibration of instruments and the accurate weighing of materials | - Work bench with drawers - 02 Stool chairs with back support. | - 04 Analytical Balance - 02 Weight box |  | - Workbench should be vibration free and separated from the wall - Automatic door or sliding door) |
| 18 | Common Sample preparation and unique sample preparation | 250 | Closed | Preparation of samples before they are analyzed or tested. This is to ensure that samples are processed in a consistent, controlled, and efficient manner to maintain the integrity of the analysis and achieve reliable results | - Work bench with drawers - 02 Stool chairs with back support - 02 Sinks (Water connection for machines with drains) | - Rotor vapor - Nitrogen concentrator - 02 Fume hood | - 01 Phone |  |
| 18 | Distillation, deionized water and ultra-pure water etc. | 250 | Closed | To prepare and handle various types of purified water | - Work bench with drawers - 02 Stool chairs with back support - 02 Sinks (Water connection for machines with drains) | - 02 Distillation unit with water in and out - 01 Ultra-pure water machine with water in and out - 01 Trace metal water machine with water in and out - Each machine should be with a reservoir. |  |  |
| 19 | Ovens, heating, desiccators and centrifuges etc. | 250 | Closed | To facilitate processes that involve temperature control, drying, and separation of substances. | - Work bench with drawers | - 02 Oven - Muffle Furner’s - 02 Desiccators - 01 Thermometer - 01 Calibration bath |  |  |
| 20 | Common and special washing and sample containers prep. Room | 180 | Closed | To clean, prepare, and maintenance of laboratory glassware, sample containers, and other equipment | - Racks for drying items - 02 Sinks - 01 Stool chair with back support | - 01 Hot air oven |  |  |
| 21 | Trace metals, inorganic digestions and extraction | 250 | Closed | To prepare, digest, and extract of trace metals and inorganic elements from various samples | - Work bench with drawers - 02 stool chairs with back support - 02 Sink with metal free taps - Cupboards - Glass wear draining rack |  |  |  |
| 22 | Pesticide, veterinary drug and antimicrobial residues extraction | 250 | Closed | To extract, analyze, and quantify of residues from pesticides, veterinary drugs, and antimicrobial agents in various samples | - Work bench with drawers - 02 Stool chairs with back support - 02 Sink with metal free taps - Cupboards - Glass wear draining rack |  |  |  |
| 23 | Wet digestion, Microwave digestion and dry digestions | 250 | Closed | To use various methods of sample digestion in the preparation of samples for chemical analysis | - Work bench with drawers - 02 Stool chairs with back support - 02 Sink - Cupboards - Draining rack | - Microwave digestor with vent - 01 Fume hood |  |  |
| 24 | Laundry Chemistry | 250 | Closed | Dedicated space for laundry and cleaning products | - 01 Washing machine - 01 Dryer - 01 Iron board - 01 Iron - Hangers - Cloths lines to dry cloths. - Work bench |  |  |  |
| 25 | Laundry Storage | 180 | Closed | To store clean lab attires, linens, and other items | - Racks |  |  |  |
| 26 | Chemistry Section |  |  |  |  |  |  |  |
| 28 | Report Generation - Chemistry Section Head - Senior Health Laboratory Technologist | 150 | Half Closed | 1 Staff (PSR 8) | - 01 Desk - 01 Chair for the executive - 02 Chairs in front of the table - 01 Filing rack - 01 Dustbin - Artwork, Plant or other decorative items to create a pleasant environment |  | - 01 Phone |  |
| 29 | MTG Chemistry Unit |  |  |  |  |  |  |  |
| 30 | Report Generation - MTG Chemistry Unit Head - Senior Health Laboratory Technologist | 120 | Open | 1 Staff (PSR 8) | - 01 Desk - 01 Chair for the executive - 02 Chair in front of the table - 01 Filing cupboard - 01 Dustbin |  | - 01 Phone |  |
| 31 | Report Generation - MTG Chemistry Unit Staff | 320 | Open | 4 Staff (PSR 5 to 7) | - 04 Desks with dividers (open concept) - 04 Chairs - 01 Filing rack - 01 Desk for printer setup - 03 Dustbins - 01 Clock |  | - 04 Phone - 01 Three in one Printer |  |
| 32 | Food Chemistry Unit |  |  |  |  |  |  |  |
| 33 | Report Generation - Food Chemistry Unit Head - Senior Health Laboratory Technologist | 120 | Open | 1 Staff (PSR 8) | - 01 Desk - 01 Chair for the executive and - 02 Chair in front of the table - 01 Filing cupboard - 01 Dustbin |  | - 01 Phone |  |
| 34 | Report Generation - Food Chemistry Unit Staff | 720 | Open | 9 Staff (PSR 4 to 7) | - 09 Desks with dividers (open concept) - 09 Chairs - 02 Filing rack - 01 Desk for printer setup - 09 Dustbins - 01 Clock |  | - 09 Phone - 01 Three in one Printer |  |
| 35 | Small Meeting space | 180 | Open | To conduct small urgent discussions. Space 6 people. | - 01 Meeting table - 06 Chairs |  |  |  |
| 36 | Staff resting area | 180 | Closed | Staff to rest | - Sofa set - Table - Rack - Plants |  | - 01 Phone |  |
| 37 | Support staff area | 160 | Closed concept | Area for the support staff to rest and work | - 01 Table - 02 chairs - 01 Rack |  | - 01 Phone |  |
| 38 | Pantry | 180 | Closed concept | Area for the staff to eat and drink | - 01 Dining table - 06 chairs - Kitchen cabinets - 01 Sink - 01 Refrigerator - 01 Dustbin |  | - 01 Phone |  |
| 39 | Network rack space | 50 |  | To mount the network switches of each floor |  |  |  |  |
| 40 | Toilet sink area for male | 45 |  |  | - 01 Mirror - 02 Sink with tap - 01 Tissue dispenser - 01 Hand dryer - 01 Dustbin |  |  |  |
| 41 | Toilet 1 (Staff use) - male use | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder - 01 Dustbin |  |  |  |
| 42 | Toilet 2 (Staff use) - male use | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder - 01 Dustbin |  |  |  |
| 43 | Toilet sink area for female | 45 |  |  | - 01 Mirror - 02 Sink with tap - 01 Tissue dispenser - 01 Hand dryer 01 Dustbin |  |  |  |
| 44 | Toilet 3 (Staff use) - female | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder - 01 Dustbin |  |  |  |
| 45 | Toilet 4 (Staff use) - female | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder - 01 Dustbin |  |  |  |
| 46 | Lift 1 (Employees and customers) | 60 |  | Customer and staff use (access controlled for upper floors) |  |  |  | 14 Pax |
| 47 | Lift 2 (Employees and customers) | 60 |  | Customer and staff use (access controlled for upper floors) |  |  |  | 14 Pax |
| 48 | Lift 3 (Food and Pharmaceutical) | 45 |  | Only for food and pharmaceutical lab staffs (access controlled) |  |  |  | 11 Pax |
| 49 | Lift 4 (Infectious Disease) | 45 |  | Only for Infectious Disease lab staffs (access controlled) |  |  |  | 11 Pax |
| 50 | Lift 6 (Heavy Duty Lift) | 80 |  | Only for heavy machineries and for garbage (access controlled) |  |  |  | 2000 Kg lift weight |
| 51 | Corridor with emergency shower |  |  | Walkways and Showers for emergency use | - 02 Shower sets - 01 Rack | - 03 Eye wash kit (three side of the building) - 01 First aid kit - 02 Spill kit | - 03 Phones (three side of the building) |  |
| 52 | Staircase |  |  | Normal use - Provide an access from one floor to another |  |  |  |  |
| 53 | Fire-exit with fire exit doors |  |  | Emergency use: access to go out of the building in case of fire |  |  |  |  |

15.Sixth Floor

| # | Room/Space | Estimated Room Size (sqft) | Room Type | Function | Non-Medical Items | Medical Items | IT Items | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Changing Room | 200 | Closed | To change clothes | - Lockers - Hangers - Bench to sit down. - 01 Shoe rack (for outdoor foot wear) - 01 Shoe rack (indoor foot wear) - Automatic foot cover machine - 01 Sink - 01 Dryer - 01 Dustbin - Small Work bench with drawers |  |  |  |
| 2 | Sample transfer room | 180 | Closed | Designated space to transfer samples to lab | - Work bench with drawers |  |  |  |
| 3 | Buffer Zone | 120 | Closed | To avoid the entrance of particle and microbial contaminants |  |  |  | - Air pressure-controlled area - Air curtains |
| 4 | Food Microbiology | 500 | Closed | To study of the microorganisms that inhabit, create, or contaminate food | - Work bench with drawers - 04 Stool chairs with back support - 02 Sink | - 02 Bio-safety cabinet - 01 Infrared sterilizer - 01 Water bath - 01 Colony counter - 01 Balance - 01 Stomacher |  |  |
| 5 | Incubation Room | 250 | Closed | To provide optimal growth conditions for cells and cultures. | - Bench to keep incubators | - 01 Incubator (42 Degree Celsius) - 01 Incubator (44 Degree Celsius) - 01 Incubator (37 Degree Celsius) - 01 Incubator (25 Degree Celsius) - 01 Incubator (41.5 Degree Celsius) - 01 Incubator (22 Degree Celsius) - 02 Oven (55 Degree Celsius) |  |  |
| 6 | Water Microbiology | 350 | Closed | To study of the micro-organisms that live in water, such as bacteria, fungi, yeast, viruses, protozoa and algae | - Work bench with drawers - 02 Stool chairs with back support - 02 Sink | - 01 Bio-safety cabinet - 01 Infrared sterilizer |  |  |
| 7 | Distillation Room | 200 | Closed | Distilling solvents or other chemicals used in microbiological analysis | - Work bench with drawers - 02 Stool chairs with back support - 02 Sink | - 02 Distillation unit - 01 Ultra-purification system |  | Setup should be made for hot water out |
| 8 | Media Room | 250 | Closed | To prepare culture media, reagents, and solutions. | - Work bench with drawers - 06 Stool chairs with back support - 02 Sink | - 03 Fridge (2 to 8-degree Celsius) - 02 Biosafety cabinet - 01 Balance |  | - Air controlled room - Dark Room without sunlight |
| 9 | Sterilization Room | 250 | Closed | To clean and sterilize of devices and equipment | - Work bench with drawers - 02 Sink | - 04 Autoclaves (40 to 60 liters) |  | - Good ventilation to remove heat. |
| 10 | Dry after Sterilization Room (Glass wear storage) | 180 | Closed | To dry and store glass wear | - Work bench with drawers | - Storage cupboard (Anti-fungus) |  |  |
| 11 | Pending Sample Storage | 180 | Closed | To store samples before processing | - Work bench with drawers | - 01 Fridge (2 degree Celsius) - 02 Freezer (-20 degree Celsius_ - 01 Freezer (-80 degree Celsius) |  |  |
| 12 | Reference culture room | 180 | Closed | To store reference culture | - Work bench with drawers | - 01 Fridge (2 degree Celsius) - 02 Freezer (-20 degree Celsius_ - 01 Freezer (-80 degree Celsius) |  |  |
| 13 | Temporary Consumable | 180 | Closed | To store in-use consumables | - Work bench with drawers - Racks |  |  |  |
| 14 | Bottle wash | 200 | Closed | To rinse various pieces of laboratory glassware | - Work bench with drawers - 02 Stool chairs with back support - 04 Sinks - Drying rack for test tube and bottles | - 02 Oven (55 degree Celsius) |  |  |
| 15 | Changing Room -Molecular Biology Food (PCR) | 200 | Closed | To change clothes to enter Molecular Biology Food lab | - Lockers - Hangers - Bench to sit down. - 01 Shoe rack (for outdoor foot wear) - 01 Shoe rack (indoor foot wear) - Automatic foot cover machine - 01 Sink - 01 Dryer - 01 Dustbin - Small Work bench with drawers |  |  |  |
| 16 | Freezer Room - Molecular Biology Food (PCR) | 200 | Closed | To store reagents, samples, and other materials at low temperatures to maintain their stability | - Work bench with drawers - 01 Sink | - 01 Freezer (-20 degree Celsius) - 01 Freezer (- 80 degree Celsius) - 01 Fridge (2 to 8 degree Celsius) - Ice maker |  |  |
| 17 | Sample Processing Lab | 200 | Closed | Dedicated space for the Preparation and initial handling of biological samples before diagnostic testing. | - Work bench with drawers - 02 Stool chairs with back support - 01 Sink | - Centrifuges - Biological Safety Cabinets (BSC) - Extraction machine. |  | Pass box to transfer items |
| 18 | Preparation room - Molecular Biology Food (PCR) | 200 | Closed | Reagent preparation, sample handling, and setting up experiments before entering the main lab for analysis | - Work bench with drawers - 02 Stool chairs with back support - Racks | - 02 Bio-safety cabinet - 02 Centrifuge - 01 Thermomixer - 01 Vortex mixture |  | Pass box to transfer items to PCR mix room |
| 20 | PCR mix room - Molecular Biology Food (PCR) | 200 | Closed | Dedicated space for preparing the PCR reagents and reaction mixtures | - Work bench with drawers - 02 Stool chairs with back support - Racks | - 01 PCR workstation - 01 Centrifuge - 01 Thermo mixture - Racks |  | Pass box to transfer items to PCR room |
| 21 | PCR room - Molecular Biology Food (PCR) | 200 | Closed | Space for PCR Testing | - Work bench with drawers - 02 Stool chairs with back support - Racks | - 02 PCR machine - 01 Extraction machine | - 02 Computer system | Pass box to transfer items |
| 22 | Small Store - Molecular Biology Food (PCR) | 200 | Closed | To store consumables, reagents, equipment, and other supplies | - Racks |  |  |  |
| 23 | Pharmaceutical Microbiology (Sterility check lab) | 500 | Closed | To test pharmaceutical products to ensure they are free from microbial contamination | - Work bench with drawers - 04 Stool chairs with back support - 02 Sink | - 01 Bio safety cabinet - 02 Fridge (2 to 8 degree Celsius) - 01 Freezer (- 20 Celsius) - 02 water bath - 01 Water filtration unit - Infrared sterilizer - Weighing balance - Stirrer - Centrifuge - Vortex machine and hot plate |  |  |
| 24 | Changing room (negative air pressure door) - Pharmaceutical Microbiology | 200 | Closed | With negative air pressure to ensure contamination control, is designed for personnel to change into cleanroom attire and maintain sterility before entering critical areas | - Lockers - Hangers - Bench to sit down. - 01 Shoe rack (for outdoor foot wear) - 01 Shoe rack (indoor foot wear) - Automatic foot cover machine - 01 sink - 01 dryer - 01 dustbin - Small Work bench with drawers |  |  |  |
| 25 | Sterile Room - Pharmaceutical Microbiology | 200 | Closed | Space for sterile testing, such as sterility checks and aseptic operations | - Work bench with drawers - 02 Stool chairs with back support. | - 01 Bio-safety cabinet |  | - lab should be constructed with highly controlled environment and should be with gloves barrier system to pass samples from the lab to outside of the lab. - Passbox |
| 26 | Incubator room - Pharmaceutical Microbiology | 200 | Closed | To house incubators for the growth of microbial cultures at controlled temperatures | - Bench to keep incubators | - 02 Incubators (25 degree Celsius) - 01 Incubators (37 degree Celsius) - 01 Incubators (44 degree Celsius) |  |  |
| 27 | Microscopy | 250 | Closed | To use microscopes to view samples. | - Work bench with drawers - 02 Stool chairs with back support | - 03 Microscope (digital) - 02 Dissection microscope | - 01 Computer system - 01 Phone |  |
| 28 | Buffer Zone - Decontamination Room | 100 | Closed | To prevent cross-contamination between clean and contaminated areas. |  |  |  | - Good ventilation to control smell. - Air pressure-controlled area - Air curtains |
| 29 | Decontamination Room | 250 | Closed | To clean and sterilized contaminated instruments | - Work bench with drawers - 01 Stool chairs with back support - 2 Sinks - Water inlet for the machine and drainage | - 01 Autoclave (front load) - 01 Autoclave (top load) - 02 Fridge (Degree Celsius) |  | Room should be near bottle wash and sterilization room |
| 30 | Laundry Microbiology | 250 | Closed | To clean and sterilize of lab clothing, reusable textiles, and other fabric items | - 01 Washing machine - 01 Dryer - 01 Iron board - 01 Iron - Hangers - Cloths lines to dry cloths. - Work bench |  |  |  |
| 31 | Landry storage - Laundry microbiology | 200 | Closed | To store clean lab attire, linens, and other items | - Racks |  |  |  |
| 32 | Microbiology Section |  |  |  |  |  |  |  |
| 33 | Report Generation - Microbiology Section Head Senior Health Laboratory Technologist | 150 | Half closed | 01 Staff (PSR8) | - 01 Desk - 01 Chair for the executive - 02 Chairs in front of the table - 01 Filing rack - 01 Dustbin - Artwork, Plant or other decorative items to create a pleasant environment |  | - 01 Phone |  |
| 34 | Food Microbiology Unit |  |  |  |  |  |  |  |
| 35 | Food Microbiology Unit Head Senior Health Laboratory Technologist | 120 | Open | 1 Staff (PSR8) | - 01 Desk - 01 Chair for the executive and - 02 chair in front of the table - 01 Filing cupboard - 01 Dustbin |  | - 01 Phone |  |
| 36 | Food Microbiology Unit Staff | 720 | Open | 9 Staff (MS4 to PSR 7) | - 09 Desks with dividers (open concept) - 09 Chairs - 02 Filing rack - 01 Desk for printer setup - 08 Dustbins |  | - 08 Phone - 01 Three in one Printer |  |
| 37 | MTG Microbiology Unit |  |  |  |  |  |  |  |
| 38 | MTG Microbiology Unit head Senior Health Laboratory Technologist | 120 | Open | 1 Staff (PSR8) | - 01 Desk - 01 Chair for the executive and - 02 chair in front of the table - 01 Filing cupboard - 01 Dustbin |  | - 01 Phone |  |
| 39 | MTG Microbiology Unit Staff | 240 | Open | 3 Staff (PSR 4 to 7) | - 03 Desks with dividers (open concept) - 03 Chairs - 01 Filing rack - 01 Desk for printer setup - 01 dustbins |  | - 02 Phone - 01 Three in one Printer |  |
| 40 | Small Meeting space | 180 | Open | To conduct small urgent discussions. Space 6 people. | - 01 Meeting table - 06 Chairs |  |  |  |
| 41 | Staff resting area | 180 | Closed | Staff to rest | - Sofa set - Table - Rack Plants |  | - 01 Phone |  |
| 42 | Support staff area | 160 | Closed concept | Area for the support staff to rest and work | - 01 Table - 02 chairs - 01 Rack |  | - 01 Phone |  |
| 43 | Pantry | 180 | Closed concept | Area for the staff to eat and drink | - 01 Dining table - 06 chairs - Kitchen cabinets - 01 Sink - 01 Refrigerator |  | - 01 Phone |  |
| 44 | Network rack space | 50 |  | To mount the network switches of each floor |  |  |  |  |
| 45 | Toilet sink area for male | 45 |  |  | - 01 Mirror - 02 Sink with tap - 01 Tissue dispenser - 01 Hand dryer - 01 Dustbin |  |  |  |
| 46 | Toilet 1 (Staff use) - male use | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder - 01 Dustbin |  |  |  |
| 47 | Toilet 2 (Staff use) - male use | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder - 01 Dustbin |  |  |  |
| 48 | Toilet sink area for female | 45 |  |  | - 01 Mirror - 02 Sink with tap - 01 Tissue dispenser - 01 Hand dryer - 01 Dustbin |  |  |  |
| 49 | Toilet 3 (Staff use) - female | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder - 01 Dustbin |  |  |  |
| 50 | Toilet 4 (Staff use) - female | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder - 01 Dustbin |  |  |  |
| 51 | Lift 1 (Employees and customers) | 60 |  | Customer and staff use (access controlled for upper floors) |  |  |  | 14 Pax |
| 52 | Lift 2 (Employees and customers) | 60 |  | Customer and staff use (access controlled for upper floors) |  |  |  | 14 Pax |
| 53 | Lift 3 (Food and Pharmaceutical) | 45 |  | Only for food and pharmaceutical lab staffs (access controlled) |  |  |  | 11 Pax |
| 54 | Lift 4 (Infectious Disease) | 45 |  | Only for Infectious Disease lab staffs (access controlled) |  |  |  | 11 Pax |
| 55 | Lift 6 (Heavy Duty Lift) | 80 |  | Only for heavy machineries and for garbage (access controlled) |  |  |  | 2000 Kg lift weight |
| 56 | Corridor with emergency shower |  |  | Walkways and Showers for emergency use | - 02 Shower sets - 01 Rack | - 02 Eye wash kit - 01 First aid kit - 02 Spill kit | - 03 Phones |  |
| 57 | Staircase |  |  | Normal use - Provide an access from one floor to another |  |  |  |  |
| 58 | Fire-exit with fire exit doors |  |  | Emergency use: access to go out of the building in case of fire |  |  |  |  |

16.Seventh Floor

| # | Room/Space | Estimated Size (sqft) | Room Type | Function | Non-Medical Items | Medical Items | IT Items | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Changing Room | 180 | Closed | To change clothes | - Lockers - Hangers - Bench to sit down. - 01 Shoe rack (for outdoor foot wear) - 01 Shoe rack (indoor foot wear) - Automatic foot cover machine - 01 Sink - 01 Dryer - 01 Dustbin - Small Work bench with drawers |  |  |  |
| 2 | Sample transfer room | 180 | Closed | Designated space to transfer samples to lab | - Work bench with drawers |  |  |  |
| 3 | Buffer Zone | 120 | Closed | To avoid the entrance of particle and microbial contaminants |  |  |  | - Air pressure-controlled area - Air curtains |
| 4 | Incubation Room | 250 | Closed | To provide optimal growth conditions for cultures. | - Bench to keep incubators | - 01 Incubators (42 Degree Celsius) - 01 Incubators (44 Degree Celsius) - 01 Incubators (37 Degree Celsius) - 01 Incubators (25 Degree Celsius) - 01 Incubators (41.5 Degree Celsius) - 01 Incubators (22 Degree Celsius) - 02 Oven (55 Degree Celsius) |  |  |
| 5 | Equipment room | 250 | Closed | To house, maintain, and manage laboratory instruments and equipment. | - Racks - Small work bench with drawers |  |  |  |
| 6 | Reagent Preparation Room | 180 | Closed | Dedicated space for the preparation, storage, and handling of chemical reagents, solutions, and other substances required for laboratory testing and analysis. | - Work bench with drawers - 02 Stool chairs with back support - Racks - 02 Sinks |  |  |  |
| 7 | Mycology Room | 250 | Closed | Dedicated space to study and analysis of fungi, including their identification, culture, and sensitivity testing | - Work bench with drawers - 02 Stool chairs with back support - Racks | - Microscope - Biosafety Cabinet - Water Bath - Growth Chambers |  |  |
| 8 | Entomology Room | 250 | Closed | Dedicated space to study and analysis of insects and other arthropods, particularly those that impact public health | - Work bench with drawers - 02 Stool chairs with back support - Racks - 01 Sink | - 01 Stereo microscope - 01 Compound microscope - Insect Rearing Cages - Environmental Chambers - Insect Nets - Biosafety Cabinets - UV Sterilizers | - Cameras and Imaging Systems |  |
| 9 | Parasitology Room | 250 | Closed | Area focused on diagnosing, researching, and monitoring parasitic infections | - Work bench with drawers - 02 Stool chairs with back support - Racks - 01 Sink | - Light Microscope - Compound Microscope - Fluorescence Microscope - Clinical Centrifuge - ELISA (Enzyme-Linked Immunosorbent Assay) Equipment Machine - Water Bath - Hot Plate - Incubators - Biosafety Cabinets - UV Sterilizers | - Cameras and Imaging Systems |  |
| 10 | Storage Room | 180 | Closed | Proper management, preservation, and organization of laboratory supplies, reagents, and equipment. | - Racks | - 01 Freezer (-80 Degree Celsius) - 01 Refrigerator (4 Degree Celsius) - 01 Flammable Storage Cabinets - 01 Chemical Storage Cabinets |  |  |
| 11 | Bacteriology room | 300 | Closed | To diagnose bacterial infections and understand bacterial pathogens. | - Work bench with drawers - 02 Stool chairs with back support - Racks - 02 Sink | - Bacterial Incubators (35-37°C) - Light Microscope - Phase Contrast Microscope - Water Baths - Media Mixers - Biosafety Cabinets - UV Sterilizers | - Cameras and Imaging Systems |  |
| 12 | Distillation Room | 200 | Closed | To distil solvents or other chemicals used in microbiological analysis. | - Work bench with drawers - 02 Stool chairs with back support - 02 Sink | - 02 Distillation unit - 01 Ultra-purification system |  | Setup should be made for hot water out |
| 13 | Media Room | 250 | Closed | To prepare culture media, reagents, and solutions. | - Work bench with drawers - 06 Stool chairs with back support - 02 Sink | - 03 Fridge (2 to 8-degree Celsius) - 02 Biosafety cabinet - 01 Balance |  | - Air controlled room - Dark Room without sunlight |
| 14 | Sterilization Room | 250 | Closed | To clean and sterilize of devices and equipment | - Work bench with drawers - 02 Sink | - 04 Autoclaves (40 to 60 liters) |  | Good ventilation to remove heat. |
| 15 | Dry after Sterilization Room | 180 | Closed | To dry and store glass wear | - Work bench with drawers | - Storage cupboard (Anti-fungus) |  |  |
| 16 | Pending Sample Storage | 180 | Closed | To store samples before processing | - Work bench with drawers | - 01 Fridge (2 degree Celsius) - 02 Freezer (-20 degree Celsius_ - 01 Freezer (-80 degree Celsius) |  |  |
| 17 | Reference culture room | 180 | Closed | To store reference culture | - Work bench with drawers | - 01 Fridge (2 degree Celsius) - 02 Freezer (-20 degree Celsius) - 01 Freezer (-80 degree Celsius) |  |  |
| 18 | Freezer Room - (PCR) | 200 | Closed | To store reagents, samples, and other materials at low temperatures to maintain their stability | - Work bench with drawers - 01 Sink | - 01 Freezer (-20 degree Celsius) - 01 Freezer (- 80 degree Celsius) - 01 Fridge (2 to 8 degree Celsius) - Ice maker |  |  |
| 19 | Sample Processing Lab | 200 | Closed | Dedicated space for the Preparation and initial handling of biological samples before diagnostic testing. | - Work bench with drawers - 02 Stool chairs with back support - 01 Sink | - Centrifuges - Biological Safety Cabinets (BSC) - Extraction machine. |  | Pass box to transfer items |
| 20 | Preparation room - (PCR) | 200 | Closed | Reagent preparation, sample handling, and setting up experiments before entering the main lab for analysis. | - Work bench with drawers - 02 Stool chairs with back support - Racks | - 02 Bio-safety cabinet - 02 Centrifuge - 01 Thermomixer - 01 Vortex mixture |  | Pass box to transfer items to PCR mix room |
| 21 | PCR mix room - (PCR) | 200 | Closed | Dedicated space for preparing the PCR reagents and reaction mixtures. | - Work bench with drawers - 02 Stool chairs with back support - Racks | - 01 PCR workstation - 01 Centrifuge - 01 Thermo mixture |  | Pass box to transfer items to PCR room |
| 22 | PCR room - (PCR) | 200 | Closed | Space for PCR Testing | - Work bench with drawers - 02 Stool chairs with back support - Racks | - 02 PCR machine - 01 Extraction machine | - 02 Computer system - 01 Phone |  |
| 23 | Sequencing Room | 200 | Closed | To do sequencing of the genetic material (DNA or RNA) of pathogens such as HIV and various sexually transmitted infections (STIs). | - Work bench with drawers - 02 Stool chairs with back support - 01 Rack | - DNA/RNA Sequencers | - 01 Computer system - 01 Phone - 01 Printer |  |
| 24 | Small Store - (PCR) | 200 | Closed | To store consumables, reagents, equipment, and other supplies | - Racks |  |  |  |
| 25 | Buffer Zone - Decontamination Room |  | Closed | To prevent cross-contamination between clean and contaminated areas. |  |  |  | - Good ventilation to control smell. - Air pressure-controlled area - Air curtains |
| 26 | Bottle wash | 150 | Closed | To rinse various pieces of laboratory glassware | 1. Work bench with drawers 2. 02 Stool chairs with back support 3. 04 sinks 4. Drying rack for test tube and bottles | 1. 02 Oven (55 degree Celsius) |  |  |
| 27 | Decontamination Room |  | Closed | To clean and sterilized contaminated instruments | - Work bench with drawers - 01 Stool chairs with back support - 2 Sinks - Water inlet for the machines and drainage | - 01 Autoclave (front load) - 01 Autoclave (top load) - 02 Fridge (Degree Celsius) |  | Room should be near bottle wash and sterilization room |
| 28 | Laundry Room |  | Closed | To clean and sterilize of lab clothing, reusable textiles, and other fabric items | - 01 Washing machine - 01 Dryer - 01 Iron board - 01 Iron - Hangers - Cloths lines to dry cloths. - Work bench |  |  |  |
| 29 | Landry storage |  | Closed | To store clean lab attire, linens, and other items | - Racks |  |  |  |
| 30 | Section Head - Bacteriology | 150 | Half closed | 01 Staff | - 01 Desk - 01 Chair for the executive - 02 Chairs in front of the table - 01 Filing rack - 01 Dustbin - Artwork, Plant or other decorative items to create a pleasant environment |  |  |  |
| 31 | Unit Head - Bacteriology | 120 | Open | 1 Staff | - 01 Desk - 01 Chair for the executive and - 02 Chair in front of the table - 01 Filing cupboard - 01 Dustbin |  |  |  |
| 32 | 5 Staff Bacteriology | 400 | Open | 5 Staff | - 05 Desks with dividers (open concept) - 05 Chairs - 02 Filing rack - 01 Desk for printer setup - 02 Dustbins |  | - 02 Computer system - 02 Phone - 01 Three in one Printer |  |
| 33 | Unit Head - Fungus and other | 120 | Open | 1 Staff | - 01 Desk - 01 Chair for the executive and - 02 Chair in front of the table - 01 Filing cupboard - 01 Dustbin |  |  |  |
| 34 | 8 Staff - Fungus and other | 640 | Open | 8 Staff | - 08 Desks with dividers (open concept) - 08 Chairs - 02 Filing rack - 01 Desk for printer setup - 02 dustbins |  | - 02 Computer system - 02 Phone - 01 Three in one Printer |  |
| 35 | Unit Head - Quality | 120 | Open | 1 Staff | - 01 Desk - 01 Chair for the executive and - 02 Chair in front of the table - 01 Filing cupboard - 01 Dustbin |  |  |  |
| 36 | 5 Staff - Quality | 400 | Open | 5 Staff | - 05 Desks with dividers (open concept) - 05 Chairs - 02 Filing rack - 01 Desk for printer setup - 02 Dustbins |  | - 02 Computer system - 02 Phone - 01 Three in one Printer |  |
| 37 | Small Meeting space | 180 | Open | To conduct small urgent discussions. Space 6 people. | - 01 Meeting table - 06 Chairs |  |  |  |
| 38 | Staff resting area | 180 | Closed | Staff to rest | - Sofa set - Table - Rack - Plants |  | - 01 Phone |  |
| 39 | Support staff area | 160 | Closed concept | Area for the support staff to rest and work | - 01 Table - 02 chairs - 01 Rack |  | - 01 Phone |  |
| 40 | Pantry | 180 | Closed concept | Area for the staff to eat and drink | - 01 Dining table - 06 chairs - Kitchen cabinets - 01 Sink - 01 Refrigerator |  | - 01 Phone |  |
| 41 | Network rack space | 50 |  | To mount the network switches of each floor |  |  |  |  |
| 42 | Toilet sink area for male | 45 |  |  | - 01 Mirror - 02 Sink with tap - 01 Tissue dispenser - 01 Hand dryer - 01 Dustbin |  |  |  |
| 43 | Toilet 1 (Staff use) - male use | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder |  |  |  |
| 44 | Toilet 2 (Staff use) - male use | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder - 01 Dustbin |  |  |  |
| 45 | Toilet sink area for female | 45 |  |  | - 01 Mirror - 02 Sink with tap - 01 Tissue dispenser - 01 Hand dryer - 01 Dustbin |  |  |  |
| 46 | Toilet 3 (Staff use) - female | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder - 01 Dustbin |  |  |  |
| 47 | Toilet 4 (Staff use) - female | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder - 01 Dustbin |  |  |  |
| 48 | Lift 1 (Employees and customers) | 60 |  | Customer and staff use (access controlled for upper floors) |  |  |  | 14 Pax |
| 49 | Lift 2 (Employees and customers) | 60 |  | Customer and staff use (access controlled for upper floors) |  |  |  | 14 Pax |
| 50 | Lift 3 (Food and Pharmaceutical) | 45 |  | Only for food and pharmaceutical lab staffs (access controlled) |  |  |  | 11 Pax |
| 51 | Lift 4 (Infectious Disease) | 45 |  | Only for Infectious Disease lab staffs (access controlled) |  |  |  | 11 Pax |
| 52 | Lift 6 (Heavy Duty Lift) | 80 |  | Only for heavy machineries and for garbage (access controlled) |  |  |  | 2000 Kg lift weight |
| 53 | Corridor with emergency shower |  |  | Walkways and Showers for emergency use | - 02 Shower sets - 01 Rack | - 02 Eye wash kit - 01 First aid kit - 02 Spill kit | - 03 Phones - Camera system |  |
| 54 | Staircase |  |  | Normal use - Provide an access from one floor to another |  |  |  |  |
| 55 | Fire-exit with fire exit doors |  |  | Emergency use: access to go out of the building in case of fire |  |  |  |  |

17.Eighth Floor

| # | Room/Space | Estimated Size (sqft) | Room Type | Function | Non-Medical Items | Medical Items | IT Items | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Equipment room | 200 | Closed | To house, maintain, and manage laboratory instruments and equipment | - Racks - Small work bench with drawers |  |  |  |
| 2 | Freezer Room | 250 | Closed | To store samples, reagents, and other sensitive materials that need to be kept at low temperatures | - Racks - 01 Sink - Water connection and drainage to connect machines | - 01 Freezer (-20 degree Celsius) - 01 Freezer (- 80 degree Celsius) - 01 Fridge (2 to 8 degree Celsius) - Ice maker |  |  |
| 3 | Storage Room | 250 | Closed | Proper management, preservation, and organization of laboratory supplies, reagents, and equipment. | - Racks | - Flammable Storage Cabinets - Chemical Storage Cabinets |  |  |
| 4 | HIV and STI Reference Lab |  |  |  |  |  |  |  |
| 5 | Change Room | 180 | Closed | To change into appropriate personal protective equipment before entering or exiting the lab, | - Lockers - Hangers - Bench to sit down. - 01 Shoe rack (for outdoor foot wear) - 01 Shoe rack (indoor foot wear) - Automatic foot cover machine - 01 sink - 01 dryer - 01 dustbin - Small Work bench with drawers |  |  |  |
| 6 | Sample receiving | 180 | Closed | To handle and process incoming samples | - 01 Counter - 02 Staff chairs - Racks | - Freezers - Refrigerators |  |  |
| 8 | Freezer Room | 180 | Closed | To store reagents, samples, and other materials at low temperatures to maintain their stability | - Work bench with drawers - 01 Sink | - 01 Freezer (-20 degree Celsius) - 01 Freezer (- 80 degree Celsius) - 01 Fridge (2 to 8 degree Celsius) - Ice maker |  |  |
| 9 | Sample Processing Lab (PCR) | 180 | Closed | Dedicated space for the Preparation and initial handling of biological samples before diagnostic testing. | - Work bench with drawers - 02 Stool chairs with back support - 01 Sink | - Centrifuges - Biological Safety Cabinets (BSC) - Extraction machine. |  | Pass box to transfer items |
| 9 | Pre PCR-Lab | 180 | Closed | Space where the preparation of samples for PCR (Polymerase Chain Reaction) testing | - Work bench with drawers - 02 Stool chairs with back support | - 01 PCR workstation (this should in mix room) - 01 Centrifuge - 01 Thermo mixture - Vortex Mixture |  | Pass box to transfer items to PCR room |
| 10 | PCR Mix | 180 | Closed | Dedicated space for preparing the PCR reagents and reaction mixtures | - Work bench with drawers - 02 Stool chairs with back support - Racks | - 01 PCR workstation - 01 Centrifuge - 01 Thermo mixture - 01 Thermo cycler |  | Pass box to transfer items to PCR room |
| 11 | PCR Lab | 180 | Closed | To test for HIV, sexually transmitted infections (STIs), and other pathogen using PCR. | - Work bench with drawers - 02 Stool chairs with back support - Racks | - 02 PCR machine | - 02 Computer system - 01 Phone |  |
| 12 | Sequencing Room | 180 | Closed | To do sequencing of the genetic material (DNA or RNA) of pathogens such as HIV and various sexually transmitted infections (STIs). | - Work bench with drawers - 02 Stool chairs with back support - 01 Rack | - DNA/RNA Sequencers | - 01 Computer system - 01 Phone - 01 Printer |  |
| 13 | Small Store - PCR | 180 | Closed | To store consumables, reagents, equipment, and other supplies | - Racks |  |  |  |
| 14 | Decontamination and waste related |  |  |  |  |  |  |  |
| 15 | Bottle Wash | 150 | Closed | To rinse various pieces of laboratory glassware | 1. Work bench with drawers 2. 02 Stool chairs with back support 3. 04 sinks 4. Drying rack for test tube and bottles | 1. 02 Oven (55 degree Celsius) |  |  |
| 16 | Decontamination Room | 200 | Closed | To properly decontaminated all materials, equipment, and waste before reuse or disposal | - Work bench with drawers - 01 Stool chairs with back support - 2 Sinks - Water inlet for the machine and drainage | - 01 Autoclave (front load) - 01 Autoclave (top load) - 02 Fridge (Degree Celsius) |  | Room should be near bottle wash and sterilization room |
| 17 | Tissue Digester Room | 150 | Closed | Dedicated space designed for the safe and efficient processing of biological waste, particularly animal tissues and other infectious materials. | - Work bench with drawers - 01 Stool chairs with back support - 2 Sinks | - Tissue Digester - Heating System |  |  |
| 18 | Biological Liquid Effluent Decontamination (BLED) Tanks Room | 200 | Closed | Dedicated space for the safe treatment and decontamination of liquid waste that may contain infectious agents. | - Work bench with drawers - 01 Stool chairs with back support - 2 Sinks | - BLED Tanks - Chemical Dosing System - Mixing Mechanism - Filtration System - Monitoring and Control Systems - Emergency Spill Kits |  |  |
| 19 | Biological Liquid Effluent Decontamination (BLED) Tanks Control Room | 160 | Closed | Specialized area dedicated to monitoring, controlling, and managing the decontamination process of liquid waste. | - Work bench with drawers - 01 Stool chairs with back support - 2 sinks | - Control Panel Data Acquisition System - Temperature and Pressure Sensors - Chemical Dosing Control System - Alarm System - Emergency Shutdown Controls | - 02 Computer system - 01 Phone |  |
| 20 | Other virological Diagnostic Lab |  |  |  |  |  |  |  |
| 21 | Change Room | 180 |  | To change into appropriate personal protective equipment before entering or exiting the lab | - Lockers - Hangers - Bench to sit down. - 01 Shoe rack (for outdoor foot wear) - 01 Shoe rack (indoor foot wear) - Automatic foot cover machine - 01 Sink - 01 Dryer - 01 Dustbin - Small Work bench with drawers |  |  |  |
| 22 | Freezer Room | 180 | Closed | To store reagents, samples, and other materials at low temperatures to maintain their stability | - Work bench with drawers - 01 Sink | - 01 Freezer (-20 degree Celsius) - 01 Freezer (- 80 degree Celsius) - 01 Fridge (2 to 8 degree Celsius) - Ice maker |  |  |
|  | Sample receiving | 180 | Closed | To handle and process incoming samples | - 01 Counter - 02 Staff chairs - Racks | - Freezers - Refrigerators |  | Pass box to transfer items |
| 24 | Sample Processing Lab | 180 | Closed | Dedicated space for the Preparation and initial handling of biological samples before diagnostic testing. | - Work bench with drawers - 02 Stool chairs with back support - Racks | - Centrifuges - Biological Safety Cabinets (BSC) |  | Pass box to transfer items |
| 25 | Pre PCR-Lab | 180 | Closed | Space where the preparation of samples for PCR (Polymerase Chain Reaction) testing | - Work bench with drawers - 02 Stool chairs with back support - Racks | - 01 PCR workstation - 01 Centrifuge - 01 Thermo mixture | - 01 Phone | Pass box to transfer items |
| 26 | PCR Mix room | 180 | Closed | Dedicated space for preparing the PCR reagents and reaction mixtures | - Work bench with drawers - 02 Stool chairs with back support - Racks | - 01 PCR workstation - 01 Centrifuge - 01 Thermo mixture - 01 Thermo cycler |  | Pass box to transfer items to PCR room |
| 27 | PCR Lab | 180 | Closed | Dedicated space for amplifying and detecting nucleic acids (DNA or RNA) to diagnose viral infections | - Work bench with drawers - 02 Stool chairs with back support - Racks | - 02 PCR machine - 01 Extraction machine | - 02 Computer system - 01 Phone |  |
| 28 | Sequencing Room | 180 | Closed | To do sequencing of the genetic material (DNA or RNA). | - Work bench with drawers - 02 Stool chairs with back support - 01 Rack | - DNA/RNA Sequencers | - 01 Computer system - 01 Phone - 01 Printer |  |
| 29 | Small Store - PCR | 180 | Closed | To store consumables, reagents, equipment, and other supplies | - Racks |  |  |  |
| 30 | Infected Tissue Culture Room | 250 | Closed | To grow, study, and manipulate viruses in a safe and contained environment for diagnostic, research, and therapeutic purposes | - Work bench with drawers - 02 Stool chairs with back support - Racks - 02 Sinks | - Biological Safety Cabinets - Incubators - Inverted Microscopes - Centrifuges - Refrigerators (4°C) - Freezers (-80°C) - Liquid Nitrogen Freezers |  | Passbox to transfer items between Infected Tissue Culture Room and Clean Tissue Culture Room |
| 31 | Clean Tissue Culture Room | 250 | Closed | Space to sterile culture and maintenance of uninfected cells. | - Work bench with drawers - 02 Stool chairs with back support - Racks - 02 sinks | - Biological Safety Cabinets - Incubators - Inverted Microscopes - Centrifuges - Autoclaves - Refrigerators (4°C) - Freezers (-80°C) - Cryogenic Storage (Liquid Nitrogen Freezers) - Water baths |  |  |
| 32 | Distillation Room | 200 | Closed | Distilling solvents or other chemicals used in microbiological analysis. | - Work bench with drawers - 02 Stool chairs with back support - 02 Sink | - 02 Distillation unit - 01 Ultra-purification system |  | Setup should be made for hot water out |
| 33 | Sterilization Room | 250 | Closed | To clean and sterilize of devices and equipment | - Work bench with drawers - 02 Sink | - 04 Autoclaves (40 to 60 liters) |  | Good ventilation to remove heat. |
| 34 | Dry after Sterilization Room | 180 | Closed | To dry and store glass wear | - Work bench with drawers | - Storage cupboard (Anti-fungus) |  |  |
| 35 | Laundry Room | 250 | Closed | To clean and sterilize of lab clothing, reusable textiles, and other fabric items | - 01 Washing machine - 01 Dryer - 01 Iron board - 01 Iron - Hangers - Cloths lines to dry cloths. - Work bench |  |  |  |
| 36 | Landry storage | 200 | Closed | To store clean lab attire, linens, and other items | - Racks |  |  |  |
| 37 | Quality Section Head - Virology | 150 | Half closed | 01 Staff | - 01 Desk - 01 Chair for the executive - 02 Chairs in front of the table - 01 Filing rack - 01 Dustbin - Artwork, Plant or other decorative items to create a pleasant environment |  |  |  |
| 38 | Unit Head - Quality Unit of Virology grade 2 | 120 | Open | 01 Staff | - 01 Desk - 01 Chair for the executive and - 02 chair in front of the table - 01 Filing cupboard - 01 Dustbin |  |  |  |
| 39 | 5 Staff Quality Unit of Virology grade 2 | 400 | Open | 05 Staff | - 05 Desks with dividers (open concept) - 05 Chairs - 02 Filing rack - 01 Desk for printer setup - 02 Dustbins - 01 Clock |  | - 02 Computer system - 02 Phone - 01 Three in one Printer |  |
| 40 | Section Head - Virology Grade 2 and 3 | 150 | Half closed | 01 Staff | - 01 Desk - 01 Chair for the executive - 02 Chairs in front of the table - 01 Filing rack - 01 Dustbin - Artwork, Plant or other decorative items to create a pleasant environment |  |  |  |
| 41 | Unit Head - Virology Grade 2 | 120 | Open | 01 Staff | - 01 Desk - 01 Chair for the executive and - 02 chair in front of the table - 01 Filing cupboard - 01 Dustbin |  |  |  |
| 42 | 8 Staff - Virology Grade 2 | 640 | Open | 08 Staff | - 08 Desks with dividers (open concept) - 08 Chairs - 02 Filing rack - 01 Desk for printer setup - 03 Dustbins - 01 Clock |  | - 02 Computer system - 02 Phone - 01 Three in one Printer |  |
| 43 | Small Meeting space | 180 | Open | To conduct small urgent discussions. Space 6 people. | - 01 Meeting table - 06 Chairs |  |  |  |
| 44 | Staff resting area | 180 | Closed | Staff to rest | - Sofa set - Table - Rack - Plants |  | - 01 Phone |  |
| 45 | Support staff area | 160 | Closed concept | Area for the support staff to rest and work | - 01 Table - 02 chairs - 01 Rack |  | - 01 Phone |  |
| 46 | Pantry | 180 | Closed concept | Area for the staff to eat and drink | - 01 Dining table - 06 chairs - Kitchen cabinets - 01 Sink - 01 Refrigerator |  | - 01 Phone |  |
| 47 | Network rack space | 50 |  | To mount the network switches of each floor |  |  |  |  |
| 48 | Toilet sink area for male | 45 |  |  | - 01 Mirror - 02 Sink with tap - 01 Tissue dispenser - 01 Hand dryer - 01 Dustbin |  |  |  |
| 49 | Toilet 1 (Staff use) - male use | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder - 01 Dustbin |  |  |  |
| 50 | Toilet 2 (Staff use) - male use | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder - 01 Dustbin |  |  |  |
| 51 | Toilet sink area for female | 45 |  |  | - 01 Mirror - 02 Sink with tap - 01 Tissue dispenser - 01 Hand dryer - 01 Dustbin |  |  |  |
| 52 | Toilet 3 (Staff use) - female | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder - 01 Dustbin |  |  |  |
| 53 | Toilet 4 (Staff use) - female | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder - 01 Dustbin |  |  |  |
| 54 | Lift 1 (Employees and customers) | 60 |  | Customer and staff use (access controlled for upper floors) |  |  |  | 14 Pax |
| 55 | Lift 2 (Employees and customers) | 60 |  | Customer and staff use (access controlled for upper floors) |  |  |  | 14 Pax |
| 56 | Lift 3 (Food and Pharmaceutical) | 45 |  | Only for food and pharmaceutical lab staffs (access controlled) |  |  |  | 11 Pax |
| 57 | Lift 4 (Infectious Disease) | 45 |  | Only for Infectious Disease lab staffs (access controlled) |  |  |  | 11 Pax |
| 58 | Lift 5 (Dumbwaiter lift) | 20 |  | To transfer samples to Infectious Disease Labs (access to only virology labs) |  |  |  | 100 Kg lift weight |
| 59 | Lift 6 (Heavy Duty Lift) | 80 |  | Only for heavy machineries and for garbage (access controlled) |  |  |  | 2000 Kg lift weight |
| 60 | Corridor with emergency shower |  |  | Walkways and Showers for emergency use | - 02 Shower sets - 01 Rack | - 02 Eye wash kit - 01 First aid kit - 02 Spill kit | - 03 Phones |  |

18.Ninth Floor

| # | Room/Space | Estimated Size (sqft) | Room Type | Function | Non-Medical Items | Medical Items | IT Items | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Equipment room | 180 | Closed | To house, maintain, and manage laboratory instruments and equipment | - Racks - Small work bench with drawers |  |  |  |
| 2 | Freezer Room | 180 | Closed | To store samples, reagents, and other sensitive materials that need to be kept at low temperatures | - Racks | - 01 Freezer (-20 degree Celsius) - 01 Freezer (- 80 degree Celsius) - 01 Fridge (2 to 8 degree Celsius) - Ice maker |  |  |
| 3 | Biosafety Level - 3 Lab |  |  |  |  |  |  |  |
| 4 | Donning Room - Decontamination room | 120 | Closed | Space where personnel put on the required Personal Protective Equipment (PPE) before entering the high-containment laboratory area. | - Bench to sit down. - 01 Shoe rack (for outdoor foot wear) - 01 Shoe rack (indoor foot wear) - Automatic foot cover machine - Shoe racks - 01 Sink - Mirror - 01 dryer - 01 Tissue dispenser - 01 Dustbin |  |  | - Negative pressure - Pressure and temperature monitoring and alarm Systems - Airlock for Entry/Exit Doors |
| 5 | Doffing Room -Decontamination room 2 | 120 | Closed | Designated section for removing PPE, which includes suits, gloves, masks, and face shields | - Racks - 01 Sink - Mirror - 01 Dryer - 01 Tissue dispenser - 01 Dustbin | - Biohazard waste bins |  | - Negative pressure - Pressure and temperature monitoring and alarm Systems - Airlock for Entry/Exit Doors |
| 6 | Decontamination room | 180 | Closed | Dedicated space to carry out the decontamination process to ensure that potentially dangerous biological agents do not escape into the environment. | - Work bench with drawers - 01 Stool chairs with back support - 02 Sinks - Water inlet for the machine and drainage | - 01 Autoclave (front load) - 01 Autoclave (top load) - Chemical Decontamination Systems - Waste Decontamination Systems - Storage for Decontaminated Materialism | - Communication Systems (walky-talky) | - Negative pressure - Pressure and temperature monitoring and alarm Systems - Airlock for Entry/Exit Doors |
| 7 | Shower room - Decontamination | 60 | Closed | To shower before leaving the lab | - High-flow shower units - Mirror - Hygiene Stations | - Waste Bins |  | - Negative pressure airflow |
| 8 | Shower room - Decontamination | 60 | Closed | To shower before leaving the lab | - High-flow shower units - Mirrors - Hygiene Stations | - Waste Bins |  | - Negative pressure airflow |
| 9 | Shower room - Decontamination | 60 | Closed | To shower before leaving the lab | - High-flow shower units - Mirror - Hygiene Stations | - Waste Bins |  | - Negative pressure airflow |
| 10 | Anteroom | 120 | Closed | Transition space between the containment area (where hazardous pathogens are handled) and the clean, non-laboratory areas. | - Racks | - Biohazard waste bins | - Communication Systems (walky-talky) | - Negative pressure airflow - Airlock for Entry/Exit Doors - Pressure and temperature monitoring and alarm Systems |
| 11 | Freezer Room | 180 | Closed | To store reagents, samples, and other materials at low temperatures to maintain their stability | - Work bench with drawers - 01 Sink | - 01 Freezer (-20 degree Celsius) - 01 Freezer (- 80 degree Celsius) - 01 Fridge (2 to 8 degree Celsius) - Ice maker |  | - Negative Pressure Airflow Systems, - Airlock for Entry/Exit Doors - Pressure and temperature monitoring and alarm Systems |
| 12 | Sample Processing Lab | 180 | Closed | Dedicated space for the Preparation and initial handling of biological samples before diagnostic testing. | - Work bench with drawers - 02 Stool chairs with back support - Racks | - Centrifuges - Biological Safety Cabinets (BSC) | - CCTV cameras - Communication Systems (walky-talky) | - Pass box to transfer items - Negative Pressure Airflow Systems - Airlock for Entry/Exit Doors - Pressure and temperature monitoring and alarm Systems |
| 13 | Pre PCR-Lab | 180 | Closed | Space where the preparation of samples for PCR (Polymerase Chain Reaction) testing | - Work bench with drawers - 02 Stool chairs with back support - Racks | - 01 PCR workstation - 01 Centrifuge - 01 Thermo mixture | - CCTV cameras - Communication Systems (walky-talky) | - Pass box to transfer items - Negative Pressure Airflow Systems - Airlock for Entry/Exit Doors - Pressure and temperature monitoring and alarm Systems |
| 14 | PCR Mix room | 180 | Closed | Dedicated space for preparing the PCR reagents and reaction mixtures | - Work bench with drawers - 02 Stool chairs with back support - Racks | - 01 PCR workstation - 01 Centrifuge - 01 Thermo mixture - 01 Thermo cycler | - CCTV cameras - Communication Systems (walky-talky) | - Pass box to transfer items - Negative Pressure Airflow Systems, - Airlock for Entry/Exit Doors - Pressure and temperature monitoring and alarm Systems |
| 15 | PCR Lab | 180 | Closed | Dedicated space for amplifying and detecting nucleic acids (DNA or RNA) to diagnose viral infections | - Work bench with drawers - 02 Stool chairs with back support - Racks | - 02 PCR machine - 01 Extraction machine | - 02 Computer system - 01 Phone - CCTV cameras - Communication Systems (walky-talky) | - Pass box to transfer items - Negative Pressure Airflow Systems - Airlock for Entry/Exit Doors - Pressure and temperature monitoring and alarm Systems |
| 16 | Sequencing Room | 180 | Closed | To do sequencing of the genetic material (DNA or RNA) of pathogens such as HIV and various sexually transmitted infections (STIs). | - Work bench with drawers - 02 Stool chairs with back support - Racks | - DNA/RNA Sequencers - PCR Machines - Automated Nucleic Acid Extraction Systems - Centrifuges - Fragment Analyzers - Biosafety Cabinets | - 02 Computer system - 01 Phone - 01 Printer - CCTV cameras - Communication Systems (walky-talky) | - Pass box to transfer items - Negative Pressure Airflow Systems - Airlock for Entry/Exit Doors - Pressure and temperature monitoring and alarm Systems |
| 17 | Small Store - PCR | 180 | Closed | To store consumables, reagents, equipment, and other supplies | - Racks |  |  | - Pass box to transfer items - Negative Pressure Airflow Systems, - Airlock for Entry/Exit Doors - Pressure and temperature monitoring and alarm Systems |
| 18 | Tuberculosis lab | 200 | Closed | Designed to safely isolate, culture, and analyze samples containing Mycobacterium tuberculosis | - Work bench with drawers - 02 Stool chairs with back support - Racks - 02 Sinks | - Biological Safety Cabinets - Autoclaves for Sterilization - Centrifuges - Microscopes - Chemical decontamination systems - UV lights - Biohazard waste disposal | - 02 Computer system - 01 Phone - 01 Printer - CCTV cameras - Communication Systems (walky-talky) | - Pass box to transfer items to sample handling lab. - Negative Pressure Airflow Systems, - Airlock for Entry/Exit Doors - Pressure and temperature monitoring and alarm Systems |
| 19 | Sample Handling Lab | 180 | Closed | dedicated space to the safe handling and analysis of samples suspected of containing the influenza virus | 1. Work bench with drawers 2. 02 Stool chairs with back support 3. Racks 4. 02 Sinks | - Biological Safety Cabinets - Centrifuges - Refrigerators - Freezers - Autoclaves - Microscopes Biohazard waste containers | - CCTV cameras - Communication Systems (walky-talky) | - Negative Pressure Airflow Systems, - Airlock for Entry/Exit Doors - Pressure and temperature monitoring and alarm Systems |
| 20 | Clean tissues culture room | 150 | Closed | Space to sterile culture and maintenance of uninfected cells. | - Work bench with drawers - 02 Stool chairs with back support - Racks - 02 sinks | - Biological Safety Cabinets - Incubators - Inverted Microscopes - Centrifuges - Autoclaves - Refrigerators (4°C) - Freezers (-80°C) - Cryogenic Storage (Liquid Nitrogen Freezers) - Water baths | - CCTV cameras - Communication Systems (walky-talky) | - Negative Pressure Airflow Systems, - Airlock for Entry/Exit Doors - Pressure and temperature monitoring and alarm Systems |
| 21 | Infected tissues culture room | 150 | Closed | Designed space for the safe handling, growth, and analysis of infected cell cultures, particularly those involving highly infectious agents | - Work bench with drawers - 02 Stool chairs with back support - Racks - 02 Sinks | - Biological Safety Cabinets - Centrifuges - Refrigerators - Freezers - Microscopes - Sample Storage Units - Incubators - Waste Disposal Bins | - 02 Computer system - 01 Phone - 01 Printer - Secure entry system - CCTV cameras - Communication Systems (walky-talky) | - Pass box to transfer items - Negative Pressure Airflow Systems, - Airlock for Entry/Exit Doors - Pressure and temperature monitoring and alarm Systems |
| 22 | Animal Biosafety level-3 Lab | 300 | Closed | To safely work with infectious agents that can cause serious or potentially lethal diseases in animals and humans. | - Work bench with drawers - 02 Stool chairs with back support - Racks - 02 Sinks | - Biological Safety Cabinets - Centrifuges - Autoclaves - Microscopes - Imaging Equipment - Anesthesia and Surgical Equipment - Animal Housing Facilities - Sample Storage Units - Waste Disposal Bins - Emergency Equipment | - 02 Computer system - 01 Phone - 01 Printer - Secure entry system - CCTV cameras - Communication Systems (walky-talky) | - Negative Pressure Airflow Systems, - Airlock for Entry/Exit Doors - Pressure and temperature monitoring and alarm Systems |
| 23 | Sterilization Room | 200 | Closed | To clean and sterilize of devices and equipment | - Work bench with drawers - 02 Sink | - 04 Autoclaves (40 to 60 liters) |  | Good ventilation to remove heat. |
| 24 | Dry after Sterilization Room (Glass wear storage) | 180 | Closed | To dry and store glass wear | - Work bench with drawers | - Storage cupboard (Anti-fungus) |  |  |
| 25 | Storage Room | 200 | Closed | Dedicated space for the safe and organized storage of samples, reagents, equipment, and other materials essential for laboratory operations | - Small work bench with drawers - Storage Racks | - 01 Freezer (-80 Degree Celsius) - 01 Refrigerator (4 Degree Celsius) - 01 Flammable Storage Cabinets - 01 Chemical Storage Cabinets |  | - Negative Pressure Airflow Systems - Airlock for Entry/Exit Doors - Pressure and temperature monitoring and alarm Systems |
| 26 | National Influenza center |  |  |  |  |  |  |  |
| 27 | Donning Room | 120 | Closed | Space where personnel put on the required Personal Protective Equipment (PPE) before entering the high-containment laboratory area. | - Bench to sit down. - 01 Shoe rack (for outdoor foot wear) - 01 Shoe rack (indoor foot wear) - Automatic foot cover machine - Shoe racks - 01 Sink - Mirror - 01 Dryer - 01 Tissue dispenser - 01 Dustbin |  |  | - Negative Pressure Airflow Systems - Airlock for Entry/Exit Doors - Pressure and temperature monitoring and alarm Systems |
| 28 | Doffing Room | 120 | Closed | Designated section for removing PPE, which includes suits, gloves, masks, and face shields | - Racks - 01 Sink - Mirror - 01 Dryer - 01 Tissue dispenser - 01 dustbin | - Biohazard waste bins |  | - Negative Pressure Airflow Systems - Airlock for Entry/Exit Doors - Pressure and temperature monitoring and alarm Systems |
| 29 | Freezer Room | 180 | Closed | To store reagents, samples, and other materials at low temperatures to maintain their stability | - Work bench with drawers - 01 Sink | - 01 Freezer (-20 degree Celsius) - 01 Freezer (- 80 degree Celsius) - 01 Fridge (2 to 8 degree Celsius) - Ice maker |  | - Negative Pressure Airflow Systems - Airlock for Entry/Exit Doors - Pressure and temperature monitoring and alarm Systems |
| 30 | Sample Processing Lab | 180 | Closed | Dedicated space to the safe handling and analysis of samples suspected of containing the influenza virus | - Work bench with drawers - 02 Stool chairs with back support - Racks - 02 Sinks | - Centrifuges - Biological Safety Cabinets (BSC) |  | - Negative Pressure Airflow Systems - Airlock for Entry/Exit Doors - Pressure and temperature monitoring and alarm Systems |
| 31 | Pre PCR-Lab | 180 | Closed | Space where the preparation of samples for PCR (Polymerase Chain Reaction) testing | - Work bench with drawers - 02 Stool chairs with back support - Racks | - 01 PCR workstation - 01 Centrifuge - 01 Thermo mixture |  | - Pass box to transfer items - Negative Pressure Airflow Systems - Airlock for Entry/Exit Doors - Pressure and temperature monitoring and alarm Systems |
| 32 | PCR Mix room | 180 | Closed | Dedicated space for preparing the PCR reagents and reaction mixtures | - Work bench with drawers - 02 Stool chairs with back support - Racks | - 01 PCR workstation - 01 Centrifuge - 01 Thermo mixture - 01 Thermo cycler |  | - Pass box to transfer items - Negative Pressure Airflow Systems - Airlock for Entry/Exit Doors - Pressure and temperature monitoring and alarm Systems |
| 33 | PCR Lab | 180 | Closed | Designed space for conducting polymerase chain reaction (PCR) assays to detect and analyze the influenza virus | - Work bench with drawers - 02 Stool chairs with back support - Racks | - 02 PCR machine - 01 Extraction machine | - 02 Computer system - 01 Phone | - Pass box to transfer items - Negative Pressure Airflow Systems - Airlock for Entry/Exit Doors - Pressure and temperature monitoring and alarm Systems |
| 34 | Sequencing Room | 180 | Closed | To analyze viral strains, understand mutations, and support research and diagnostics related to influenza. | - Work bench with drawers - 02 Stool chairs with back support - Racks | - DNA/RNA Sequencers - PCR Machines - Automated Nucleic Acid Extraction Systems - Centrifuges - Fragment Analyzers - Biosafety Cabinets | - 02 Computer system - 01 Printer - 01 Phone | - Pass box to transfer items - Negative Pressure Airflow Systems - Airlock for Entry/Exit Doors - Pressure and temperature monitoring and alarm Systems |
| 35 | Small Store - PCR | 180 | Closed | To store consumables, reagents, equipment, and other supplies | - Racks |  |  | - Negative Pressure Airflow Systems - Airlock for Entry/Exit Doors - Pressure and temperature monitoring and alarm Systems |
| 36 | Infected Tissue Culture Room | 200 | Closed | To grow, study, and manipulate viruses in a safe and contained environment for diagnostic, research, and therapeutic purposes | - Work bench with drawers - 02 Stool chairs with back support - Racks - 02 Sinks | - Biological Safety Cabinets - Incubators - Inverted Microscopes - Centrifuges - Refrigerators (4°C) - Freezers (-80°C) - Liquid Nitrogen Freezers | - 02 Computer system - 02 Printers - 01 Phone | - Pass box to transfer items clean tissue culture room - Negative Pressure Airflow Systems - Airlock for Entry/Exit Doors - Pressure and temperature monitoring and alarm Systems |
| 37 | Clean Tissue Culture Room | 200 | Closed | Space to sterile culture and maintenance of uninfected cells. | - Work bench with drawers - 02 Stool chairs with back support - Racks - 02 Sinks | - Biological Safety Cabinets - Incubators - Inverted Microscopes - Centrifuges - Autoclaves - Refrigerators (4°C) - Freezers (-80°C) - Cryogenic Storage (Liquid Nitrogen Freezers) - Water baths | - 02 Computer system - 02 Printers - 01 Phone | - Negative Pressure Airflow Systems - Airlock for Entry/Exit Doors - Pressure and temperature monitoring and alarm Systems |
| 38 | Serology Room | 200 | Closed | To diagnose infections, monitoring immune responses, and conducting research on vaccine efficacy and immunity | - Work bench with drawers - 02 Stool chairs with back support - Racks - 02 sinks | ELISA (Enzyme-Linked Immunosorbent Assay) Plates, Microplate Readers, Centrifuges, Water Baths, Gel Electrophoresis Equipment, Immunofluorescence Microscopes, Biohazard waste containers |  | - Negative Pressure Airflow Systems - Airlock for Entry/Exit Doors - Pressure and temperature monitoring and alarm Systems |
| 39 | Buffer Zone | 100 | Closed | To avoid the entrance of particle and microbial contaminants |  |  |  | - Air pressure-controlled area - Air curtains |
| 40 | Decontamination Room | 200 | Closed | To clean and sterilized contaminated instruments | - Work bench with drawers - 01 Stool chairs with back support - 2 Sinks - Water inlet for the machine and drainage | - 01 Autoclave (front load) - 01 Autoclave (top load) - 02 Fridge (Degree Celsius) | - 01 Phone | Room should be near bottle wash and sterilization room |
| 41 | Bottle wash | 200 | Closed | To rinse various pieces of laboratory glassware | - Work bench with drawers - 02 Stool chairs with back support - 04 Sinks - Drying rack for test tube and bottles | - 02 Oven (55 degree Celsius) |  |  |
| 42 | Sterilization Room | 200 | Closed | To clean and sterilize of devices and equipment | - Work bench with drawers - 02 Sink | - 04 Autoclaves (40 to 60 liters) |  | Good ventilation to remove heat. |
| 43 | Dry after Sterilization Room | 180 | Closed | To dry and store glass wear | - Work bench with drawers | - Storage cupboard (Anti-fungus) |  |  |
| 44 | Laundry Room | 180 | Closed | To clean and sterilize of lab clothing, reusable textiles, and other fabric items | - 01 Washing machine - 01 Dryer - 01 Iron board - 01 Iron - Hangers - Cloths lines to dry cloths. - Work bench |  | - 01 Phone |  |
| 45 | Landry storage | 150 | Closed | To store clean lab attire, linens, and other items | - Racks |  |  |  |
| 46 | Unit Head - Virology Grade 3 | 120 | Open | 01 Staff | - 01 Desk - 01 Chair for the executive and - 02 chair in front of the table - 01 Filing cupboard - 01 Dustbin |  |  |  |
| 47 | 5 Staff - Virology Grade 3 | 400 | Open | 05 Staff | - 05 Desks with dividers (open concept) - 05 Chairs - 02 Filing rack - 01 Desk for printer setup - 02 Dustbins - 01 Clock |  | - 02 Computer system - 02 Phone - 01 Three in one Printer |  |
| 48 | Unit Head - Quality Unit of Virology grade 3 | 120 | Open | 01 Staff | - 01 Desk - 01 Chair for the executive and - 02 chair in front of the table - 01 Filing cupboard - 01 Dustbin |  |  |  |
| 49 | 3 Staff Quality Unit of Virology grade 3 | 240 | Open | 03 Staff | - 03 Desks with dividers (open concept) - 03 Chairs - 01 Filing rack - 01 Desk for printer setup - 02 Dustbins - 01 Clock |  | - 01 Computer system - 01 Phone - 01 Three in one Printer |  |
| 50 | Small Meeting space | 180 | Open | To conduct small urgent discussions. Space 6 people. | - 01 Meeting table - 06 Chairs |  |  |  |
| 51 | Staff resting area | 180 | Closed | Staff to rest | - Sofa set - Table - Rack - Plants |  | - 01 Phone |  |
| 52 | Support staff area | 160 | Closed | Area for the support staff to rest and work | - 01 Table - 02 chairs - 01 Rack |  | - 01 Phone |  |
| 53 | Pantry | 180 | Closed | Area for the staff to eat and drink | - 01 Dining table - 06 chairs - Kitchen cabinets - 01 Sink - 01Refrigerator |  | - 01 Phone |  |
| 54 | Network rack space | 50 |  | To mount the network switches of each floor |  |  |  |  |
| 55 | Toilet sink area for male | 45 |  |  | - 01 Mirror - 02 Sink with tap - 01 Tissue dispenser - 01 Hand dryer - 01 Dustbin |  |  |  |
| 56 | Toilet 1 (Staff use) - male use | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder - 01 Dustbin |  |  |  |
| 57 | Toilet 2 (Staff use) - male use | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder - 01 Dustbin |  |  |  |
| 58 | Toilet sink area for female | 45 |  |  | - 01 Mirror - 02 Sink with tap - 01 Tissue dispenser - 01 Hand dryer - 01 Dustbin |  |  |  |
| 59 | Toilet 3 (Staff use) - female | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder - 01 Dustbin |  |  |  |
| 60 | Toilet 4 (Staff use) - female | 35 |  |  | - 01 Toilet set - 01 Hand shower - 01 Toilet paper holder - 01 Dustbin |  |  |  |
| 61 | Lift 1 (Employees and customers) | 60 |  | Customer and staff use (access controlled for upper floors) |  |  |  | 14 Pax |
| 62 | Lift 2 (Employees and customers) | 60 |  | Customer and staff use (access controlled for upper floors) |  |  |  | 14 Pax |
| 63 | Lift 3 (Food and Pharmaceutical) | 45 |  | Only for food and pharmaceutical lab staffs (access controlled) |  |  |  | 11 Pax |
| 64 | Lift 4 (Infectious Disease) | 45 |  | Only for Infectious Disease lab staffs (access controlled) |  |  |  | 11 Pax |
| 65 | Lift 5 (Dumbwaiter lift) | 20 |  | To transfer samples to Infectious Disease Labs (access to only virology labs) |  |  |  | 100 Kg lift weight |
| 66 | Lift 6 (Heavy Duty Lift) | 80 |  | Only for heavy machineries and for garbage (access controlled) |  |  |  | 2000 Kg lift weight |
| 67 | Corridor with emergency shower |  |  | Walkways and Showers for emergency use | - 02 Shower sets - 01 Rack | - 02 Eye wash kit - 01 First aid kit - 02 Spill kit | - 03 Phones |  |

19.Terrace

| # | Room/Space | Estimated Size (sqft) | Room Type | Function | Non-Medical Items | Medical Items | IT Items | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Recreational area | 3500 | Closed | Space where people can engage in various activities that promote physical well-being, mental relaxation, and social engagement. |  |  |  |  |
| 2 | Dry Ice Unit | 250 | Closed | To make dry ice to store samples, transfer samples. | - Work desk - Chair - Cupboard | - Dry ice machine |  |  |
| 3 | Catering area | 500 | Closed | Designated space where employees can dine in | - 5 Tables - 20 Chairs - 01 Sink - Cabinets |  |  |  |
| 4 | Document Archive Store | 500 | Closed | To store documents | - Racks |  |  |  |
|  | Drone helipad | 300 | Open | Ensures safe landing, takeoff, and operation of drones to support lab sample delivery | - Guardrails or barriers |  | - Lighting and Navigation |  |
| 5 | HAVC system Outdoor units |  |  |  |  |  |  |  |
| 6 | Solar panels |  |  |  |  |  |  |  |
| 7 | Water tanks |  |  |  |  |  |  |  |
| 9 | Jib crane |  |  | To lift heavy items to the building (2000kg to 3500kg lift weight) |  |  |  |  |