Section 6: Employer’s Requirements

Table of Contents

Specifications	6-2

Environmental, Heath and Safety Management Requirement	6-3

Drawings	6-4

Supplementary Information Regarding Works to Be Procured	6-5

Personnel Requirements	6-6

Equipment Requirements	6-7

Part 1: Scope

The Enhancing Climate Resilience and Food Security Project (the Project), financed by the Asian Development Bank (ADB, Project Nr. 57002-001), aims to strengthen the climate resilience of inhabited islands and agri-food systems by enhancing the country’s weather monitoring capacity and safeguarding urban infrastructure, agricultural productivity and food security. As part of Output 3 of the Project - Enhanced resilience of supply capacity of islands urban farming and agribusiness - the existing Hanimaadhoo Agriculture Center (HAC) facilities on Hanimaadhoo island in Haa Dhaalu Atoll will be upgraded into a multipurpose facility.

Starting in 1994 as an agricultural nursery but expanded in 2000 to a proper research and training center, the HAC operates under the Training, Extension and Adaptive Research Section of the Ministry of Agriculture and Animal Welfare (MAAW) with the goal to play a central role in Maldives as an agriculture advisory center for knowledge generation and dissemination. It has also been the main residence of most of the agriculture training and extension personnel, particularly to farmers from the northern atolls who have direct access to the services of HAC.

The Project is financing the detailed design and construction of a new HAC multipurpose facility, which will include:

- The detailed architectural design will include: site inspection, confirmation of user requirements, reviewing the conceptual design, schematic design (detailed floor plans, elevations and sections), structural design, design of mechanical, electrical and plumbing, detailed design, and submit the design for necessary approvals.

- The construction phase will include: preparatory works (site clearance, work camp, site office, storage areas, disposal areas, access roads etc.), civil engineering and construction, roofing, plumbing and electrical work, air-conditioning and ventilation, telephone and internet line connection, finishing, and inspection and handover.

The new HAC multipurpose facility will be constructed at the current HAC premises in Hanimaadhoo. The HAC is located on the eastern side of Hanimaadhoo island, occupying a plot assigned by the local island council. The new HAC multipurpose facility will be constructed on the northern side of the current plot (see drawings, Figure 3).

The Client of the Contract resulting from this call for tenders will be called the Employer. It will be the Government of Maldives, which includes MCCEE, the Project Management Unit (PMU) located in MCCEE, MAAW and any other delegated authority as well.

For technical and contractual purposes, the Employer shall appoint a Project Manager to represent the Employer. Upon agreement with the Client, the Employer may delegate the authority/responsibility of the Project Manager to the "Team Leader of the Construction Supervision Consultant", a Firm that is planned to be hired under the Project.

The terms ‘the Contractor’ by and large will include the Lead firm, associate or JV partners, Designer, Civil works sub-contractors, any other sub-contractor, any supplier, any manufacturer, or any service provider etc., including the personnel appointed (contractual/employee/ad-hoc appointee etc.). The Prime Lead Firm will assume the overall responsibility of the contract. Any activity/delivery out of the regular business of the prime/lead firm can be allowed to Sub-contractor(s). The Prime Lead Firm will bear overall responsibility of the contract and coordination/compliance by its Specialized Sub-contractors.

Part 2: General specifications

The Contractor shall design and construct the HAC multipurpose facility meeting the requirements stated in the following sections. Based on the design requirements, a preliminary design concept should be submitted with the bid documents.

- OVERALL BUILDING AND ROOM REQUIREMENTS

The HAC multipurpose facility will be composed of two separate buildings: an office building and a laboratory building, with a distance of 10m between the two buildings. The office building will have a total of three floors, consisting of a ground floor, first floor and second floor. The laboratory building will have two floors, consisting of a ground floor and first floor.

The required facilities/rooms/spaces for each floor are listed below. The unit area indicated for each facility/room/space is a minimum value, only to be used as a design guideline. The Contractor is allowed to relocate items between floors, provided that the access requirements are met and per agreement with the Employer.

- Office Building

1 lift will be placed in the office building.

The following facilities/rooms/spaces are required per each floor of the office building:

  - Ground Floor

    - Lobby

- A small lobby area with reception should be allocated on the ground floor which acts as the main reception area for all offices in the building.

    - Meeting room

- There should be a meeting room adjacent to the lobby with capacity for 10 people.

- The meeting room should be accessible from the lobby as well as the office space.

    - Kiosk

- A small kiosk area should be allocated on the ground floor which is accessible to the public.

    - Guest Toilet

- There should be a guest toilet/washing room which is accessible from the lobby.

    - Office

- There should be sufficient office space with one main office area and a separate room for management.

- There should be a toilet/washing room located adjacent to the office space.

    - Storage Spaces

- The ground floor should have sufficient space for Office Storage, Cold Storage, Perish Storage and Dry Storage.

    - Multipurpose Hall

- A multipurpose hall or event space shall be provided on the ground floor which can accommodate 75 people.

- The main access to the multipurpose hall shall be provided directly from outside.

- The multipurpose hall shall be accessible to Persons with Disabilities (PWDs) (for details see section F).

- There should be a separate storage room adjacent to the multipurpose hall for storage of all relevant IT equipment.

- There should be a toilet/washing room located adjacent to the multipurpose hall.

    - AV Room

- The multipurpose hall will have an adjacent Audiovisual (AV) Room with a capacity for 12 people.

    - Control + Server Room

- The ground floor will have a control and server room.

    - Parking

- The ground floor should contain parking for approximately 9 motorcycles and 2 cars with adjacent washing area.

  - First Floor

    - Common Area

- There will be an open space common area.

    - Kitchen

- The kitchen will be adjacent to the mess room and accessible from the mess room.

    - Mess Room

- The mess room will have a capacity of approximately 25 people.

    - Bedrooms

- The first floor will have 12 bedrooms.

- Each room will have a separate bathroom, with toilet.

- Each room should be able to accommodate two single beds and a wardrobe.

- All rooms should have access to the balcony.

    - Laundry

    - Prayer Room

- There will be an ablution area adjacent to and accessible from the prayer room.

  - Second Floor

    - Common Area

- There will be an open space common area.

    - Mess Room

- The mess room will have a capacity of approximately 25 people.

    - Bedrooms

- The second floor will have 13 bedrooms.

- Each room will have a separate bathroom, with toilet.

- Each room should be able to accommodate two single beds and a wardrobe.

- All rooms should have access to the balcony.

    - Laundry

    - Prayer Room

- There will be an ablution area adjacent to and accessible from the prayer room.

- Laboratory Building

The following facilities/rooms/spaces are required per each floor of the laboratory building:

  - Ground Floor

    - Chemical Storage

    - Soil Laboratory

    - Toilet

  - First Floor

    - Pathology Laboratory

    - Entomology Laboratory

    - Toilet

- AREA REQUIREMENTS

The area requirements for the Office Building are as follows (Table 1):

Table 1: Indicative area requirements for the rooms/spaces in the Office Building

| FLOOR | SPACES | NO. | AREA(SQM) | TOTAL AREA(SQM) |
| --- | --- | --- | --- | --- |
| Ground Floor | Lobby | 1 | 16.24 | 16.24 |
|  | Meeting Room | 1 | 16.8 | 16.8 |
|  | Kiosk | 1 | 9.74 | 9.74 |
|  | Toilet | 1 | 3.58 | 3.58 |
|  | Guest Toilet | 1 | 5.93 | 5.93 |
|  | Office | 1 | 97.26 | 97.26 |
|  | Office Storage | 1 | 4.46 | 4.46 |
|  | Perish Storage | 1 | 25.12 | 25.12 |
|  | Dry Storage | 1 | 22.93 | 22.93 |
|  | Cold Storage | 1 | 21.97 | 21.97 |
|  | Multipurpose Hall | 1 | 78.52 | 78.52 |
|  | Multipurpose Hall Washroom | 1 | 5.17 | 5.17 |
|  | Multipurpose Hall Storage | 1 | 4.48 | 4.48 |
|  | AV Room | 1 | 29.7 | 29.7 |
|  | Control+Server Room | 1 | 11.29 | 11.29 |
|  | Car Parking | 1 | 43.42 | 43.42 |
|  | Motorcycle Parking | 1 | 43.58 | 43.58 |
| First Floor | Common Area | 1 | 81.43 | 81.43 |
|  | Kitchen | 1 | 14.92 | 14.92 |
|  | Mess Room | 1 | 30.67 | 30.67 |
|  | Room-Type 1 | 12 | 13.67 | 164.04 |
|  | Toilet-Type 1 | 12 | 3.17 | 38.04 |
|  | Laundry | 1 | 25.79 | 25.79 |
|  | Prayer Room | 1 | 16.84 | 16.84 |
| Second Floor | Common Area | 1 | 81.43 | 81.43 |
|  | Mess Room | 1 | 30.67 | 30.67 |
|  | Room-Type 1 | 13 | 13.67 | 177.71 |
|  | Toilet-Type 1 | 13 | 3.17 | 41.21 |
|  | Laundry | 1 | 25.79 | 25.79 |
|  | Prayer Room | 1 | 14.12 | 14.12 |

The area requirements for the Laboratory Building are as follows (Table 2):

Table 2: Indicative area requirements for the rooms/spaces in the Laboratory Building

| FLOOR | SPACES | NO. | AREA(SQM) | TOTAL AREA(SQM) |
| --- | --- | --- | --- | --- |
| Ground Floor | Soil Lab | 1 | 127.92 | 141.96 |
|  | Toilet | 1 | 3.05 | 141.91 |
|  | Chemical Storage | 1 | 127.92 | 141.91 |
| First Floor | Pathology Lab | 1 | 101.42 | 113.51 |
|  | Entomology Lab | 1 | 101.43 | 113.51 |
|  | Toilet | 1 | 3.05 | 3.05 |

- FINISHES REQUIREMENTS

The finishes requirement for different rooms/ facilities are given below (Table 3):

Table 3: Finishes requirements for the rooms/spaces in the Office and Laboratory Buildings

| FLOOR | SPACE | WALL FINISHES | FLOOR FINISHES |
| --- | --- | --- | --- |
| Ground Floor | Lobby | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Meeting Room | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Kiosk | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Toilet | 300 x 300 homogenous wall tiles | 300 x 300 homogenous wall tiles |
|  | Guest Toilet | 300 x 300 homogenous wall tiles | 300 x 300 homogenous wall tiles |
|  | Office | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Office Storage | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Perish Storage | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Dry Storage | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Cold Storage | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Multipurpose Hall | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Multipurpose Hall Washroom | 300 x 300 homogenous wall tiles | 300 x 300 homogenous floor tiles |
|  | Multipurpose Hall Storage | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | AV Room | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Control+Server Room | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Car Parking | Ground smooth finished with selected emulsion Paint | Self Levelling cement floor screed with epoxy floor paint |
|  | Motorcycle Parking | Ground smooth finished with selected emulsion Paint | Self Levelling cement floor screed with epoxy floor paint |
|  | Washing area | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Soil Lab | 300 x 300 homogenous wall tiles | 600 x 600 Homogenous non-skid floor tiles |
|  | Chemical Storage | 300 x 300 homogenous wall tiles | 600 x 600 Homogenous non-skid floor tiles |
|  | Veranda | Ground smooth finished with selected emulsion Paint | Landscaping / White sand |
|  | Walkway | Ground smooth finished with selected emulsion Paint | Landscaping / White sand |
| First Floor | Common Area | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Kitchen | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Mess Room | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Walkway 1 | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Walkway 2 | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Room-Type 1 | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Room-Type 2 | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Toilet-Type 1 | 300 x 300 homogenous wall tiles | 300 x 300 homogenous floor tiles |
|  | Laundry | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Prayer Room | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Balcony 1 | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Balcony 2 | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Pathology Lab | 300 x 300 homogenous wall tiles | 600 x 600 Homogenous non-skid floor tiles |
|  | Entomology Lab | 300 x 300 homogenous wall tiles | 600 x 600 Homogenous non-skid floor tiles |
|  | Toilet-Type 2 | 300 x 300 homogenous wall tiles | 300 x 300 homogenous floor tiles |
|  | Walkway 3 | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
| Second Floor | Common Area | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Study Area | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Prayer Room | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Walkway 1 | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Walkway 2 | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Room-Type 1 | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Room-Type 2 | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Toilet | 300 x 300 homogenous wall tiles | 300 x 300 homogenous floor tiles |
|  | Laundry | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Balcony 1 | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |
|  | Balcony 2 | Ground smooth finished with selected emulsion Paint | 600 x 600 Homogenous non-skid floor tiles |

As for the ceilings, the following should be applied:

- The ceilings of all building areas other than the toilets shall be constructed using reinforced concrete slabs with selected emulsion paint.

- Suspended ceilings shall be provided in all toilet areas as part of the finishing works.

- ENERGY AND WATER EFFICIENCY OPTIMIZATION

Certain principles should be followed in the design and construction stages to optimize energy and water efficiency. These principles are stated below:

- Use of passive design strategies like building orientation, daylighting and natural ventilation to improve the thermal comfort and reduce energy use.

- Ensure proper insulation of walls, roofs and windows to reduce heat gain.

- Use of energy-efficient lighting, such as LEDs, and controls like daylight and occupancy sensors to reduce unnecessary lighting energy use.

- Rainwater harvesting system, capturing at least 25% of run-off volumes from the roof area for collection and reuse or connect to existing stormwater collection network if any.

- HVAC equipment’s and home appliances to comply with the “Hakathari” program approved brands introduced by the Ministry of Climate Change, Environment and Energy to promote energy-efficient appliances and equipment.

- The solar system installed should offset at least 2.5% of the total energy building costs.

- Post-occupancy energy management strategies to further reduce energy use during the operational phase of the building.

A solar panel system is required, installed on the roof area available from the laboratory building with minimal shading and meeting the following requirements:

Table 5: Requirements for the rooftop solar PV system

| Specifications | Units | Number |
| --- | --- | --- |
| Available Area | sqm | 170 |
| Rated Power | Watts | >400Wp-650 |
| Performance Ratio |  | 0.85-0.90 |
| Total Power of System | kWp | >24 kWp |

The system is expected to provide on-site renewable energy generation, contributing to reduced grid dependency and lowering operational energy costs.

The estimated solar photovoltaic (PV) system for the laboratory building is designed with a capacity higher than 24 kWp , comprising solar panels of  >400 kWp. These panels will occupy a total roof area of 170 m².

The installation should meet the following minimum requirements:

- 1. Photovoltaic Modules:

- Modules must conform to IEC 61215, IEC 61701, and IEC 61730 Part I & II, IEC 62804

- Monocrystalline silicon PERC half cut cell modules with aluminum frames and durable face covers.

- Tested and packaged to withstand shipping without damage.

- Product warrantee > 10 years - Material warranty guaranteeing the modues are free from defects and/or failures due to manufacturing defects and/or failures due to materials, including PID defects and (ii) non-conformity to specifications due to faulty manufacturing and/or inspection processes

- PV modules shall be guarantee 25 years power performance with not more than 1% power degradation in first year and 0.4% annual power attenuation

- PV Modue Efficiency: Above 20%

- Panel Wattage should be 400W or higher

- PV module brands must be from reputable and renowned manufacturers (Tier I Manufacturers)

- 2. Grid-Tied Inverter:

- Inverters sufficienty sized to meet the > 24-kW PV System (String inverters idea for redundancy requirements)

- AC Output – 3-phase, 400V (3/N/PE), 50 Hz ±2.5 Hz

- Inverters of multiple MPPT’s (minimum 2 MPPTs)

- High-efficiency inverters (max efficiency exceeding 98%).

- Capable of remote monitoring via the internet.

- Power Quality & Grid functionality –

Total Harmonic Distortion (THD): <3%

Adjustable Power Factor: 0.8 leading to 0.8 lagging

- Grid Support functions

LVRT/HVRT (Preferrred)

Power factor control

Ramp rate control (Preferred)

Anti-isanding

Frequency/votage protection

- Other Mandatory Protection features

DC Insulation monitoring / faut detection

Earth/Ground faut monitoring and protection

DC and AC Surge protection

DC side disconnector (Isoator)

Integrated string fuse holders or circuit breakers for over-current prtotection

Reverse polarity protection

Residual current detection

- Monitoring, Contro and Communication

Local display: LCD / touchscreen showing string-level DC voltage/current, total DC input, AC output voltage/current/power/frequency, total energy produced (kWh), daily energy (kWh), alarms/faults, etc
Remote monitoring systemreal-time monitoring of production (DC & AC), per-MPPT data, fault detection & diagnosis, event log, remote firmware updates.

Communication interfaces: RS485 (Modbus RTU) minimum

- Minimum IP65 rated for outdoor use, preferably IP66 rated

- Utilizes natural cooling technology.

- 3. Combiner Box:

- IP65-rated distribution boards and combiner boxes adhering to IEC 62271.

- Properly mounted with durable materials resistant to environmental conditions.

- 4. Earthing System:

- A complete and reliable earthing system designed for the PV installation.

- 5. Mounting Structures:

- Anodized Aluminium or corrosion-resistant materials.

- Designed to withstand wind speeds of at least 60 km/h.

- Guaranteed stability for 12 years in harsh environments.

- 6. Cables:

- All DC cables must be tinned copper, corrosion-resistant, chemical-resistant, and UV-resistant.

- All cables should be enclosed in conduits or cable trays.

- All AC cables must be copper with UV protection.

- 7. Other Details

- The contractor is responsible for all regulatory and service documentation required for permits, net metering, and grid connection. Relevant signatures from the client will be provided, but the contractor must handle all other formalities.

- ACCESSIBILITY AND INCLUSIVE DESIGN

The HAC multipurpose facility will be designed and constructed to ensure the building is inclusive for all, in line with international best practice on inclusive design:

- All areas of the building shall be accessible to PWDs. This could be through including ramps, lifts, wide doorways, lower reach ranges, avoiding protruding objects, and accessible restrooms.

- Each floor should have washrooms, showers, toilets, prayer rooms etc. that are accessible to PWDs and women, ensuring privacy, dignity and independent use.

- Signage should be clear and easy-to understand to help everyone navigate the building. Tactile and auditory signs for those with visual or hearing impairments should be considered.

- Spaces will be designed with appropriately placed good lighting, clear sightlines and secure access points to ensure safety and security.

Part 3: Technical specifications

- PRELIMINARIES

A.1 	Standard and Codes

A.1.1	The Contractor shall, perform the Works in compliance with all regulations, standard specifications or statutes of the Government of Maldives unless otherwise conform to this specification.

A.1.2 	The current British Standard Specifications and Codes of Practice shall apply to and form part of these specifications unless otherwise specified in respect of all materials and works to which they have application.

A.2 	Drawings and Specifications

A.2.1	Drawings and Specifications are intended to complement each other, so that if anything is shown on the Drawings, but not mentioned in the specifications or vice versa, it is to be furnished and built as though specifically set forth in all three. If any discrepancies, errors, ambiguities or omissions occur in the Drawings or Specifications, the same shall be referred to the Employer before proceeding with the Works, and the Employer decision on such discrepancies, errors, ambiguities or omissions shall be final.

A.2.2 	In addition to the Drawings and Specifications attached hereto, the Employer will during the progress of the Works, require the Contractor to furnish additional Drawings, per specifications, and instructions as may be necessary, in the opinion of the Employer for the purpose of the proper and adequate execution and maintenance of the Works, and the Contractor shall make their work conform. Such drawings and instructions shall be deemed to be part of the Contract Documents. In Compliance with the laws and regulations, the necessary drawings submitted by the Contractor must be endorsed with official stamps from a registered engineer and architect duly authorized by the Ministry of Construction, Housing and Infrastructure of the Maldives.

A.3	Transportation to the Site

A.3.1	The Contractor shall provide all necessary transport, handling and storage of all materials, components and the like to their points of installation on site including transport to and from storage. The Contractor shall provide all necessary transport of labour to and from the site.

A.4	Schedule and Execution Plan

A.4.1	The Contractor shall prepare and submit to the Employer for approval the construction schedule and an execution plan of temporary facilities, stockyards, etc., before the start of the Works.

A.5	Repairing and Correction

A.5.1	Any breakage(s) or defect(s) of existing buildings, road utilities, or part(s) of them caused by the Works including transportation for the works shall be repaired or corrected by the Contractor with his/her responsibility.

A.6	Workmanship and Materials

A.6.1	All workmanship shall be of the best standard. All goods and materials to be incorporated in the Works must be new, unused, of the most recent or current models and incorporate all recent improvements in design and materials unless provided otherwise in the contract.

A.6.2	The Contractor shall submit for the approval of the Employer a list of names and addresses of the manufacturers and trademarks or names of all the various types of materials and goods s/he proposes to use in the Works. The list shall include reference to the specifications clause or article to which the materials and goods apply.

A.6.3	Materials shall be obtained from approved sources and used in accordance with the manufacturer’s printed instructions. In the absence of a specification all materials shall comply with a relevant standard. The employer shall order the removal of any materials which s/he has not approved.

A.6.4	No orders for materials and goods shall be placed until approval has been obtained for the materials and goods from the employer.

A.6.5	The Contractor shall note that it is their responsibility to include the price for the cost of the materials and products as specified and no adjustment will be allowed should the employer reject the alternatives.

A.7	Obvious Work

A.7.1	Where an item of work is obviously required for the type of work being undertaken then it shall be deemed to have been included even though the item is not specifically mentioned or shown in the Drawings or Specifications.

A.8	Protection

A.8.1	The Contractor shall have the Works and adjoining properties protected from inclement weather. Any loss or damage caused by weather, carelessness or lack of skill of workers, accident or otherwise shall be of such property that is affected. The Contractor shall provide all necessary dustsheets, barriers and guardrails and clear them away at completion.

A.8.2	The work shall be suspended for such time as may be directed and/or approved by the Employer if the specified quality of work is difficult to maintain during inclement weather.

A.9	Scaffolding

A.9.1	The Contractor shall provide, erect, maintain, dismantle and clear away at completion proper and adequate including that required for subcontractor and suppliers. Putlog holes shall be made good to match the adjacent surface as the scaffolding is dismantled.

A.9.2	The Contractor shall be responsible for all safety precautions in connection with the scaffolding including the provision of all bracing, scaffold boards, toe boards and the like and for entire sufficiency for the work.

A.10	Construction Machinery, Plants and Equipment’s

A.10.1	All necessary construction machines shall be provided and maintained by the Contractor and shall be approved by the Employer.

A.10.2	If cranes or any other type of plant which places any load on the structure are proposed, all details of such plant shall be submitted to the Employer for approval before the work is actually commenced. If approved by the Employer and contractually acceptable, permission may be given for the structure to be strengthened, in order to carry out loads, and the Contractor shall be responsible for any resulting additional costs.

A.10.3	The Contractor shall be responsible for making good to the satisfaction of the Employer any damage to the permanent structure that may be caused by his/her plant and equipment.

A.11	Samples

A.11.1	The Contractor shall furnish for the approval with reasonable promptness, all samples as directed by the employer. The Employer shall check and approve such materials with reasonable promptness only for conformance with the design concept of the Works and for compliance with the information given in the Contract Document. The Work shall be in accordance with the approved samples

A.11.2	All samples shall be delivered to the Employer’s office with all charges in connection therewith paid by the Contractor and deemed to be included in the Contract Price.

A.11.3	Duplicate final approved samples, in addition to any required for the Contractor’s use, shall be furnished to the Employer, one for office use and one for the site.

A.11.4	Samples shall be furnished so as not to delay fabrication, allowing the employer reasonable time for consideration of the sample submitted.

A.11.5	Each sample shall be properly labelled with the name and quality of the material, manufacturer’s name, name of project, the contractor’s name and date of submission, and the specification clause to which the sample refers.

A.12 	Ordering Materials

A.12.1	The Bills of Quantities shall not be used as a basis for ordering materials and the Contractor is entirely responsible for assessing the quantities of materials to be ordered.

A.12.2	Upon receipt of the Employer’s order to commence the Works, the Contractor shall immediately place orders for all required materials and will be held responsible for any delays occurring due to late placing of such orders.

A.12.3	The Contractor shall pay all expenses, taxes and dues etc. incurred on the procurement of materials from abroad

A.13	Water and Electricity for the Works

A.13.1	The Contractor shall make all necessary arrangements and provide all water for the proper execution of the Works, together with all transport, temporary plumbing, storage and distribution, pay all charges and alter, adept and maintain temporary work as necessary and remove and make good at completion.

A.13.2 	The Contractor shall make all necessary arrangements and provide all artificial lighting and power (maintain a generator if necessary) for the proper execution and security of the Works and its protection, with all meters, temporary wiring and fittings, pay all charges and alter adapt and maintain the temporary work as necessary and remove and make good at completion.

A.14	Site Offices for Contractor

A.14.1	The Contractor shall provide maintain and clear away on completion of the Contract all necessary site offices, canteens, messing and welfare facilities, temporary buildings, toilets and the like for all site staff employed by the Contractor and required by subcontractors and suppliers.

A.14.2	The offices shall be open at all normal working hours to receive instructions, notices and other communications.

A.14.3	The Contractor shall obtain the approval of the Employer of the proposed site layout, type and drainage arrangement of all the buildings prior to erection of same. All buildings shall be supplied and maintained in good condition and of neat appearance, all maintenance to same as instructed by the Employer shall be carried out at the Contractor's expense.

A.14.4	Under no circumstances shall overnight accommodation be permitted on Site except for the site watchman in carrying out their duties.

A.15	Contractor’s Site Area

A.15.1	Throughout the period of the Contract the Contractor shall maintain the area of operation within the limits of the Site in a clean, tidy and safe condition by arranging materials and the like in an orderly manner. All rubbish, debris, waste materials and the like shall be systematically cleared from the Site as it accumulates.

A.15.2	The Contractor shall take all steps necessary as directed by the Employer to minimize or eliminate dust, noise or any other nuisance which may occur. Plant emitting dust, smoke, excessive noise or other nuisance shall not be permitted.

A.16	Progress Meetings

A.16.1	During the course of the Works, progress meetings shall be held at fortnightly intervals for the purpose of coordinating the Contractor's works and to ensure that full compliance is maintained.

A.16.2	Minutes of such meetings should be recorded; copies will be distributed to all persons concerned and full effect shall be given to all instructions contained therein.

A.16.3	Prior to such meetings the Contractor shall give the Employer's Representative details in writing of that portion of the Works s/he proposes to construct during the coming two weeks with details of the plant and method s/he proposes to employ. These proposals shall be discussed at the meeting and no work based on such proposals shall proceed without the approval of the Employer's Representative.

A.16.4	The Contractor shall submit all reports as instructed by the Employer in connection with progress meetings and the day-to-day management of the Works.

A.17	Progress Photographs

A.17.1	The Contractor shall supply once a month, at the time of submitting Interim Certificates, twelve photographs from 36 exposures showing the progress of the Works.  The Employer shall direct the times and position from which the photographs are to be taken.

A.18	Setting Out

A.18.1	The Contractor shall be responsible for accurately setting out the Works to the specified positions, dimension, levels and Building Lines and also checking the site surveys for dimensional and level accuracy and reporting any discrepancies before building work commences.

A.18.2	The Contractor shall provide the Employer with all facilities, equipment and labour to enable him/her to check the setting out and levels of the Works at all times. The checking of any setting out point, line or level by the Employer shall not in any way relieve the Contractor of their responsibility

A.18.3	All setting out points, benchmarks, site rails, pegs and other survey points shall be clearly marked and protected from damage or disturbance during the execution of the Works

A.19	Billboards

A.19.1	The Contractor shall provide and maintain two billboards for the Site each consisting of a plastic board panel of size not more than 2.4m x1.2m (2.88m2) supported 2.5m above the ground with steel angle framing or similar material and fixed in concrete foundations.

A.19.2	Each board shall have the following written in both Dhivehi and English (letter height not to exceed 100mm) by a skilled sign writer:

- The name of Project

- The name, logo and address of Employer

- The name, logo and address of Contractor

- Complaint handling contact information (email and/ or phone number)

- Bank’s name and logo (in accordance with Branding Tool Kit – Visual Identity guidelines of the Asian Development Bank and, as the case may be, other financiers’ requirements)

A.19.3	A scaled layout shall be prepared and submitted for the Employer's approval before fabrication.

A.19.4	No advertising material other than the above will be permitted.

A.19.5	The location and layout of Sub‐Contractors or Manufacturer's billboards, if allowed, must be submitted for the Employer's approval.

A.20	Loading in Excess of Design Load

A.20.1	No loading in excess of the design loading shall be placed on any portion of the structure without the written permission of the Employer.

A.20.2	If such permission is granted, all beams or other members of the structure which are subjected to loading other than the designed loading shall be strengthened and supported to the satisfaction of the Employer, and the Contractor shall be responsible for any resulting additional costs

A.20.3	The Contractor shall be responsible for making good to the satisfaction of the Employer any damage to the permanent structure that may be caused by such excess loading.

A.21	Building Permit

A.21.1	The Contractor shall allow for obtaining the building permit and for paying all fees in connection therewith.

A.22	Permanent Drainage, Electricity and Water connection

A.22.1	The Contractor shall allow for arranging and obtaining the permanent drainage, water and electricity connections to the proposed development and shall be responsible for making all payments in connection therewith.

A.23	Handing Over

A.23.1	Prior to handing over the proposed development, the Contractor shall gain the approvals and respective Completion Certificates from all the local government authorities and the like that the work has been completed in accordance with their requirements.

A.23.2 Any payment in connection therewith shall be paid by the Contractor.

- SITE WORKS

B.1	Demolition

B.1.1	Demolition includes the complete demolition including grubbing up of foundations and the proper termination of all services as required by the Drawings including the removal and disposal of all demolished materials. The demolition work shall be executed in a systematic manner.

B.1.2	Demolition operations and the removal of debris shall be carried out to ensure minimum interference with roads, streets, footpaths and other adjacent occupied or used facilities.

B.1.3	Damage caused to adjacent facilities by demolition operations shall be repaired by the Contractor at their own expense. The Contractor shall arrange and pay for the disconnecting, removal and capping of utility services, notify the affected utility agency in advance and obtain written approval before commencing work.

B.1.4	Before commencement of work, submit a method statement to the Employer as to the proposed method and sequence of demolition of the building and a safety plan which shall cover the risk assessment and safety measures for such method statement. The Employer reserves the right to prohibit any method of execution of the Works which s/he regards as unsafe.

B.1.5	Drawing information, particularly for unconventional layouts and special structures, will be made available to the Contractor if possible. The Contractor shall state in the method statement if it is based on such drawings. In the absence of drawings, the Employer may require a detailed structural survey to be carried out and endorsed by a Registered Structural Engineer (or equivalent) to define the existing structure and the appropriate method and sequence of demolition.

B.1.6	No work on site shall be allowed to commence until the proposed method statement has been accepted and all precautionary measures, hoardings, covered walkways, and other requirements are in place.

B.2	Site Clearance

B.2.1	The Site shall be cleared of all vegetation, rock, boulders, etc. and surface soil shall be removed as directed by the Employer. The trees which are to be retained shall be protected from damage.

B.2.2	Spreading, levelling and consolidating on site where required, shall be made with suitable surplus excavated material obtained from the Site. Other soils used for filling shall be approved by the Employer.

B.2.3	The Contractor shall dispose all unsuitable and surplus excavated material.

B.2.4	The Contractor shall tidy up and leave the Site in a clean and sanitary condition at all times during the execution of the Works.

B.3	Excavation

B.3.1	Excavation shall be performed to the required depth as shown in the Drawings.

B.3.2	A survey of the existing site shall be made and the results of same submitted to the Employer before commencement of the work.

B.3.3	Excavation area shall be protected from any water flowing in. Sides of excavations shall be shored or inclined to retain excavation unless otherwise specified.

B.3.4	Excavation near adjoining structures shall be executed with care so as not to damage those structures.

B.3.5	The Contractor shall take all necessary precautions during the excavation for the Works particularly those excavation which are adjoining existing buildings and shall protect such buildings from the damage or collapse by means of temporary or permanent shoring, strutting, sheet piling or underpinning or excavation in short lengths and/or other methods as s/he deems fit and shall properly support all foundations, trenches, walls, floors, etc. affecting the safety of the adjoining existing buildings.

B.3.6	The Contractor shall alter, adopt and maintain all such works described above for the whole period of the Contract and shall finally clear away and make good all damage done.

B.3.7	The construction and efficiency of the shoring, underpinning, strutting and the like for the purpose for which it is erected shall be the responsibility of the Contractor, should any subsidence or any other damage occur due to the inefficiency of the shoring, underpinning, strutting and the like or any other support provided, the damage shall be made good by the Contractor at their own expense and responsibility.

B.3.8	The shoring, strutting, piling and the like, shall be executed in such a manner as to cause as little inconvenience as possible to adjoining owners or the public and the Contractor shall be responsible for negotiating with the adjoining owners the means to safeguard their property and for the use of any portion of their land for the purpose of executing the excavations and no claims submitted on this ground will be entertained.

B.3.9	The Contractor shall be held solely responsible for the safety of the adjoining existing buildings, the sufficiency of all temporary or permanent shoring, underpinning, piling, and the like.

B.3.10	The Contractor shall keep the Employer informed as to manner in which they intend to proceed with the execution of the excavations and obtain approval. Such approval if given shall not absolve the Contractor of their responsibility.

B.3.11	Excavation shall extend a sufficient distance from walls, footings, etc. to allow space for placing and removing shoring and formwork, for performing all work in the excavations and for the inspection of same.

B.3.12	Excavated material shall be deposited within specified areas as directed unless otherwise specified.

B.3.13	The Contractor is deemed to have inspected the site and to leave ascertained as to the nature of the soil, etc. and also the areas where to collect and stack the materials for which necessary site clearance shall have to be made at their own cost.

B.3.14	Stacking or excavated materials shall be done at places approved by the Employer and shall have recorded the original ground levels of such places jointly with the Contractor before commencement of stacking operation.

B.3.15	Extra excavation and allied lead/lift required specifically for providing working space to workmen or shuttering to walls of basement etc. shall be measured for payment, no extra claim being allowed for such work incidental to development and executions of allied jobs. Only authorized excavation approved by the Employer shall be paid for.

B.3.16	Sufficient clear working space shall be left all around excavated area. The disposal of waste/unserviceable materials may be in filling and/or in embankment according to nature of place of disposal. The appropriate specifications for filling and/or embankment shall apply.

B.3.17	All foundation trenches shall be excavated to the full widths and depths shown on the drawings or to such greater or smaller depths as may be found necessary in the opinion of the Employer and so instructed by his/her representative.

B.3.18	Should any excavation be taken down below the specified levels, the Contractor shall fill in such excavation at their own cost with cement concrete specified for foundations, well rammed in position until it is brought up to the level.

B.3.19	The Contractor shall notify the Employer when the excavation is completed and no concrete or masonry shall be laid until the Employer has inspected of the soil for each individual footing.

B.3.20	All foundation pits shall be refilled to the original surface of the ground with approved materials, which shall be well consolidated as instructed by the Employer.

B.3.21	The Contractor shall erect temporary barricades around the excavations and if necessary make provisions of red lamps.

B.3.22	The Contractor shall remove/maintain/restore all service lines like telephone, water supply, electricity etc. without any extra charges.

B.4	De-watering

B.4.1	Where the excavation level is below the natural water table and it is necessary to pump continuously from the excavation or to install a specialist type of dewatering equipment around the perimeter of the site or excavation, the Contractor will be responsible for ensuring the safety and stability of all adjoining structures and services or utilities above or below ground level.

B.4.2	It will also be the responsibility of the Contractor that the equipment installed shall ensure that the excavation and subsequent construction is carried out in dry conditions.

B.4.3	Continuous or permanent de‐watering of the excavation or Site may not be undertaken without the written approval of the Employer and the methods to be employed shall also comply with Codes of Practice and Local Authority requirements.

B.4.4	The water pumped from the excavations or well points shall be pumped to disposal points or sumps approved by the Employer and the Local Ward Office and if so required be passed through settling tanks before disposal.

B.4.5	Unless prior approval has been obtained no water must be disposed of in the Municipality's sewer systems.

B.5	Backfill

B.5.1	All earth used for filling shall unless otherwise stated, be selected hard dry material from the excavation. The maximum dry density of the fill material shall be not less than 1600 kg/m3.

B.5.2	The backfill of excavations shall be placed in horizontal layers not exceeding 300mm in thickness. Each layer shall be compacted by hand or other mechanical means to the required density before the next layer is added.

B.5.3	Care shall be taken when filling or back‐filling to avoid any wedging action or eccentric action upon or against the structure of the work.

B.5.4		Before placing the fill, the surface of the sub‐grade shall be compacted at optimum water content to the same percentage of maximum dry density required of subsequent lay.

B.5.5	The Employer will inspect all compacting devices that the Contractor proposes and shall have the right to reject any device which is unsuitable for the job.

B.5.6	Heavy equipment for spreading and compacting fill and backfill shall not be operated closer to walls than a distance to the difference in height between the top of the footings and the layer being compacted.

B.5.7	When back‐filling behind retaining walls, basement walls and the like the said structures shall be kept propped during the complete operation. The hydraulic compaction of fill shall not be permitted and the back filling shall be carried out in layers not exceeding 150mm thick.

B.5.8	Each layer shall be compacted to 90% of the modified compaction. No back filling shall be carried out until the wall concrete has achieved its full works cube strength and care shall be exercised so as not to damage the external tanking membrane and its protection.

- CONCRETE WORKS

C.1	General

C.1.1	Materials used in the Works shall be new, of the qualities and kinds specified herein and equal to approved samples. Delivery shall be made sufficiently in advance to enable further samples to be taken and tested if required. No materials shall be used until approved and materials not approved shall be immediately removed from the Works.

C.1.2	Materials shall be transported, handled and stored on the site or elsewhere in such a manner to prevent damage, deterioration or contamination.

C.2	Cement

C.2.1	Cement shall be Ordinary Portland cement of an approved brand.

C.2.2	Cement shall conform to BS 12.

C.2.3	Cement shall be of recent manufacturer and used within 6 months of manufactured date.

C.2.4	The Contractor shall with each fresh consignment of cement delivered to the site furnish the Employer with a copy of the Manufacturer's statement of compliance with the above Standard Specifications together with the date of manufacture, certified by an independent agency in the country of origin and its date of delivery to Site.

C.2.5	Check tests will be required by the Employer. These tests shall be carried out at the Contractor's expense.

C.2.6	Any cement failing to meet the required standards will be rejected and replaced at the Contractor's expense.

C.2.7	Any cement not conforming to BS 12 shall not be used unless otherwise approved by the Employer.

C.2.8	Concrete compressive strength grade to be minimum C30/35

C.3	Aggregate

C.3.1	Fine aggregate shall be river sand conforming to BS 882.

C.3.2	Coarse aggregate shall be crushed stone excluding limestone or derivatives of limestone conforming to BS 812.

C.3.3	Aggregate shall not contain injurious amounts of rubbish, dirt, organic impurities and other foreign matters.

C.3.4	Strength of aggregate shall be more than that of hardened concrete paste.

C.3.5	Shape of coarse aggregate shall not be flat or slender.

C.3.6	Aggregate to be used in concrete shall possess the qualities indicated in the following tables.

Quality of Aggregates

| Aggregate type | Open dry specific gravity | Percentage of water absorption (%) | Percentage of solid volume for the evaluation of particle shape (%) | Clay lump (%) | Loss in washin g test (%) | Organic impurity (%) | Water soluble chloride (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Coarse aggregate | <= 2.5 | <= 3.0 | => 55 | <= 0.25 | <= 1.5 | 0 | <= 0.25 |
| Fine aggregate | >=2.5 | <= 3.5 | ‐ | <= 1.0 | <= 3.0 | 0 | <= 0.01 |

* Colour of test solution not to be darker than standard solution

Grading requirements for aggregates

Percentage passing each sieve by weight (%)

| Agg. | Max. size (mm) | Nominal sieve size (mm) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 40 | 30 | 25 | 20 | 15 | 10 | 5 | 2.5 | 1.2 | 0.6 | 0.3 | 0.1 5 |
| Coarse | 25 | 100 | 100 | 90 ‐ 100 | 60 ‐ 90 |  | 20 ‐ 50 | 0 ‐ 10 | 0 ‐ 5 |  |  |  |  |
|  | 20 |  |  | 100 | 90 ‐ 100 |  | 20 ‐ 55 | 0 ‐ 10 | 0 ‐ 50 |  |  |  |  |
| Fine |  |  |  |  |  |  | 100 | 90 ‐ 100 | 80 ‐ 100 | 50 ‐ 90 | 25 ‐ 65 | 10 ‐ 35 | 2 ‐ 10 |

C.3.7	Manufactured sand and blast furnace slag to be use in concrete shall not be used unless otherwise specified or approved by the Employer.

C.3.8	In case of using fine aggregate of 0.01% or more water soluble chloride content, the necessary measures for corrosion inhibiting of reinforcement shall be instructed by the Employer.

C.3.9	The maximum size of coarse aggregate shall be 25 mm.

C.3.10	Sources of aggregate shall be to the approval of the Employer and samples of aggregate from the proposed source shall be submitted to the Employer at least 28 days before its intended use.

C.4	Water

C.4.1	Water shall not contain injurious amount of impurities that may adversely affect concrete and reinforcement.

C.4.2	Ground water shall not be used for concrete works.

C.4.3	Water shall be obtained from a public supply where possible and shall be taken from any other sources only if approved by the Employer.

C.4.4	Only water of approved quality shall be used for washing out formwork, curing concrete and similar surfaces.

C.5	Handling and Storage of Material

C.5.1	Cement

C.5.1.1		Cement shall be stored in a manner to prevent weathering.

C.5.1.2		Bagged cement shall be piled no more than 10 bags so as to permit easy inspection

C.5.2	Cement caked even to the slightest extent shall not be used. Such cement and rejected cement shall be immediately separated from other bags of cement so that they shall not be mistaken for others.

C.5.3	Aggregate

C.5.3.1		Aggregate shall be stored in a manner effectively separating coarse and fine aggregate according to type and shall be prevented from inclusion of dirt, rubbish and other undesirable foreign matters.

C.5.3.2		Coarse aggregate shall be unloaded and piled in a manner not to cause segregation of small and large particles. Aggregate to be stored in piles shall be in mounds of moderate height and at a location where good drainage is provided.

C.6	Mix Proportion and Strength

C.6.1	Mix ratio for reinforced concrete shall be in the proportion 1:2:3 (cement: fine aggregate: coarse aggregate) by dry volume or shall be adjusted based on trial mix to achieve Grade C30.

C.6.2	Mix ratio for lean concrete shall be in the proportion 1:2:5 (cement: fine aggregate: coarse aggregate) by dry volume.

C.6.3	Water‐cement ratio for concrete shall be 0.45%.

C.6.4	The specified design strength of reinforced concrete shall be 30 N/mm².

C.6.5	The required slump of concrete shall be 100 mm.

C.6.6	Design mix proportion shall be to obtain required workability, consistency and durability.

C.7	Production of Concrete

C.7.1	Field‐mixed Concrete Plant

C.7.1.1		The Contractor shall select the necessary facilities for storage, batching, mixing and transporting of each of the materials and submit them for approval of the Employer prior to start work.

C.7.2	Measuring

C.7.2.1		All materials shall be measured by volume for each batch and water may be measured volumetrically.

C.7.2.2		Cement shall be measured by number of bags unless automatic cement weight measure is in use.

C.7.3	Mixing Control

C.7.3.1		Concrete mixture shall be constantly controlled to obtain required workability and mixed strength. Mixing time for each batch shall be not more than 3 minutes.

C.7.4	Quality Control

C.7.4.1		The Contractor shall conduct tests for quality control toward insuring that concrete of the required quality is constantly produced.

C.7.4.2		The Contractor shall have all quality control tests report ready for submission as required by the Employer.

C.7.5	Quality Inspection of Concrete at the Point of Placement

C.7.5.1		The Contractor shall conduct tests on concrete at the point of placement. When test results meet the tolerances given below, the concrete shall be qualified to have passed the tests.

(a)	The tolerance between actual slump and required slump of the concrete shall be 2.0 mm

C.7.5.2		For the estimation of compressive strength of concrete in compressive strength tests, when the average value of compressive strength of concrete obtained in a test is not less than the specified design strength, it shall be qualified to have passed the test. In case of failure to meet the above requirements, the Contractor shall take necessary measures such as to perform appropriate test as instructed by the Employer.

C.8	Transporting and Placing

C.8.1	General

C.8.1.1		The Contractor shall establish manner and schedule for transporting and placing of concrete and obtain approval of the Employer.

C.8.1.2		Concrete shall be transported in a manner to minimize segregation, spill, age and other changes in quality thereof.

C.8.1.3		Concrete shall be placed and consolidated in a manner to insure uniformity and optimum density.

C.8.1.4		In case of rain or other conditions that may affect the quality of concrete during concreting, the Contractor shall take necessary measures as instructed by the Employer.

C.8.2	Time Limit

C.8.2.1		The time limit from start of mixing to completion of placing of a batch as rule, shall be 30 minutes.

C.8.3	Preparation prior to Placing.

C.8.3.1		The place where concrete is to be deposited shall be cleaned and sheathing shall be sprinkled with water. Subsequently, water accumulated in the form shall be removed.

C.8.4	Construction Joint

C.8.4.1		Joint surfaces shall be cleaned, made free of laitance and other foreign matters, and wetted prior to concreting. Joint surface shall be roughened if directed by the Employer.

C.8.4.2		The locations of shapes of construction joints shall be consulted and approved by the Employer.

C.8.5	Concrete Placing

C.8.5.1		Concrete placing shall be proceeded to keep the surface of placed concrete

as horizontal as possible.

C.8.5.2		Concrete shall be continuously poured to compact around reinforcing bars and corners of formwork.

C.8.5.3		The maximum time interval between placements of continuous concreting shall not exceed 0.5 hours. However, when special measures are taken this time limit may be changed according to instruction or approval of the Employer.

C.8.6 	Consolidation

C.8.6.1		Vibrating of concrete and tapping of formwork shall be performed to wall, column and other places difficult for concrete to proceed. Proper number of workers for placing and compacting concrete shall be arranged.

C.8.6.2		Vibrator shall be operated for concrete called for water tightness, difficult portion for concrete to proceed and other cases directed by the Employer. However, vibrator shall not be touched reinforcing bars and shall not be operated more than 30 seconds at same spot.

C.8.6.3		Concrete shall be placed 300 ‐ 600 mm thickness at once in case vibrator is performing. In case flexible‐insert‐vibrator is called for, concrete shall not be placed thicker than the length of the insert or vibrator at one pouring.

C.8.7	Placing Speed

C.8.7.1		Concrete shall be placed at the speed suited for the workability of the concrete and condition of the place of placement, which insures proper consolidation of concrete.

C.9	Concrete Curing

C.9.1	Curing Method

C.9.1.1		After concrete has been placed, the concrete surface shall be kept moist by spraying with water or by other appropriate methods, and shall be protected from direct sunlight and rapid drying. The top surface of slabs shall be kept flooded with water at all times after concreting for the duration of curing period. This curing period shall be for not less than 14 days.

C.9.1.2		As a rule, no foot traffic or loads shall be permitted on concrete for at least 24 hours after placement.

C.10	Test

C.10.1	General

C.10.1.1	The contractor shall be required to conduct all tests according to BS method and procedure.

C.10.1.2	Tests, as a rule, shall be conducted at the locations directed or at the testing institutions approved by the Employer.

C.10.1.3	The Consultant shall conduct tests, as a rule.

C.10.1.4	In case of failure in test, measures shall be taken as instructed by the Employer.

C.10.1.5	The Contractor shall keep test records during the work and for 2 years after completion of the contracted work.

C.10.2	Material

C.10.2.1	Cement Test:

(1)	Setting test.

(2)	Soundness test.

(3)	Compressive strength test.

Note: Item (1) shall be conducted once in every manufacturer.

Item (2) & (3) shall be conducted once in every 2,000 bags.

C.10.2.2	Aggregate test:

(1) Grading and fineness modules.

C.11	Concrete

C.11.1	Fresh concrete

Slump, air content, shall be conducted daily, and more often at request of the Employer.

C.11.2	Compressive strength test of concrete

Test for estimation on strength of concrete in structure:

C.11.2.1	In order to assume estimated strength of concrete in structure, compressive strength test shall be conducted for prepared test pieces on the 7th day and 28th day and those test pieces shall be made for sampling at placing of concreting.

C.11.2.2	Strength test shall be conducted for each of the following conditions: each days pour, each class of concrete, each change of supplies or source and each 100 cubic meters of concrete or fraction thereof. The number of test pieces to be used in a test shall be not less than 3 for each test on the 7th day and the 28th day unless otherwise instructed by the Employer.

C.11.2.3	Test pieces shall be made in accordance with British Standards, and sampling shall be taken as near as possible at the point of placement.

C.11.2.4	Test pieces shall be stored without being disturbed and shall be covered during the first 24 hours, and carefully transported specimens to the testing laboratory. Test pieces shall be cured in water after demoulding. The temperature of test pieces shall be kept as close as possible to the temperature of the concrete in structure until the time of testing.

C.11.2.5	The test results shall be expressed in the average value by calculating the average compressive strength of all test pieces. The average value must be equal to or greater than the specified strength.

C.12	Defective Concrete and Finishes

C.12.1	Honeycombed surfaces shall be made good or on the instruction of the Employer be cut out by the Contractor and make good at their own expense.

C.12.2	Concealed concrete faces shall be left as from the formwork except honeycombed surfaces shall be made good. Faces of concrete to be rendered shall be roughened by approved means to form a key. Faces of concrete that are to have finished other than those specified shall be prepared in an approved manner as instructed by the Employer

- CONCRETE FORMWORK

D.1	Structure and Material

D.1.1	Structure

D.1.1.1		Formwork shall be performed to obtain accurate concrete in accordance with the designated drawings.

D.1.1.2		Formwork shall be firmed and secured to bear the force of concreting and tightened to avoid cement paste seeping.

D.1.2	Materials

D.1.2.1		Sheathing for formwork shall be waterproof plywood of not less than 12 mm thick. Joint of sheathing shall be butt joint and firmly assembled. In case of using wood board for sheathing, boards shall be 15 mm thick and applied planer. Joint shall be tongued and grooved unless otherwise approved by the Employer.

D.1.2.2		Form liners shall be sound and suitable materials to accurately and safely cast the in situ concrete structure as shown on the Drawings.

D.1.2.3		Timber form boards for sheathing where used for fair‐faced concrete shall be of such new materials as not to cause any defects to the surface of the concrete. Special care shall be taken in fabrication, storage and protection of these boards.

D.1.3	Other Material

D.1.3.1		Fastening hardware to be used shall be those with allowable tensile strength guaranteed by manufacturer through strength tests.

D.1.3.2		Form oil shall not have injurious effects on quality of concrete nor to bonding of surface finishing materials and shall be subject to approval of the Employer.

D.2	Performance

D.2.1	Design of formwork

D.2.1.1		Formwork shall be designed to withstand construction leads during concreting, lateral pressure of fresh concrete, shock and vibrators due to concrete placing.

D.2.1.2		Formwork shall be free of injurious leakage of water, easy to remove, and shall not damage concrete at removal.

D.2.1.3		Supports shall be provided with adequate horizontal and diagonal bracing and/or stays to prevent collapsing, heaving and twisting of formwork due to horizontal loads working during concrete placing.

D.2.2	Tolerance

The dimensional tolerances in location and cross section of concrete member used for designing and construction of formwork shall conform to the following table.

### Standard Values of Dimensional tolerances

| Item | Tolerance (mm) |
| --- | --- |
| Tolerance in distance from datum line of each floor to respective members | + 10 |
| Tolerance in cross section of columns, beams and walls | ‐ 5, + 10 |
| Tolerance in thickness of floor and roof slabs | 0, +10 |

D.2.3	Fabrication and Erection

D.2.3.1		Erection of formwork, and transportation and storage of materials thereof shall be started only after previously placed concrete has reached an age which acceptance of these loads will not have any adverse effect on the concrete.

D.2.3.2		Sheathing shall be fabricated and installed accurately to match the locations, shapes and dimensions of members called for in the Drawings.

D.2.3.3		Sheathing shall be installed tightly so as not to permit cement paste or mortar to escape from joints.

D.2.3.4		Pipes, boxes and other embedded hardware shall be properly secured to sheathing or others so that they will not move during concrete placing.

D.2.3.5		Supports shall be erected plumb. Supports at any two vertically consecutive floors shall be erected as near as possible to identical locations on a common plane.

D.2.3.6		Shoring shall be erected paying special attention to safety.

D.2.3.7		If sheathing is reused, the surface in contact with the concrete shall be thoroughly cleaned off and sufficiently repaired before reuse. In case of using for fair‐faced concrete, the same sheathings shall be used twice after approval of the Employer.

D.2.4	Inspection

D.2.4.1		Formwork shall be inspected by the Employer prior to placing of concrete.

D.2.5	Striking of forms

D.2.5.1		The minimum period for keeping the forms in position and for watering after laying the concrete shall be as stated below, except otherwise specified in drawings. Forms shall be removed in such a manner as to ensure the complete safety of the structure, so that there is no shock or vibration as would damage the reinforced concrete.

D.2.5.2		The responsibility for the safety of the concrete shall rest entirely with the Contractor and the Contractor shall be held liable for any damage done and shall have to make good the same at their own expenses.

D.2.5.3		The Contractor shall inform the Employer when they intends to remove shuttering and shall obtain consent, but the consent of the Employer shall not relieve the Contractor of his responsibility.

D.2.5.4		The minimum time for formwork to remain in place shall be as per the following table.

| Vertical sides of beams, slabs and columns | 24 hours |
| --- | --- |
| Soffits of slab | 10 days |
| Soffits of beams | 21 days |
| Cantilevers | 28 days |

D.2.6	Relocation of Support

D.2.6.1		Supports under concrete shall be not relocated.

D.2.7	Removal of formwork

D.2.7.1		Formwork shall be removed gently, after its removal has been approved by the Employer.

D.2.7.2		Inspection by the Employer shall be conducted immediately after the removal of sheathing. Any defects shall be immediately remedied according to instruction of the Employer.

D.2.7.3		After shoring has been removed, members shall be carefully observed for cracking and deflection, when found, they shall be reported immediately to the Employer.

- STEEL REINFORCEMENT

E.1	Material

E.1.1	Reinforcing steel shall be of the dimensions given in the Drawings.

E.1.2	Reinforcing bars shall comply with the requirement of B.S.4449. and welded wire fabric, square bar fabric and expanded metal shall comply with appropriate part of B.S.4483.

E.1.3	Dia 6mm reinforcing steel shall be round mild steel bars, and 12mm, 16mm, 20mm and 25mm shall be deformed high strength bars.

E.1.4	Any other non‐specified reinforcing steel shall be used only with the approval of the Employer.

E.1.5	All reinforcing steel and binding wire shall be stored under cover and shall be at least 250mm above the ground.

E.2	Cleaning

E.2.1	Reinforcing bars shall be cleaned before use so that it is free from rust, oil, dirt or other coatings that reduce bond.

E.3	Bending and Laps

E.3.1	The reinforcement shall be bent cold in an approved bar bending machine.

E.3.2	Preferably bars of full length shall be used. Lapping of bars where necessary shall conform to BS1487 ‘Bending Dimensions of Bars of Concrete reinforcement.’

E.4	Reinforcement Cover

E.4.1	Concrete cover for reinforcement shall be as follows:

FOR ANY STEEL IN UNDER GROUND CONRETE		55	MM

CLEAR COVER IN SLABS						30	MM

CLEAR COVER IN BEAMS SOFFIT				35	MM

CEAR COVER IN SIDES OF BEAMS				35	MM

CLEAR COVER IN COLUMNS					40	MM

E.5	Placing

E.5.1	Reinforcement intended for contact when passing each other shall be securely tied together with binding wire.

E.5.2	Binders and stirrups shall tightly embrace the longitudinal reinforcement to which they shall be security bound or spot welded.

E.5.3	Binding wire shall be turned in from the formwork and shall not project beyond reinforcing bars.

E.5.4	All reinforcement shall be inspected by the Employer and approved before concrete is placed in the forms.

- WATER PROOFING

F.1	Description of work

F.1.1	Extent of water proofing work is shown on drawings.

F.1.2	Install slurry type waterproofing to top surfaces of balcony slabs and external surfaces of underground concrete work.

F.1.3	Install crystalline type water proofing to underground water tanks and roof slabs in strict accordance with the approved manufacture’s printed instructions.

F.2	Materials

F.2.1	Crystalline Type: Material used shall be a cementitious coating containing catalytic chemicals which migrate in to the concrete using moisture present in the concrete as the migrating medium, and which cause the moisture and the un‐hydrated cement in the concrete to react causing the growth of non‐soluble crystals of dendritic fibers in the void and capillary tracks of the concrete that allow passage of water, there by rendering the concrete itself water proof.

F.2.2	Acceptable products: Xypex concentrate, modified, ultra-plug and quick set as manufactured Xypex chemicals (Canada) Limited (or equivalent).

F.3	Storage of materials

F.3.1	General: All materials shall be stored in original undamaged containers with manufacturers’ seals and labels intact. Material shall be stored off the ground in a dry enclosed area.

F.4	Surface preparation

F.4.1	General: All surfaces shall be examined for form tie holes and defects such as honeycombing, rock pockets, cracks, etc. These areas shall be repaired in accordance with these specifications and the manufacturer’s printed instructions.

F.4.2	Concrete finish: concrete surfaces shall have an open capillary system to provide tooth and suctions shall be clean; free from scale, excess form oil, laitance, curing compounds and other foreign matter.

F.4.3	Smooth surfaces or surfaces covered with excess form oil or other contaminants shall be washed lightly, sandblasted, water blasted, or acid‐etched with muriatic acid, as required to provide a clean absorbent surfaces.

F.4.4	Horizontal surfaces shall not be troweled or power‐troweled and shall be left with a rough float finish or a broom finish. Vertical surfaces may have a sacked finish. Comply with manufactures specifications for requirements pertaining to minimum ‘age’ of concrete deck surface scheduled to receive water proofing.

F.4.5	Surface moisture: Water proofing shall be applied to ‘green’ concrete as soon as possible after forms have been stripped or to older pours which have been thoroughly moistened with clean water prior to application. Free water shall be removed prior to application.

Mixing of crystalline water proofing compound: comply with manufactures specification for 2‐coat installation.

F.5	Application

F.5.1	General: Apply all materials under the direction of the manufacturer’s representative.

F.5.2	Constructions joints and surface defects: Comply with waterproofing material manufacturer’s printed directions in the preparation, and treatment of construction joints and surface defects.

F.5.3	Surface application: After all repair, patching and sealing strip placement has been prepared in accordance with manufacturer’s recommendations and approved by manufacturer’s representative, treat concrete surface with first coat slurry mix of crystalline waterproofing compound.

F.5.4	Brushing: Use a short bristle or broom to work the slurry well into the concrete, filing all hairline cracks and surface pores.

F.5.5	Second coat: Apply second coat while first coat is still ‘green’ but after it has reached an initial set, all as recommended by the water proofing material manufacturer.

F.6	Curing

F.6.1	General: Curing shall begin as soon as the waterproofing materials have set up sufficiently so as not to be damaged by a fine spray. Treated surface shall be sprayed three times a day for a three‐day period. Allow material to set 12 days before filling the structure with liquid

F.6.2	Clean up: Remove all surplus materials from the premises and leave all areas broom clean. In the case of temporary protections remove all such items carefully to avoid damage to treated surfaces. Assemble all such materials and remove from premises followed by broom cleaning as noted.

- EMBEDDED DAMPPROOF MEMBRANE

G.1	General

G.1.1	This section deals with laying flexible sheets as dampproof membranes (DPM).

G.2	Products

G.2.1	Polythene sheets for under slab DPM: gauge 500, manufacturer and reference items should be submitted for approval of the employer.

G.2.2	Adhesive tape: A type recommended by the sheet manufacturer.

G.3	Workmanship

G.3.1	Manufacturers Recommendations: to be strictly followed for all products and materials. Apply sheets to clean, dry surfaces with all joints sealed to give a completely waterproof continuous membrane.

G.3.2	Polythene Sheet Under‐Slab DPM: lay a level bed of fine sand, not less than 13mm thick or as specified to receive membrane.

G.3.3	Polythene Sheet DPM: ensure that sheets are clean and dry. Lay single layer loose on base, lap edges 150mm and seal with mastic or adhesive tape.

G.3.4	Pipe Etc.: where pipe etc. pass-through sheeting make junction completely watertight by forming collars fully bonded / sealed to both pipes and sheeting.

G.3.5	Project: finished sheeting adequately and prevent puncturing during following work. Sheet to be covered by permanent overlaying construction as soon as possible.

- STRUCTURAL STEEL

H.1	Scope

H.1.1	This section shall apply to the work involved with structural steels. All incidental items of structural steel shall be stated in the particular specification.

H.2	Materials

H.2.1	Steel

H.2.1.1		Shape of steel shall be precise and straight and free of injurious scratches and rust.

H.2.1.2		All steel sections shall be galvanized sections of strength class 43 A.

H.2.1.3		Dimensions of steel section and tolerance of dimension shall conform to standard dimension of steel regulated in ASTM or BS standard.

H.2.2	Bolt

H.2.2.1		Shape of bolt, nut, washer shall be in accordance with requirement of BS 4190 & BS 3692.

H.2.2.2		Quality of bolt shall be SC 43 A.

H.2.3	Welding Rod

H.2.3.1		Arc welding rod shall conform to materials to be welded, and position.

H.3	Fabrication

H.3.1	Main fabrication shall be done in workshop unless otherwise specified or approved by the Employer.

H.3.2	Full scale drawing of each section shall be drawn prior to fabrication and checked by the Employer.

H.3.3	Section of each material shall be cut perpendicular to axis unless otherwise specified in the drawing.

H.3.4	Saw and angle cutter shall be used for cutting and cut section shall be free of any  noticeable defect.

H.3.5	Deformation caused by cutting shall be corrected.

H.3.6	Normal temperature or hot drawn process shall do bending process. Steel shall be red heat in hot drawn process.

H.3.7	All elements indicated in the drawing shall be chiselled to a finished state and completely attached. Materials shall be checked for bend, distortion, warp, etc. before fabrication.

H.4	Bolt

H.4.1	Bolt Hole

H.4.1.1		Spacing of bolt holes shall be as directed in the following table.

| Diameter of Bolt | Standard Pitch | Minimum Pitch | End Distance | Edge Distance |
| --- | --- | --- | --- | --- |
| 12 | 50 | 30 | 30 | 25 |
| 16 | 50 | 40 | 40 | 30 |

H.4.1.2		Minimum pitch and end distance for lightweight steel shape shall be more than 3 times and 2.5 times a Bolt diameter respectively.

H.4.1.3		Diameter of hole shall not be over 0.5 mm larger than bolt diameter. However, for anchor bolt 5mm clearance shall be allowed between bolt diameter and diameter of hole unless otherwise specified.

H.4.1.4		Bolthole shall either be drilled open or reamed after subpunching. Punching can only be permitted for a material thickness less than 13 mm.

H.4.1.5		Rolled edge around a hole shall be removed.

H.4.1.6		Position of a bolthole shall be precise so that the center of all holes aligns.

H.4.2	Protection against loosening of Nuts

H.4.2.1		Nuts shall be protected against loosening by concrete covering, double nuts or other proper means.

H.5	Welding

H.5.1	Welding

H.5.1.1		Welder shall have an authorized qualification in Maldives and be approved by the Employer.

H.5.1.2		Other tests shall be conducted to confirm welder’s skill in accordance with type of work.

H.5.1.3		Tack welding shall be carried out by the welder approved by the Employer.

H.5.2	Welding Machine

H.5.2.1		Arc welding machine shall be alternate or direct current type, which provides sufficient and adequate current.

H.5.3	Preparation

H.5.3.1		Welding shall be done as much downward as possible using a jig such as Rotary frame.

H.5.3.2		Welding rod shall be always kept in a dry area and if necessary, dried by drying equipment.

H.5.3.3		Welding surface shall be free of water, scale or others injurious to welding work. Slag appeared on the created surface in the middle of welding shall be cleaned before starting again.

H.5.4	Fabrication

H.5.4.1		Welding edge shall be smoothed by automatic gas cutting or other proper finishes.

H.5.5	Finishes

H.5.5.1		Surface of welds shall be as smooth as possible and size and length of welds shall not be less than designed dimensions.

H.5.5.2		Reinforcement of weld shall not exceed 0.1s + 1 mm ( s: Designated size) in fillet welds.

H.5.5.3		Welded parts shall be free of undercut, overlap, crack, blow hole, lack of welds, lack of weld settlement, rolled up slag or other defects.

H.5.5.4		Crater at the end of bead shall be carefully heaped up and slag, sputter, etc. shall be completely removed after welds.

H.5.6	Safety

H.5.6.1		Safe scaffoldings shall be provided for the welding work.

H.5.6.2		Welding facilities shall be electrically safe, preventing electric leakage and shock. There shall also be sufficient protection for fire.

H.5.6.3		Electric shock protection devices shall be used and care shall be taken not to get suffocated or intoxicated by gas when welding in small area.

H.5.7	Inspection

H.5.7.1		Welding parts shall be inspected before, during and after welding in accordance with work schedule.

H.6	Erection and Field Painting

H.6.1	Erection

H.6.1.1		Erection procedure shall be prepared by the contractor and be approved by the Employer prior to the erection.

H.6.1.2		Material shall be stored on flat surface in order not to get distortion, twist or other defects. Correction shall be made to those distortions or twists before erection.

H.6.1.3		Horizontal reinforcement and bracing shall be placed and bolts are temporarily tightened as trusses are put up.

H.6.1.4		Connection of materials by bolts, etc. shall be made after distortion on plumb is thoroughly corrected.

H.6.1.5		Temporary bracing or other reinforcement shall be placed to resist wind pressure or other loads erection.

H.6.1.6		When heavy objects are placed on a horizontal element in the course of erection, they shall be reinforced with prior approval of the Employer.

H.6.1.7	Care shall be taken on all facilities so that there is no accident.

H.6.2	Field Painting

H.6.2.1		All steel work delivered to site unprimed shall be cleaned of impurities, scrapped and wire brushed to remove rust and painted with one coat of priming paint applied by brush.

H.6.2.2		Primed steelwork delivered to site shall be cleaned of impurities and repaired where the primer is damaged.

H.6.2.3		Galvanized steelwork to be painted shall be cleaned of impurities. Where rusting has occurred, the rust shall be removed by wire brushing and made good with an approved rust inhibitor. The surfaces shall be coated with a mordant solution, washed with clean water and painted with two coats of priming paint applied by brush.

H.6.2.4		Steelwork, which is to be concealed shall be prepared and primed as above and shall be painted with two priming coats and one finishing coat of paint applied by brush.

H.7	Anchor Bolt

H.7.1	The methods for anchor bolt installation shall be as directed by the Employer.

- MASONRY

I.1	Materials

I.1.1	Material used for masonry and plastering work shall conform to Section ‐ CONCRETE WORKS.

I.1.2	Masonry work shall be done with cement bricks or blocks of approved quality unless specified otherwise.

I.1.3	The blocks shall be free from excessive amounts of salt or other impurities and shall be inspected and approved by the Employer.

I.2	General

I.2.1	Execution Drawing

I.2.1.1		Work shall be complied with this specification unless otherwise stated on particular Specification or Drawings. Any work not specified shall be discussed and directed by the Employer. Execution drawing of block or brick alignment (inclusive of indication for hanging bolt, wood plug and conduit pipe), detail reinforcement, window opening, and other requirement shall be prepared and submitted for the Employer.

I.2.2	Stake‐Board

I.2.2.1		Stake‐board shall be provided at each 5m in length and shall be inspected by the Employer for the accuracy, firmness and secureness. However, suitable ruler, plumb bob and leveler shall be provided for minor performance of cement block and bricks.

I.2.3	Transportation and storing

I.2.3.1		Care shall be taken during transportation of materials to prevent damage. Concrete blocks or bricks with natural finish defects must be rejected.

I.2.3.2		Different sizes of material shall be stored separately and protected from dirt and other impurities.

I.2.4	Curing

I.2.4.1		No shock or load shall be applied until concrete mortar or other fills have fully cured and hardened. Corner, projection and top surface of cement block or brick work shall be protected from rain, dryness, cold, damage and stain by covering.

I.2.4.2		Void between blocks or bricks shall not be intruded by rainwater.

I.3	Blockwork

I.3.1	Material

I.3.1.1		Blocks shall be of standard quality low permeability blocks with no defects and samples shall be submitted for approval of the Employer.

Blocks shall be Solid cement blocks 125 mm thick for external walls and hollow cement blocks 100 mm thick for internal walls. The average compression strength should not be less than 2.8N/mm2 and shall comply with physical requirements of ISO 6073:1981.

I.3.2	Horizontal reinforcement for concrete block wall;

I.3.2.1		Horizontal reinforcement shall be provided at end of wall adjoining to

concrete column. Reinforcing bar shall be anchored into end block and column.

I.3.2.2	Horizontal reinforcing bar for block wall shall be 6 dia. @ 1000 mm.

I.3.3	Placing Blocks & Bricks

I.3.3.1		Cement blocks shall be saturated with water and joint shall be cleaned.

I.3.3.2		Bonding mortar shall be used immediately after mix, and mixed mortar left for more than one hour shall be rejected.

I.3.3.3		Vertical and horizontal joint of blocks shall be filled completely and suitable with mortar. Blocks must not be moved or repositioned once mortared. Joint and surface of block of exposed finished block wall shall be cleaned immediately after joint is filled.

I.3.3.4		In case concrete block wall is attached to structural concrete, block wall shall be placed before concreting structure.

I.3.3.5		Mortar for joint shall be touched with steel trowel before hardened and exposed joint shall be finished with uniform width and planned without roughness or cavity.

I.3.3.6		Height for placing block per day shall be maximum 1.2 m unless otherwise specified.

I.3.3.7		Blocks shall be placed with cavity side under.

I.3.4	Joints

I.3.4.1		The thickness of joints shall be in between 10 mm – 15mm as to provide for proper bond for the plaster.

I.3.5	Lintel

I.3.5.1		Lintel shall be reinforced concrete as approved or directed by the Employer.

I.3.5.2		Main reinforcing bar shall be anchored more than 40D ( 40 x diameter of the bar) at both end.

I.3.5.3		In case lintel is prefabricated, shop drawing shall be submitted for approval of the Employer.

I.3.6	Frame of Opening

I.3.6.1		In case frame is temporarily installed before placing of blocks, frame shall be firmly placed and joiner shall be bonded with mortar as placing each block at side and top of frame.

I.3.6.2		In case frame is installed after placing of blocks, joiner shall be bonded with additional mortar at space or every two blocks or more.

I.3.6.3		Back of frame shall be filled and compacted with mortar by providing shuttering board.

I.3.6.4		Wood plugs and anchor bolt shall be covered with mortar or concrete.

I.3.7	Piping

I.3.7.1		Principally, piping shall not be placed in block wall unless piping block is in use.

I.3.7.2		In case electric conduit pipe is placed in cavity of concrete blocks, care shall be taken not to obstruct reinforcing bar, and cavity shall be completely filled.

I.3.7.3		In case chipping and piping on face of blocks is unavoidable, performance shall conform to instruction of the Employer.

I.3.7.4		Joiner and supporter for exposed piping shall be buried at joint which is backfilled or otherwise approved by the Employer.

- PLASTERING

J.1	General

J.1.1	All masonry walls shall have smooth finished cement plaster on both sides with a surface setting coat of neat cement applied within an hour of the completion of rendering.

J.1.2	Cement rendering to floor shall be same as above.

J.2	Materials and Storage

J.2.1	Plaster materials which are affected by moisture such as plaster and cement shall be stored properly

J.2.2	Materials used for plastering shall conform to those of Section 3 ‐ Concrete Works. Grading of sand, however, shall be as in table below

| Grading of sand | Mortar plastering | Plastering |
| --- | --- | --- |
| 5mm sifting thorough 100% 0.15mm sifting less than 10% | for first coat for finish coat | for first coat and dubbing out |
| 2.5mm sifting through 100% 0.15mm sifting less than 10% | for finish coat | for second coat |

J.2.3	White cement or filler or similar shall conform to the requirements of BS 12: 1996 Specification for Portland cement.

J.2.4	The use of mixtures shall be approved by the Employer’s representative. The amount of admixture shall be such that it effects mortar strength very little.

J.3	Mixing ratio

J.3.1	Mixing volume ratio of mortar shall be as in table below:

| Base | Area of application | First coat cement: sand | Dabbing out cement: sand | Finish coat cement: sand |
| --- | --- | --- | --- | --- |
| Masonry blocks | Floor | ‐ | ‐ | 1:4 |
|  | Interior wall | 1:4 | 1:4 | 1:4 |
|  | Exterior wall | 1:4 | 1:4 | 1:4 |

J.4	Thickness of Coating

Standard thickness of coating (mm)

| Base | Area of application | First coat | Dubbing out | Second coat | Finish coat | Total |
| --- | --- | --- | --- | --- | --- | --- |
| Masonry | Floor | ‐ | ‐ | ‐ | as per dwg | as per dwg |
| block | Interior wall | 8 | ‐ | 8 | - | 16 |
|  | Exterior wall | 12.5 | ‐ | 12.5 | - | 25 |

Thickness of coating shall be standard thickness of coating unless otherwise indicated on the Drawings.

J.5	Finish

J.5.1	Type of finish and work schedule

| Type | Work Schedule | Notes |
| --- | --- | --- |
| 1.Smooth Trowel finish | - Shall be applied flat by metal trowel - Shall be finished by pressing with the trowel. | Before applying second coat, corner and edge shall be screed well. |
| 2. Wooden float finish | Shall be applied by wooden float |  |

J.6	General Preparation

J.6.1	Remove efflorescence, laitance, dirt and other loose material by thoroughly dry brushing.

J.6.2	Remove all traces of paint, grease, dirt and other materials incompatible with coating. Scrub with water containing detergent. Washing off thoroughly before applying the coating unless specified otherwise.

J.6.3	In Situ Concrete Surfaces: Scrub with water containing detergent to ensure complete removal of mould oil, surface retarders and other materials incompatible with coating. Rinse with clean water and allow to dry unless specified otherwise.

J.6.4	Organic Growths: Treat with fungicide to manufacturer’s recommendations and brush off.

J.6.5	Hacking For Key: roughen specified surfaces thoroughly and evenly by removing the entire surface to a depth of 3mm by scabbling, bush hammering or abrasive blasting. Clean surfaces by washing and brushing.

J.6.6	Smooth Concrete Surfaces: where no keying or mix or bonding agent is specified, wet smooth concrete surfaces immediately before plastering.

J.7	External Plastering

J.7.1	Dissimilar Solid Backgrounds for Plastering: where plaster is to be continued without break across joints between dissimilar solid backgrounds which are rigidly bonded together, cover the joints with a 200mm wide mesh strip (back grounds in the same plane) or with the corner mesh (internal angle) fixed at not more than 600mm centers along both edges, unless specified or otherwise.

J.7.2	Dissimilar Solid Backgrounds for Plaster: where plaster is to be continued without break and without change of plane across the face of 300mm span, and rigidly bonded to the background.

J.7.2.1		Cover the face of the column/beam/lintel with building paper extending 25 mm on the adjacent background.

J.7.2.2		Over lay with expanded metal lathing extending 50mm beyond the edges of the paper and securely fixed with masonry nails at not less than 100mm centers along both edges.

Alternatively, an approved paper and mesh lathing may be used.

J.7.3	Dissimilar Solid Backgrounds for Rendering: where rendering is to be continued without break across joints between dissimilar solid backgrounds which are in the same plan and rigidly bounded together, cover joints with a 150mm wide strip of building paper overlaid with 300mm wide metal lathing fixed at not more than 600mm centers along both edges unless specified otherwise.

J.7.4	Service Chases: cover with steel mesh strip fixed at not more than 600mm centers along

both edges.

J.7.5	Conduits bedded in under coat to be covered with 90mm wide jute scrim budded in finishing coat mix, pressed flat and trowelled in. Do not lap ends of scrim.

J.8	Internal Plastering

J.8.1	Accuracy of plaster 15mm thick or more: maximum permissible gap between an 1800mm straight edge and any point on the surface to be 3mm.

J.8.2	Dubbing Out: if necessary to correct inaccuracies, dub out in thickness of not more than 10mm in same mix as first coat. Allow each coat to set before the first is applied. Cross scratch surface of each dubbing out coat immediately after set.

J.8.3	Metal Mesh Lathing: Work undercoat well into interstices to obtain maximum key.

J.8.4	Under Coats: Generally, to be not less than 8mm with thickness greater than 16mm applied as two equal coats. Rule to an even surfaces and cross scratch ‐ end coat to provide a key for the next hand applied coat.

J.8.5	Cement Based Under Coats: dry out thoroughly but not rapidly, to ensure that drying shrinkage is substantially complete before applying next coat.

J.8.6	Dissimilar Backgrounds: where scrim or lathing or beads are not specified, cut through plaster with a fine blade in a neat, straight line at junctions of:

J.8.6.1		Plastered rigid sheet and plastered solid backgrounds.

J.8.6.2		Dissimilar solid backgrounds.

J.8.7	Smooth Finish: trowel or float to product a tight matt, smooth surface with no hollows abrupt change of level or trowel marks. Do not use water brush and avoid excessive trowelling and over polishing.

J.9	External Rendering

J.9.1	Dubbing Out: if necessary to correct inaccuracies, dub out in thicknesses of not more than 10mm in same mix as first coat. Allow each coat to dry before the next is applied. Cross scratch surface of each dubbing out coat immediately after set.

J.9.2	Under Coats for hand applied finishes:

J.9.2.1		Apply first undercoat or dubbing out coat by throwing from a trowel.

J.9.2.2		Coats to be no less than 8mm thick, with thickness greater than 16mm applied as two equal coats. On weak backgrounds first under coat to be not less than 10mm thick.

J.9.2.3		Brush down each under coat to remove dust and loose particles and wet thoroughly before application of next coat.

J.9.2.4		Cross scratch under coat without penetrating the coat, to provide key for following coat(s).

Drying: Keep each coat damp for the first three days by covering it with polythene sheet and/or spraying with water. Thereafter prevent it from drying out too rapidly. Work in shade whenever possible.

Allow each coat to dry out thoroughly to ensure that drying shrinkage is substantially complete before applying next coat.

Playing Floated Finish: Finish with wood or other suitably faced float to give an even texture. Do not draw excessive laitance to surfaces.

J.10	Metal Mesh Lathing/ Reinforcement for Plastered/Coatings.

J.10.1	Lathing to be provided as reinforcement for plastering in columns, walls or specified in drawings products.

J.10.2	Products:

J.10.2.1	Plain Expanded Metal Lathing: To BS EN 13658:2005 with a minimum weight of 1.9 kg/mm2. Manufacturer to approval of the Employer.

J.10.2.2	Wire Ties: Unless other specified, annealed iron, galvanized as per specified in BS EN 10244-2:2023 - TC.

J.10.2.3	Clout Nails: galvanized steel or stainless-steel nails to B.S 1202: Part 1, table 3.

J.10.2.4	Staples: Galvanized steel wire staples to B.S 1494: Part 2.

J.10.3	Workmanship

J.10.3.1	Framing: fix securely and accurately to help ensure that coatings on lathing, when finished, are true to line and level, within specified tolerances and free from cracks, rippling, hollows, ridges and sudden changes of levels.

J.10.3.2	Runners/Bearers spanning between concrete beams/ribs: fix with 3mm wire ties twisted around 38 mm X 10-gauge screws driven well into fixing blocks or plugs in sides of beams/ribs.

J.10.3.3	Wire Ties: twisted ends tightly together, cut off surplus and bend ends of wire away from face of coating.

J.10.3.4	Plain Expanded Metal Lathing:

(a)	Stretch lathing and fix securely in accordance with manufacturers recommendations to give a taut, firm base for plaster/ rendering.

(b)	Fix with the long way of the mesh at right angles to supports and with all strands sloping in the same direction.

(b) Lap side edges not less than 25mm. Lap ends 50mm at supports and 75mm between supports. Laps must not occur within 100mm of angles or bends.

- CARPENTRY AND JOINERY

K.1	Materials

K.1.1	Timber shall be in accordance with the requirements of BS 1186 ‘Quantity of Timber and Workmanship in Joinery’, Part 1, ‘Quality of Timber’.

K.1.2	Timber and timber products shall be subject to the inspection and approval of the Employer.

K.1.3	Timber shall be seasoned to stable moisture content compatible with the finished use, straight and true and free from wind, warp and distortion and in lengths suitable for the members required.

K.1.4	All timber shall be in long lengths and laps, scars or splices shall be over a bearing surface. Where obtainable, finishing timber exposed to view shall be in single lengths.

K.2	Preservation of Timber

K.2.1	All timber shall be treated for insect attack and is to be of the correct moisture content and free from surface moisture content and dirt.

K.2.2	All rafters, purlins, framing scribe pieces, wall plates, and trusses etc. shall be treated for insect attack with approved timber preservative. No extra payment shall be made for such coating and will be considered inclusive in the rate of the respective item in the BOQ.

K.2.3	Treatment shall be carried out after all cutting and shaping is completed.

K.3	Hardware

K.3.1	Hardware shall be standard quality and samples shall be submitted to the Employer for approval.

K.3.2	All hinges shall be stainless steel or brass and shall be approved by the Employer.

K.3.3	The dimensions and quality of hardware shall meet the requirements and shall not be rested, deformed or defective.

K.4	Dimensions and Finish

K.4.1	All dimensions of timber given are finished dimensions.

K.4.2	All elements and others of structural nature, which are exposed, must be machine planed to a smooth finish.

K.4.3	All unexposed timber shall be machine planed to a rough finish.

K.4.4	All joinery work shall be dressed on all four sides and hand dressed where necessary and sanded on all exposed surfaces. All arises in any way accessible shall be sanded and smoothed off.

K.5	Workmanship

K.5.1	All connections whether nailed, screwed, glued, mortised or dove‐tailed shall be accurately made and properly executed to provide sound, satisfactory connections for the class of work required.

K.5.2	Timbers containing defects or distortions shall not be used.

K.5.3	All joinery shall be manufactured by skilled tradesman with accurate tolerances and set out and with tools, jigs, machines and equipment appropriate for the work.

K.5.4	Assembly of the joinery units and joinery frames, etc. shall be by means of glued connections appropriate to the work ‐ mortise and tenon, housing and doweling, etc. where practicable, including the use of glued blocks wherever required. Nailing, screwing shall only be used with prior approval of the Employer, corrugated fasteners shall not be used for affecting connections.

- ALUMINIUM WORKS

L1.1	Aluminium Doors and Windows

L.1.1	All windows and doors are to be constructed by approved specialist suppliers of medium section to the particular requirements noted on the drawings as to weight and profile. All sections shall generally conform to relevant British Standard Specifications.

All frames should be made to fit the actual openings with a 3 mm clearance all around. Discrepancies in overall width or height exceeding 3mm will not be allowed and the frames will be rejected in such cases. Any small discrepancies shall have the gaps suitably backed and filled with gun‐applied water repellant mastic sealant

All sealants used in the assembly of, and in the fixing of cladding and window framing, shall be non‐ setting to allow thermal movement without detriment to those joint sealants used for peripheral caulking and shall be one part silicone sealant and shall conform to BS 4245. All spliced joints between mullions should be sealed with an approved silicone product, compatible with other sealants and packings used.

L.1.4	The auxiliary components in sashes as locks, pivots, sliding gear etc. shall comprise of stainless steel or resisting materials.

L.1.5	The tolerances are to be as follows:

a)	Inside width of frame			3mm Maximum

b)	Inside height of frame			3mm Maximum

c)	Depth of frame			2mm Maximum

d)	Opposite side, Inside distance	2mm Maximum

L.1.6	The performance ‐ associated requirements are

1)	Strength (resistance to wind pressure and other forces applied in use)

2)	Air tightness or ability to cut out drafts

3)	Water tightness against rain or dew

4)	Sound arresting effect to (shut off noise from outside as well as inside)

L.1.7	All surfaces shall have timber textured powder surface layer of minimum 60 Micron thickness.

L.1.8	Glazing shall be done as specified by the Employer. Glass shall be as specified in the drawings. Thickness shall be according to the size of panels as given hereunder.

| Not exceeding 1 sq. ft. | 2mm |
| --- | --- |
| Exceeding 1 sq. ft. but not exceeding 2 sq. ft. | 3mm |
| Exceeding 2 sq. ft. but not exceeding 4 sq. ft | 4mm |
| Exceeding 4 sq. ft. but not exceeding 6 sq. ft | 5mm |
| Exceeding 6 sq. ft. | 6mm |

L.1.9	Prior to import and/or purchase of the Aluminium Doors and Windows, the relevant specification of the manufacturer, along with samples has to be submitted to the Employer for approval. This clause shall not be contravened on any account.

L.1.10	The fitting shall be done with utmost care not to spoil the finishes given by the manufactures, and any cleaning done shall be done with cleaners etc. as specified by the Manufactures.

L.1.11 The Contractor shall provide all items, articles, materials, operations, mentioned, or scheduled on the drawings, including all the labour materials, including fixing devices, equipment and incidentals necessary as required for their completion.

L.1.12	The Contractor shall submit shop drawings and/or samples of each type of door, windows, railings and other items of metal work to the Employer for approval. The shop drawings shall show full size sections of doors and windows etc. thickness of metal, details of construction hardware as well as connection of windows, doors and other metal work to adjacent work.

L.1.13	Aluminium doors and shutters shall be manufactured by an approved manufacturer and shall be of sections, sizes combination and details shown on the drawings. The frame member shall be one piece, corners shall be electrically welded, ground smooth and true and glazing bare shall be threaded or interlocked as approved by the Employer.

L.1.14	Glazing for doors and windows shall be of specified thickness and of approved quality and shall conform to specification of glazing. Fixing for glazing shall be done with aluminium Snap‐On beading as per detail drawing and instructions. Necessary continuous rubber gaskets of approved make shall be provided.

L.1.15	Colour for doors and windows shall be approved by the Employer.

L.2	Aluminium louvers

L.2.1	Samples shall be submitted for approval.

L.2.2	All metal louvers shall be installed according to manufacturer’s instructions.

L.2.3	All units shall be installed plum, well fitted and securely attached to supporting frames.

L.3	Top hung windows, ventilators and side hung doors

L.3.1	All windows and doors should be weather stripped. The weather protection should be achieved by a positive compressive action against the section and should not depend on external contact. At every contact between two profiles two weather stripping sections should be provided to complete weather protection.

L.3.2	The bottom section for hinges must be capable of being adjusted vertically if necessary. The gap between section and the floor should be covered with a pair of special splay‐tube sections.

L.3.3	The shutter sections for both windows as well as doors shall be hollow section type and shall be overall size 57 x 45 mm and the door sections shall be overall size 81 x 45 mm (including flanges).

L.3.4	The shutters of the windows and doors should be assembled with stainless steel pins and nylon washers. Handles shall be anodised aluminium finished to match the aluminium sections and mounted with self-lubricating nylon washers.

L.3.5	A mortice cylinder rim automatic deadlock of high quality with double pin tumbler shall be used.

L.3.6	Windows shall have anodised aluminium handles, colour as framing and a latching mechanism securing the shutter to the frame both at the top and bottom.

L.3.7	Required fittings;

•	Single action door closer concealed in the head bar of the outer frame and mounted on an adjacent pivot at the threshold and deadlock fitted.

•	The left hand leaf of double doors with flush bolts at head and sill with deadlock fitted to the right hand leaf.

•	Escape doors to have panic bolts assembly with vertical elements concealed in the sill and door closer.

L.4	Installation

L.4.1	Aluminium work shall be installed adjusted and glazed by experienced workmen all in accordance with the manufacturer's installation instructions and in full conformity with the approved shop drawings, samples and other submitted data. Under no circumstances shall materials be installed on surfaces that contain condensation, dirt, grease or other foreign encountered materials that would hinder or prevent proper installation and functioning for the use intended.

L.4.2	Aluminium work shall be carefully and accurately assembled with proper and approved provision for contraction and expansion and set in correct locations as per approved detailed shop drawings, all level, square, plumb and aligned with other work. All joints between framing and structural building shall be sealed in order to be watertight and weatherproof and to satisfy all other requirements of the Employer.

L.4.3	Frames shall be designed and manufactured with a maximum 2.5mm tolerance around the opening in the structure. These joints are to be finished by applying an approved sealant into a polystyrene foam backing strip.

L.4.4	All aluminium works are to be fully protected for the duration of the contract from damage by other trades. The Employer shall approve the method of protection.

L.4.5	If for any reason final finishes become scratched, abraded or damaged during transport, delivery, storage or erection, it shall be the Contractor's responsibility to remove or repair those defective areas or components as directed and to the complete satisfaction of the Employer.

L.4.6	Repair work shall be identical to the manufacturer's applied finish with regard to gloss, finish and visual appearance. Field touch up of painted aluminium is permitted only with the written permission of the Employer. Where touch up is not an authorised means of repair the damaged materials must be replaced by new ones.

L.4.7	Upon completion of work all protective coverings from all exposed surfaces shall be removed. All surfaces shall be cleaned using soap or detergents as recommended by the aluminium manufacturers to remove sealants, discolouration and any other foreign material. Defections of any type determined by the Employer shall be repaired at the Contractor's expense.

L.4.8	Extreme care shall be taken when cleaning the exterior portion to protect all other adjacent works.

L.5	Sealing joints

L.5.1	The Contractor shall ensure that joints are dry and remove all loose material, dust and grease.

L.5.2	Joints shall be prepared in accordance with sealant manufacturer's recommendations using recommended solvents and primers where necessary.

L.5.3	Adjoining surfaces which would be impossible to clean if smeared with sealant shall be masked. Backing strips shall be inserted in all joints to be pointed with sealant. When using backing strips, the

L.5.4	Contractor shall not leave gaps and shall not reduce depth of joint for sealant to less than the minimum recommended by the manufacturer.

L.5.5	Cavities shall be filled and jointed with sealant in accordance with the manufacturer's recommendations. Sealant shall be tooled to form a smooth flat bead.

L.5.6	Excess sealant shall be removed from adjoining surfaces using cleaning materials recommended by the sealant manufacturer, and shall be left clean.

L.6	Glass installation

L.6.1	Workmanship shall generally be in accordance with CP 152 and respective British Standards.

L.6.2	The glass is to be delivered to the site with adequate protection to prevent damage and where possible it is to be fixed in position immediately after delivery. When fixed the Contractor is to take all necessary precautions to prevent damage during succeeding building operations and will be entirely responsible for the replacement of any broken or damaged glass at his/her own cost.

L.6.3	The Contractor is to be solely responsible for determining the exact sizes of glass required, including a tolerance of 2mm to each edge and is recommended to check the necessary dimensions on site.

L.6.4	No glazing is to be carried out until rebates have been painted with primer. Glazing beads as applicable are also to be primed before fixing.

L.6.5	All mastic is to be neatly struck off to agree exactly with site lines inside and out.

L.6.6	Rates are to include for all necessary springs, clips, setting blocks, location blocks and distance pieces and for taking off and later re‐fixing loose beads

L.6.7	Glass apertures in timber doors are to be bedded in chamois leather glazing strip, black ribbon velvet or P.V.C. glazing strip to the approval of the Employer.

L.7	Aluminium Cladding (exterior fin system)

L.7.1	Dimensions given on the drawings are finished dimensions.

L.7.2	Specifications from the manufacturer shall be forwarded to Employer for approval.

L.8	WPC and Wooden Doors

L.8.1 Dimensions given on the drawings are finished dimensions.

L.8.2	Specifications from the manufacturer shall be forwarded to Employer for approval.

- ROOFING

M.1	Scope

M.1.1	This Section deals with steel profiled sheeting used as external weatherproof cladding of roofs.

M.2	Roof Cladding

M.2.1	Sheet type: SPANDEK® ZINC HI-TEN® roofing sheets manufactured by John Lysaght, No.18 Benoi Sector, Jurong , Singapore 2262 or equivalent.

M.2.2	Structural support: timber sections as per drawings.

M.2.3	Fastening: No. 12‐14x45mm hexagonal head self-drilling and tapping screw seal.

M.2.4	End laps: 200mm and should be sealed with a recommend sealant for pitches below 7 degrees.

M.2.5	Side laps: as per manufacturer’s recommendations.

M.3	Products

M.3.1	The profiled sheeting shall be in galvanized sheet steel with a factory per finished protective PVC film with colour to approval.

M.4	Workmanship

M.4.1	Accessories: Flashing, trims, filler pieces, spacers, tapes, sealant, etc. where not specified to be the types recommended by the sheet manufacturer.

M.4.2	Fastening: Select types and location of fastenings to meet the following requirements.

M.4.2.1	Wind suction loaded: Calculate in accordance with BS Code of Practice CP 3: Chapter5: Part 2, making due allowance for any internal pressure.

•	Basic wind speed: 45 m/sec.

•	Topography factory S1: 1.0

•	Ground roughness, building size and height Factory (S2): as determined from 	CP3: Chapter 5: Part 2, Table 3.

•	Statistical factor (S3): 1.0

M.4.3.2	Imposed loads other than wind and maintenance load, 1.5 KN/m2 concentrated   on   a   300mm2      which   ever   produces   the   greater   stress. Maintenance point load: 0.9 KN concentrated on any 125mm2.

M.4.3.3	Dead load: allow for self-weight of sheeting.

M.4.3.4	Roof pitch: as indicated on drawings.

M.4.3.5	Distance between not less than 900mm or as indicated on the drawings.

M.5	Fixing

M.5.1	Quality of Work: Handle and store to preserve surface using clean dry gloves. Do not slide sheets over rough surface or each other. Packs of all sheets must be kept dry in transit and stored clear of the ground under cover to prevent water and /or condensation being trapped between adjacent surfaces. If packs become wet, sheets should be separated, wiped with a clean cloth without delay and placed so that air calculation completes the drying process.

M.5.2	Structure: Check that structure is in a suitable state to receive sheets before commencing fixing. Contractor must confirm acceptance to Employer

M.5.3	Structure: Do not fix profiled sheeting until final coats of paints have been applied to outer surfaces of supporting structure.

M.5.4	Isolating Tape: Apply to those surfaces of supports which would otherwise be in contact  with sheeting or accessories after fixing.

M.5.5	Cutting and drilling :

M.5.5.1	Cuts sheets accurately with clean, true lines and no distortion with a power saw with abrasive cutting disc.

M.5.5.2	Cut openings in sheet for outlets, vent pipes , flues etc. to the minimum size necessary . Reinforce edges of openings with structural members.

M.5.5.3	Drill all holes. Position at regular intervals in straight lines. Holes for primary fastenings to be 1.5mm larger than the diameter of fastening unless self-drilling type is used.

M.5.5.4	Remove burrs, drilling swarf, lubricant, dust and any other foreign matter before finally fixing sheets into position.

M.5.6	Direction of Laying: Lay sheets with exposed joints of side lap away from prevailing wind.

M.5.7	End Laps: to be fully supported.

M.5.8	Sealant:

M.5.8.1	Install to manufactures recommendation.

M.5.8.2	Position in straight, unbroken lines parallel to edges of sheets. Placed into corrugations. Do not allow to sag into position.

M.5.8.3	Ensure continuity and effectiveness of seal , especially at corners of sheets.

M.5.8.4	Do not over compress.

M.6	Fittings and Features

M.6.1	Profile Fillers : use where specified and wherever necessary to close off corrugation cavities from the outside and inside of the building. Position on the line of, or above, fastening and ensuring a tight fit and leaving no gaps. Where sealed laps are specified bed profile fillers in sealant on top and bottom surface, but do not obstruct channels for ventilation or condensation drainage.

M.6.2	Flashing Trims: All fittings for flashing / trim shall be as per manufacturers recommendation and lapped at joints as follows:

M.6.2.1	Vertical and sloping flashing / trims : end lap to be the same as for adjacent sheeting.

M.6.2.2	Horizontal flashing / trims: end laps to be 150mm and sealed.

M.6.3	Gutter: Ensure that gutters are fully supported at each joint and at intermediate position not

more than 900mm apart. Fix with spigot ends up the slope and make all the joints fully watertight. Position sheeting to leave a clear width across the gutter of not less than 230mm.

M.6.4	Insulation:

•	75mm thick Rock Wool insulation blanket with aluminium foil backing on both sides laid between purlins at 1000 centres, including wire mesh. Manufacturer and reference ‐ to approval.

- FINISHES

N.1	General

Glazed Ceramic Tile shall comply with British Standard specifications No. BS-EN-ISO-10545-13

and BS-EN-ISO-10545-14 and shall be approved sizes as shown on Drawings and the product of a reputable manufacturers approved by the Employer.

Unglazed Ceramic Tile shall comply with the requirements of British Standard No. BS 6431-20:1984; BS 6431-21:1984; BS 6431-23:1986; and BS 6431-1:1983, and shall be of approved sizes as shown on the drawings and the product of a reputable manufacturer.

N.2	Manufacturers

All tiles, ceramic or homogenous, for the project shall be manufactured as mentioned on finishing schedule / details drawings. Required brands of tile shall be used only for described locations and tile brands shall not change if only approved by Employer.

Following brands of tiles shall be used for described locations as per details drawings. All the tiles shall be submitted to Employer for approval prior to use.

•	Niro

•	Cotto

•	Lanka tile

•	White horse

•	MML

N.3	Ceramic and Vitreous Tile Materials

N.3.1	Ceramic and Vitreous clay Wall Tiles:

N.3.1.1		All tiles for wall installation shall have cushion edge, impervious porcelain and highly glazed surface. Colours shall be as selected by the Employer and shall include trimmers, corner pieces, bullnose and all other special shapes indicated or required. All this shall be free from flaws, cracks and crazing.

N.3.2	Floor Ceramic and Vitreous Tiles

N.3.2.1		Non‐slip ceramic tile shall be used on all floor locations. Floor tiles shall be specially prepared for floor use but shall have all the qualities of ceramic tiles listed above for wall use.

N.4	Mortar Materials

N.4.1	Standard brand of light gray or white Portland cement as specified in drawings/Finishing schedule/ BOQ, conforming to current British Standard specifications shall be used.

N.4.2	Sand: shall be clean, sharp, river sand, conforming to British Standard Specifications and graded fine to coarse within the following limits: 100% passing 8 sieve, 90% to 100% passing 16 sieve, 60% to 90% passing 30 sieve, 25% to 55% passing 50 sieve and 0% to 15% passing 100 sieve.

N.5	Cement Colour

N.5.1 Dry cement colour, chemically inert, non-fading, alkalifast, mineral pigment, as approved shall be used wherever refinished.

N.6	Waterproofing

N.6.1 Floors of toilet areas, corridors and planter boxes shall be treated with an appropriate water proofing coating, approved by the Employer

N.7	Floor tile Installation

N.7.1	As far as possible, tile lay out work should be in such a way that no tile less than half size occurs.

N.7.2	Align joints in wall tile vertically and horizontally except where other patterns are shown or specified, Align joints in floor tiles at right angles to each other straight with walls to conform to the patterns selected.

N.7.3	Verify locations of accessories before installing tiles. Work shall be coordinated with plumbing and other trades before starting of tile work.

N.7.4	Installation of ceramic and vitreous tile shall be in accordance with manufacturer’s instructions.

N.7.5	All ceramic and vitreous clay tile floors shall be in Portland cement setting beds. Concrete surfaces shall be cleaned and surface of concrete shall be wetted prior to placing of setting bed mortar. Tiles shall be immersed in water for minimum of 4 hours before laying.

N.7.6	Setting Bed Mortar Mix: shall consist of one (1) part Portland cement and two (2) parts dry sand, by volume, to which not more than 1/10 part of hydrated lime may be added.

N.7.7	When mixed with water, the mortar mix shall be of such consistency and workability as to produce maximum density. Determine consistency by stroking the mortar surface with a trowel. Whereof correct consistency, the troweled surface readily assumes a smoothed, slickened appearance.

N.7.8	Spread setting bed mortar and screed to provide smooth, dense beds with true planes pitched to drains. The thickness of bed shall be such that the floor tile will finish flush with adjacent finished flooring, but bedding shall have average thickness of 38mm.

N.7.9	After bed has set sufficiently to be worked over, trowel or brush a thin layer, 3mm in thickness, of neat Portland cement or special tile adhesive (approved by Employer) paste over the surface of the back of tile.

N.7.10	Do not prepare larger setting bed than can be covered with tile before the mortar sets.

N.7.11	Press tile firmly into the bed tapping with wood blocks to obtain firm bedding of total tile area and a smooth top surface.

N.7.12	All tiles shall be properly aligned with straight joints in even widths. Joints width shall be determined by spacers on ceramic tiles. Tamping shall be completed within one (1) hour after placing tile. Adjust work out of line within this period.

N.7.13	Tiles shall be fitted closely around pipes running through walls and floors. Pitch floors to drains.

N.8	Wall Tile Installation

N.8.1	Base Plaster 13mm thick applied to masonry wall shall be one‐part Portland cement, three‐ parts of river sand by volume. Where additional thickness build‐up is required to conform to indicated lines, apply as separate coat at no cost to employer.

N.8.2	Setting bed of tiles shall be done with cement slurry or special tile Adhesive (approved by Employer). The thickness of slurry bed shall be 3mm thick minimum for setting tiles and walls.

N.8.3	Installation of tiles shall be in accordance with standards and applicable requirements previously specified for floor tile.

N.8.4	Tiles shall be installed in perfect vertical plumb and as per the pattern and joints if shown on drawings

N.9	Grouting

N.9.1	Grouting shall not commence for at least 24 hours after placing of tiles.

N.9.2	Grout for floor and wall ceramic and vitreous tiles shall be waterproof, neat white Portland cement with dry cement colour added as directed by the Employer. If white grout is selected, cement shall be white.

N.9.3	Grout mixed to a creamy consistency in accordance with manufacturer’s directions shall be used for joint filling. Maximum width of joints shall be 3mm.

N.9.4	Force maximum grout into the joints with trowel. Before grout sets, strike or tool joints to base of cushion and fill all skips and gaps. Do not permit setting bed materials to show through grouted joints.

N.9.5	Cure grout joints by maintaining damp condition for three (3) days by sponging down, or other methods approved by the Employer. Allow floors to set 48 hours before permitting ordinary foot traffic.

N.10	Defects in Tiles and Tile Laying

N.10.1	The surface of all tiled floors shall be perfectly in level and shall be executed by experienced workers in the field of tile laying.

N.10.2	A sample panel of laid tiles of each type shall be approved by the Employer before commencement of tile laying.

N.10.3	Mismatches of colour, chipped or damaged tiles installed by the Contractor shall be rejected and shall have to be replaced by the Contractor’s own cost and risk.

N.10.4	Mismatches of colour in tiles installed by the Contractor shall be rejected and shall have to be replaced by the Contractor’s own cost and risk.

N.11	Guarantees

N.11.1	Manufacturer shall provide his standard guarantees for work under this section. However, such guarantees shall be in addition to not in lieu of all other liabilities which manufacturers and Contractor may have by other provisions of the Contract Documents.

- PLUMBING

O.1	General

O.1.1	The materials used and workmanship shall be of highest quality and grade unless otherwise specified. They shall conform to the latest specifications of British Standards and Codes of Practice for “Water Supply”, “Sanitary”, “Pipe Work”, “Building Drainage” and “Surface Water and Sub‐ Soil Drainage”. All work must adhere to details and work indicated on the Drawing and Bill of Quantities. In case of any discrepancy / ambiguity, the decision of the Employers shall be final, and the contractor will act and perform accordingly.

O.1.2	The work shall be executed strictly in accordance with the rules and regulations set by the relevant local authority of the Maldives.

O.1.3	The Contractor shall be responsible for obtaining the necessary approvals and test certificates from the concerned departments of Maldives.

O.1.4	Plumbing work shall be carried out by competent plumbers and shall produce the copy of the license along with the tenders or approved by the Employer.

O.1.5	Any damage done by the Contractor to any existing work during the course of execution of the work, shall be made good by themselves at their own cost. Failing which it shall be get done by the Employers at Contractor’s risk and cost.

O.1.6	The Contractor shall be responsible to connect the drainage and water supply to the mains and to obtain the necessary approvals and certificates from the relevant authorities of the Maldives.

O.1.7	All connections to mains and meter installation shall be arranged by the Contractor and payment of fees thereof, if any, shall also be made by the Contractor.

O.1.8	The Contractor shall be responsible for the watch and ward of all fittings until the Works is fully completed and handed over to the owner.

O.1.9	The levels, measurements and other information concerning the existing site as shown on the drawings or as described as are supposed to be correct. The Contractor shall, however, verify and no extra claim whatsoever shall be entertained on account of the errors or omissions in such matters or on account of the descriptions turning cut to be different from what was excepted.

O.1.10	The Employer shall instruct the Contractor to purchase and use such materials of particular make or from particular source as may in his/her opinion be necessary for proper and reasonable compliance with the specification and execution of the Works.

O.1.11	After all plumbing fixtures and equipment have been set ready for use, and before the Contractor leaves the job, they shall thoroughly clean all fixtures installed during the contract, removing all plaster, stickers, rust stains and other foreign matter of discolouration on fixtures, leaving every part in acceptable condition and ready for use to the satisfaction of the Employers.

O.2	Drawings and Information Required

O.2.1	The Contractor shall submit shop drawing for the entire installation including installation details for all items required or asked for approval of the Employer.

O.2.2	Approved by the Employer of shop drawing for any material, apparatus, devices and layout, shall not relieve the Contractor from the responsibility of furnishing same of proper dimension, size, quantity and all performance characteristic to efficiently perform the requirements and intent of the Contract Documents. Such approval shall not relieve the Contractor from responsibility for errors of any sort in the shop drawing.

O.2.3	If the shop drawings deviate from the contract Documents the Contractor shall advise the Employers of the deviations in writing accompanying the shop drawings including the reasons for the deviations. At the start of the Project the Contractor shall periodically and thereafter submit to the Employers list of all shop drawings which will be submitted in the course of the project. The list shall show the disposition of each item including date of submission, approval etc. The list shall be kept up to date through the entire course of construction.

O.3	Record Drawing

O.3.1	During Construction the Contractor shall keep an accurate record of all deviations between the work as shown on the Contract Drawings and that which is actually installed.

O.3.2	The Contractor shall secure from the Employers after approval of the Shop Drawing a complete set of drawing and note changes thereon in ink.

O.3.3	The Contractor shall make a complete record of all changes and revisions in the original design which exist in the completed work.

O.3.4	The cost of furnishing above prints and preparing these for record shall be deemed to be included in the tendered cost and its effects spread over other items of work, and as such item shall not be a subject to payment. When all revisions showing the work as finally installed the corrected Original Transparencies shall be submitted to the Employers before final payment for the completed work is made.

O.4	Operating and Maintenance Instructions

O.4.1	Three sets of operating and maintenance instruction covering completely the operation and maintenance of all plumbing equipment, controls, heaters, pumps and the like shall be furnished to the Owner, by the Contractor.

O.5	Tests

O.5.1	The entire system of drains, waste and vent piping inside and outside the building shall be tested by the Contractor under a water test, which shall include the entire system from the lowest point to the highest pipes above the roof.

O.5.2	The water test shall be made in accordance with all local requirements. Every portion of the system shall be tested to a hydrostatic pressure equivalent to latest 15 feet head of water. After filling, the Contractor shall shut off water supply and shall allow it to stand 2 hours under test during which time there shall be no loss or leakage.

O.5.3	The Contractor shall furnish and pay for device, material supplies, labour and power required for all tests. All tests shall be made in the presence and to the satisfaction of Employer.

O.5.4	Defects disclosed by the test shall be repaired, or if required by the Employer, defective work shall be replaced with new work without any extra charge to the Owner. Test shall be operated as directed until the work is proved satisfactory.

O.5.5	Fixture shall be tested for soundness, stability of support and satisfactory operation.

O.5.6	The Contractor shall notify the Employer at least one week in advance of doing the required test, so that arrangements may be made for their presence to witness the test.

O.5.7	Equipment shall be tested in service and the Contractor shall demonstrate that the equipment performs the work intended for it and that it complies with the requirement of these specification for such equipment, to the satisfaction of Employers.

O.5.8	The rates shall include all costs associated with tests.

O.6	Work in Common Piping

O.6.1	Material

O.6.1.1	Piping and fitting material shall be uP.V.C., Hard Impact P.V.C. or High Temperature P.V.C. and approved by the Employer.

O.6.1.2	Piping material shall comply with requirements of water supply and sewerage guidelines and other relevant authorities.

O.6.1.3	Materials for the piping and service requirements shall basically conform to the service pressures encountered.

O.6.2	Providing Drawings and Manuals

O.6.2.1	The Contractor shall submit one set of originals and further two copies of

layout drawings to the Employer after completion of the Works. These drawings must give the following information:

(a)	Run of all piping and diameter on all floors and the vertical stacks.

(b)	Location and sizes of all control valves, access panels and other equipment.

(c)	Location of all manholes and their sizes.

O.6.2.2	No completion certificate will be issued until the drawings are submitted.

O.6.2.3	The Contractor shall submit to the Employer for approval, samples, shop drawings, manufacturers’ drawings, equipment characteristics and capacity data etc. of all equipment, accessories, devices etc. that s/he proposes to use in the installation.

O.6.3	Samples

O.6.3.1	The Contractor shall provide samples of all sanitary fittings, pipes and specials man‐hole cover and frames, gratings and water supply pipes and fittings etc. and shall be deposited with the Employer (which will be returned to the Contractor at the completion of the Works) and shall obtain approval from the Employer before using in the Works. Any material rejected by the Employer shall be removed from the site within 24 hours of rejection.

O.6.4	Drawings

O.6.4.1	The works shall be done in conformity with the plans and within the requirements of the general architectural, electrical and structural plans. This work shall be properly coordinated with the work of the other trades. Hangers and sleeves shall be furnished in time for their installation as other work proceeds.

O.6.4.2	The plumbing drawings are diagrammatic but shall be followed as closely as actual construction. All deviations from drawings required to conform to the building construction shall be made by the Contractor’s own expense.

O.6.4.3	The architectural drawings shall take precedence over the plumbing drawings as to all dimensions.

O.6.4.4	Large size details shall take precedence over small size drawings. The special dimensions in the specifications or schedule of quantities or instructions of the Employer shall supersede the drawings. The Contractor shall verify all dimensions at site.

O.6.4.5	The recommended position of the fittings, fixtures, control valves, tanks etc. as shown on the drawings will be adhered to as far as practicable.

O.6.4.6	Should there be any discrepancy due to incomplete description, ambiguity or omission in the drawings and other documents, whether original or supplementary, forming the contract, either found on completion or during the currency of the installations work, the Contractor shall immediately, on discovering  the  same,  draw  the  attention  of  the  Employers. The Employer’s decision on the matter will be final and binding on the Contractor.

O.6.5	Existing pipes

O.6.5.1	The site shall be examined for field drains and those when found, shall be either entirely removed or diverted, trenches filled with dry earth in 200mm to 300mm layers and consolidated as directed by the Employer.

O.6.6	Spare Parts

O.6.6.1	Necessary spare parts of the plumbing equipment for the one (1) year operation shall be supplied by the Contractor.

O.6.7	Excavation

O.6.7.1	All excavations shall be timbered to the satisfaction of the Employer and the type of timber shall be suitable to the kind of earth encountered. Fixing of timber and removal after completion of work shall be done as directed by the Employer.

O.6.7.2	Should any water accumulate in the trenches, headings or other excavation, the Contractor shall do such work as may be necessary to drain away the accumulated water and shall install pumps as may be required to keep the excavation and trenches dry. The Contractor shall ensure that the flow water in trenches or excavation does not injure or remove cement or aggregate of any concrete that has not set. No subsoil water shall be discharged into open drains or sewer at the site.

O.6.7.3	In refilling trenches after excavation this should be done in layers of 150mm after consolidating each layer. Special care shall be to see that the earth is packed uniformly and no injury to the pipe.

O.6.7.4	Rates for excavation should include for backfilling in consolidated layers where necessary and as directed by the Employer.

O.6.8	Piping

O.6.8.1	The Contractor shall, as soon as possible after the award of the contract, prepare and submit to the Employer for approval, working drawings showing exact locations and pipe runs for all pipework, the layout and setting up of equipment and the connection of piping to the equipment. Such drawings shall include details and methods of supports, anchors and sleeves etc.

O.6.8.2	Pipe runs shown in the drawings are approximate and intended to indicate the general run and locations only. The exact locations of all pipework shall be determined on Site.

O.6.8.3	All pipes, fittings etc. shall be kept closed against moisture and foreign matters when stored at site and during installation.

O.6.8.4 	All pipes shall be fixed clear of one another  and  be  so  arranged  as  to provide easy access for maintenance and repair.

O.6.8.5	All plumbing work shall be carried out by suitably qualified plumbers in accordance with the British Code of Practice and Regulations and requirements of related Authorities.

O.6.8.6	Materials for the piping and service requirements shall basically conform to the service pressures encountered.

O.6.8.7	Each part of the installation of the plumbing work shall be completed in all details as shown in the drawings or as specified and provided with all necessary control valves, etc. that will be necessary for their satisfactory operation.

O.6.8.8	All piping shall be run plumb, and straight and parallel to walls, except drain line which shall pitch 6mm per 300mm in the direction of flow.

O.6.8.9	Pockets, unnecessary traps, turns and off‐sets shall be avoided. When traps or pockets are unavoidable they shall be valved drains.

O.6.8.10	Piping installed on the concrete slab shall be firmly fixed or anchored to the floor with packing to prevent damage to pipes. Pipes shall not be bent with bender where cross with other pipe or change to upward.

O.6.8.11	Where pipes are to be laid directly in the ground, bed shall be sufficiently compacted, necessary protection for piping shall be taken.

O.6.8.12	Backfill shall be done after the approval of the Employer in such a manner not to damage the pipeline and shall be restored to the original stage.

O.6.8.13	Where pipes penetrate through waterproof part or fire partition or fire wall, pipe sleeves shall be provided and clearance between pipe sleeve and pipe shall be filled with caulking material approved by the Employer.

O.6.8.14	Pipes, fittings, valves and accessories shall be thoroughly cleaned, both internally and externally before installation and shall be cleaned before putting into service.

O.6.8.15	Plumbing work shall be completed in accordance with the details shown on the Drawings or as specified and provided with all necessary control valves, etc. that will be necessary for their satisfactory operation.

O.6.8.16	All pipes shall be cut square and true to the pipe axis by means of suitable tools without reducing pipe diameter and cut ends shall be finished smooth. Before making connections, chips, dirt and other foreign matter shall be removed from inside interior of each pipe. Fixing of hangars and embedding of pipe sleeves shall be carried out without delay along with the progress of the work where required.

O.6.8.17	Pipe connections for the water supply system shall be by uP.V.C high pressure. Jointing shall be generally by means of solvent cement according to manufacturer’s instructions

O.6.8.18	Vertical pipes shall be braced at more than 2 point in every story.

O.7	Water Supply Work

O.7.1	Materials

Pipes, joints and fittings for water supply work shall be high pressure uP.V.C.

Materials and workmanship shall comply with the local water supply authority requirements.

O.8	Water Pumps

O.8.1	The specifications herein stated are basic guides only. Other items not so indicated but which are obviously necessary for the proper operation of the system as intended shall be supplied  and installed, in accordance with accepted Consulting standard.

O.8.2	Manuals of operation and maintenance and list of spare parts shall be supplied together with the equipment.

O.8.3	The contractor shall submit at least four copies of pump performance curves showing among others, the pump rating and efficiency, properly marked out.

O.8.4	A metal name plate indication in indelible letters for the correct specification of the pump and motor shall be properly attached to the assembly at a location such that the information written thereon can be conveniently read by all concerned.

O.8.5	Well water pump and Fresh water pump: Flow rate = 60L/min, Head = 70m, Type : End suction Hydro pneumatic pump, 220/440V, 3‐Phase, 50 Hz. Alternate and parallel operation. Fire pump: 50L/min, 70m head, Vertical multistage pump with alternative operations.

O.9	Spacing of supports

O.9.1	Support spacing for uP.V.C pipes shall be as follows

| Nominal Dia. (mm) | upto 40 | more than 50 |
| --- | --- | --- |
| Space (m) | 1.2 | 1.5 |

O.10	Drainage Work

O.10.1	General

O.10.1.1	High Pressure uP.V.C pipe and fittings shall be used for all drainage work including vent pipes.

O.10.1.2	Joints shall be made by the cold‐jointing method, and the pipe interior shall have not offset at the joint interfering with the flow. Joint adhesive shall be good quality and shall not be affected by heat and shock.

O.10.1.3	Where horizontal drain branch joins the main, such branch shall be connected to the main in a substantially horizontal position and at an acute angle of not more than 45 degree to the main in all cases.

O.10.2	Vent stack pipes

O.10.2.1	Vent pipe shall be vertically branched out upward from a horizontal drain branch pipe or other appropriate point. Horizontal branching of the vent pipe shall be done on approval of the Employer.

O.10.2.2	Where vent pipes on each floor are to be connected to the vent stack, all connections shall be made at least 150mm above the respective overflow edges of fixture on that floor.

O.10.2.3	The provision of the preceding item shall also apply to the connection of vent stack vent pipe.

O.10.2.4	Vent stack shall be connected to the waste stack or soil stack at the lowest part to stack pipe.

O.10.2.5	Where vent pipe is to be connected to the horizontal drainpipe, such angle shall be more than 45 degree to upward.

O.10.2.6	Vent stack shall be extended 600 mm from the top of the roof or lead to the wall and top of pipe shall be covered with vent cap.

O.11	Laying of Pipes

O.11.1	The pipes shall be laid to proper lines and levels as shown in the plans and directed by the Employer, as the main is laid, the front pipes in the trench shall always be closed with a plug either of iron or wood and security fastened. The plug shall not be removed except when pipe laying is resumed or for purposes of testing.

O.12	Laying of sewer water Mains

O.12.1	All mains shall be laid on a good solid bottom to prevent subsidence and consequent fracture.

O.12.2	Mains running under buildings, if unavoidable, shall be completely surrounded by 150mm of concrete.

O.12.3	In case of mains passing through a well, the weight of the latter shall be carried by a lintel or a suitable relieving arches.

O.12.4	All rising mains shall be properly plugged to all wall brackets at regular intervals as given in the drawings.

O.12.5	All mains shall be concealed inside wall as far as possible except for vertical sewer mains, cleaning doors shall be provided in the walls whenever necessary and as directed by the Employer.

O.13	Sewers

O.13.1	After the cement has had time to set, the pipes shall be tested in length between manholes in following manner.

O.13.2	In the lowest manhole/intercepting trap as the case may be, a plug shall be inserted in the pipe. The disc in the pipe at the upper manhole shall be fitted with a filling pipe with a right angle  bend and an air cook.

O.13.3	The pipeline shall then be filled with water by means of the pipe connection on the upper disc. The air cock on the upper disc shall be kept open while the pipeline is being filled to permit the escape of air.

O.13.4	When the pipes are filled with water and air excluded, the air cock shall be shut and the water shall be poured into conical filler, attached to the filling pipe until the water remains in the filter.

O.13.5	The filling pipe shall then be raised and fastened so that the height of surface of the water in the filler above the invert of the pipe is 1828 mm which will be usual test pressure for S.W. (sewer and waste) pipes. If the water level does not fall more than 16mm (12mm) in a length of 91.4 metre the test may be considered satisfactory.

O.13.6	The Contractor shall make good all defective work at own expense.

O.14	U.P.V.C Pipes

O.14.1	Manufacturer’s instruction should be followed in pipes to be used for water mains. Where specified, pipes shall have integral rubber ring joints and where solvent cement joints are specified, a sufficient number of expansion/contraction joints shall be incorporated in the length of mains to allow for variation of temperature to the recommendation of the pipe manufacturers.

O.14.2	These pipes shall be effectively protected from the direct rays of sun immediately after they are laid and until permission is given for the trenches to be refilled by the Employer. Subject to such permission being obtained, trenches shall be refilled without delay. Final connection at a fixed point shall be deemed up to the majority of the length of the pipeline has been covered by backfill in order to reduce the effect of expansion and contraction caused by temperature variations.

O.15	Bends and other Specials

O.15.1	In fixing bends, care shall be taken to see that the axis of the bend is truly vertical or horizontal as the case may be and the spigot of the bend is well in the socket of the pipe with which a joint has to be formed. The Contractor shall be called on to replace any faulty work at his/her own expense.

O.16	Flanged Joints

O.16.1	All flanged joints shall be made by painting the faces of the flanged with red lead freely and bolting the flanges evenly on all sides. A thin fiber of lead wool may be used in making the joints watertight when facing of the flanges is not true. Rubber insertions may be used with approval. Sewage resistant rubber insertion is to be used for sewer lines.

O.17	Support for U.P.V.C Pipes

O.17.1	When U.P.V.C pipelines incorporate metal valves or other heavy fittings, it is essential to support the valves directly rather than allowing their weight to be carried by the uP.V.C pipe and support shall be placed on either side of the fittings mentioned above. Moulded plastic fitting also should be supported.

O.17.2	Maximum allowable horizontal support distance for uP.V.C are given below.

| Nominal bore | 12 mm (1/2”) | 18 mm (3/8”) | 25 mm (1”) | 32 mm (1¼”) | 38 mm (1½”) | 50 mm (2”) |
| --- | --- | --- | --- | --- | --- | --- |
| Support distance | 533 mm (1’9”) | 616 mm (2’0”) | 686 mm (2’3”) | 764mm (2’6”) | 840 mm (2’9”) | 915 mm (3’0”) |
| Nominal bore | 75 mm (3”) | 100 mm (4”) |  |  |  |  |
| Support distance | 1220 mm (4’0”) | 1290 mm (4’6”) |  |  |  |  |

O.17.3	For vertical installation supports, distances shall be doubled.

O.18	Sewer pipes

O.18.1	All ‘P’, ‘S’, ‘I’ junctions bends etc. required shall be furnished and set without extra charge and shall confirm to the pipe specifications as to quality

O.19	Air Valves

O.19.1	These valves to be fitted as per drawings and Bill of Quantities shall be tested and accompanied by certifying their efficiency.

O.19.2	The floating ball in the valve shall be suitable metal or vulcanite or rubber specially manufactured for tropical conditions.

O.20	Scour Washout Valve

O.20.1	These shall be provided at portions shown in place and shall contain in one unit a flanged scour valve with short connection pieces, cast iron bend and T pieces for connection to main pipe.

O.20.2	The rate shall also provide for short length of straight pipe as per details, complete with covers and surface boxes.

O.21	Foot valves and Strainers

O.21.1	Foot valve and strainers should be of reputable manufacture approved by the Employer and shall be fitted with flushing lever attachment where specified.

O.22	Pressure Reducers

O.22.1	Pressure reducing valves shall be of the equilibrium type of approved manufacture and capable of reducing the pressure to the valve required as per plan and Bill of Quantities.

O.23	Water Meter

O.23.1	The water meters shall be from a reputed manufacture and shall be approved by the employer before installation.

O.24	Equilibrium Ball Valves

O.24.1	These should be of reputable manufacture approved by the Employer and be of the angle pattern with gun metal valve seats guide bush, copper float with wrought iron lever and links with bronze pins.

O.25	Fittings

O.25.1	All sanitary pipes, gullies, water closets/bidets, squatting basins, sinks, bathtubs etc. to be of approved design and to be obtained from approved Manufacture and to be of the best stoneware, glazed inside and outside, with burnt hard and sound, free from flaws, blisters, cracks and other imperfections and best quality commonly called ‘Firsts’.

O.25.2	Rates should include for all bends, junctions, traps, cleaning, painting, fixing clear of wall etc. complete as specified as per Bill of Quantities.

O.25.3	All pipes, fittings, flushing cisterns, valves, stop cocks, taps, tanks, surface boxes etc. to be of the best of their kinds and in addition to complying with previous clauses to be from approved Manufacturers and all taps, cocks, valves etc. to be screwed down pipe. Taps to be as per manufacturers’ specifications (following with clause 15.28.1) and valves to be of gun metal. All tanks to be made fly‐proof and to the complete satisfaction of the Employer.

O.25.4	Rates should include all cutting and waste, bends, taps, junctures, cleaning eyes, tees.

O.26	Manholes, Manhole covers and Frames

O.26.1	Concrete cover slabs or top rings of manholes shall provide a suitable seating for a rectangular cover.

O.26.2	The frame shall have a clear opening of 0.61m x 0.61m or alternatively a circular or double triangular cover depending on the type of cast iron manhole cover to be used. The rate for manholes shall allow for such provision.

O.26.3	Where the supply of cast iron manhole cover and frames is payable separately the cost of setting, surrounding, painting and materials for same shall be allowed for in the rate for manholes.

O.26.4	Suitable lifting rings, hooks or brackets shall be provided in the precast manhole sections. Box holes shall be separately grouted with 1:2 cement mortar.

O.26.5	The contractor shall supply two manhole keys for each pattern of cover without additional charge over the rate for covers (or manholes).

O.26.6	Heavy duty (grade a) cast iron manhole cover and frames shall be of the double triangular type to bs and having a clear opening of 550mm dia.

O.26.7	Medium duty (grade b) cast iron manhole covers and frames shall be of the circular type having a clear opening of 550mm dia or the rectangular type having a clear opening of 0.61m x 0.61 and conform to relevant British Standards. They shall be of the single seal type, the weight of cover frame being approximately 127.00 kgs.

O.26.8	Light duty (grade c) cast iron manhole cover and frames shall be of the doubles seal flat type having a clear opening of 0.61m x 0.61m conforming to bs. Weight of cover and frame approximately 50.75kgs.

O.26.9	All manhole covers and frames shall be supplied, coated with a black bituminous composition and be given two coats of bituminous paint after bedding.

O.26.10 No extra rate is payable for drop and/or junction manholes but piping in and surrounds of drop lines are payable at that relevant rates for sewer and waste piping and manholes.

O.26.11 In drop manholes where the difference in level between the incoming drains and the sewer does not exceed 0.610m in 75mm and there is sufficient room in the manhole, the connecting pipe may be brought directly through the manhole wall, and the fall accommodated by constructing a ramp in the benching of the manhole. The ramp shall be of concrete and finished equal to that of the benches. No extra rate is payable.

O.27	Interceptor Manhole

O.27.1	All gravity sewer lines should be, connected through an intercepting inspection chamber before connecting to the main sewer line, and the dimensions of the manhole and trap to be in conformity with the Utility Regulatory Authority standards.

O.28	Fixtures and Accessories

O.28.1	Only following brands shall be used for all sanitary wares and to be installed as per finishing schedule / detail drawings. Samples to be approved prior to use.

1.	Duravit

2.	Grohe

3.	Cotto

4.	Arrow

5.	Water tec (Only for taps where required)

O.29	As built Drawings

O.29.1	The Plumbing Contractor shall mark down with red pencil on two sets of plumbing plans all the revisions, omissions and/or additions to the various plumbing installation drawings as the construction progresses. One set of the plans as marked shall be submitted to the Employer after completion of the work.

O.29.2	Before the final payment is made to the Contractor, s/he shall submit to the Owner through the employer, all As‐Built Drawings incorporating the changes made and noted in the marked plans retained by him/her. The As‐Built Drawings shall be prepared in reproducible form.

O.29.3	The Plumbing contractor shall prepare and submit the As‐Built Drawings without extra cost to the Owner.

O.30	Miscellaneous

O.30.1	Throughout the construction period, open ends of all installed pipelines shall be kept closed by temporary plugs. Drainage lines shall not be used to drain any construction waste, especially, those with cement, to avoid possible clogging.

O.30.2	A temporary fire protection system at each building shall be provided by the Contractor during the construction period. This shall be of sufficient capacity to put out any fire that may break out on any of the building floors due to construction period. This in addition to temporary fire extinguishers are required.

O.30.3	A temporary potable water supply shall be available to construction workers at each building floor as construction work progresses.

O.30.4	A temporary human Excrete Disposal System shall be provided by the Contractor to serve the workers during the construction period.

O.31	Height of Fixture Installation

Height of fixture shall be as follows unless otherwise specified on the Drawings

| Fixture |  | Height (mm) |
| --- | --- | --- |
| Wash Basin | Floor finish to front top edge ‐ Male Floor finish to top of mirror ‐ Male ‐ Female | 700 1675 1660 |
| Lavatory | Floor finish to front top edge | 760 |
| Shelf | Floor finish to top of shelf ‐ Male ‐ Female | 1005 990 |
| Cistern | Floor finish to bottom of cistern Floor mounted Japanese type Western type | 500 550 |
| Drinking fountain | Floor to front top edge | 765 |
| Flush valve, WC | Floor to center of valve | 600 |
| Paper holder | Floor to center of holder ‐ Japanese type ‐ Western type | 400 750 |
| Faucets Sink Lavatory Bathroom | Sink floor to top of faucet Lav. top to top of faucet Floor finish to top faucet | 300 150 300 |

O.32	Pool pump/filter system

O.32.1	Methodology and specifications shall be submitted to Employer before finalizing and brand for pump & filtering system quality shall be top of its class.

O.32.2	Installation shall be done by qualified personal of this trade and all the accessories and fixing shall be followed accordance with manufacture’s specifications (Approved).

- PAINTING

P.1	Material

P.1.1	All paints shall be approved by the Employer for colour, quality and type. All painting work shall be carried out in accordance with the paint manufacturer’s specifications unless otherwise directed by the Employer.

P.1.2	All paints and finishes used for the project shall be manufactured by or under license from the following manufacturers:

Nippon Paints (Japan)

Imperial Chemical Industries (UK)

Sigma Paints (Saudi Arabia)

Paints from manufacturers not listed above shall only be used with prior written approval of the Employer

P.1.3	Paint shall be ready mixed and all paints, varnishes, enamels, lacquer stains, paste fillers and similar materials shall be delivered to the site in the original containers with the seals unbroken and labels intact. Each container shall give the manufacturer’s name, type of paint, colour of paint and instructions for reducing. Thinning shall be done only in accordance with the manufacturers’ directions.

P.1.4	Use of product by the same manufacturer shall be a general rule in each stage of work in this Specification.

P.1.5	Colour, luster, colour scheme, finish shall be decided by the Employer after checking sample paint test.

P.1.6	The painting shall be performed by experienced and competent painter.

P.1.7	Where walls are specified to be painted, all columns, arises, groove, rough surfaces, reveals, soffits and returns, etc. shall be included and no extra shall be payable.

P.2	Definition of Terminology

Surface Sealing

Surface to be painted shall be sealed to have uniform suction and prevent lye from oozing out.

Spot Puttying

All cracks and depressions shall be filled flush with putty.

Puttying

All surfaces to be painted shall be puttied uniformly flat surface.

Spot painting

Spot puttied area shall be touched up by paint

Touch‐up

Any damaged area after the prime coat has been applied shall be touched up

Drying hour

The drying time of double‐coated paint shall be measured at the temperature of 20°C and humidity of 70%.

Amount of paint

The amount shall be standard amount of paint itself not including thinner. It shall increase or decrease depending on shape and surface condition in the process of painting.

P.3	Paint Finish Symbols

OP		Synthetic resin mix paint finish

VP		Solvent‐polyvinyl chloride resin paint finish

EP		Polyvinyl acetate resin emulsion paint finish

AEP		Synthetic resin emulsion paint finish

CL		Clear lacquer finish

EXP		Epoxy resin paint finish

Stipple (OP)	Stippled finish (oil mix paint finish)

Stipple (EP) 	Stippled finish (polyvinyl acetate resin emulsion paint finish

P.4	Painting in General

P.4.1	Preparation of Paint

P.4.1.1		Mixing: Paint content with pigment shall be thoroughly stirred to make a uniform consistency.

P.4.1.2		Thinning: Portable water shall be used for thinning of emulsion paint and water‐soluble paint. Proper thinner, product of the same manufacturer as paint, as a rule, shall be used for other types of painting. Percentage of thinning and viscosity shall be conducted with direction of manufacturer or catalogue as they vary with the method of paint, temperature, type of material to be painted.

P.4.1.3		Allowable period of Use: Paint mixed with more than 2 types shall be used with direction of a manufacturer or catalogue as allowable period of use, mixing ratio and mixing method vary. The paint which has passed allowable period of use shall not be used.

P.4.2	Conditions of Painting

P.4.2.1		Work shall not be executed in the following situations:

P.4.2.1.1	When humidity is above 85%

P.4.2.1.2	When raining or it is forecast

P.4.2.1.3	When dusts are present

P.4.2.1.4	When temperature of surface is high under hot weather and bubbles are likely to develop on the painted surface.

P.4.2.2		Conditions of Surface to be painted: Work shall not be executed or proper means shall be taken in the following situations:

P.4.2.2.1	When surface is damp and wet

P.4.2.2.2	When condensation is likely to develop on the surface.

P.4.2.2.3	All nail holes on veneer, board. etc., shall be covered with proper rust‐proof paint before the subsequent painting is applied in accordance with this specification.

P.4.3	Performance

P.4.2.2.4	Paint shall be evenly and uniformed applied on the surface. Areas of difficult application such as pointed part, internal angle, welded part, etc. shall be thoroughly painted and double coated as necessary to deep uniform coating thickness.

P.4.2.2.5	Painting shall be properly done by carefully selecting the painting method by the shape of surface and types of paint.

P.4.4	Protection

P.4.4.1		Dangerous material such as paint, thinner, etc., excluding emulsion paint and water‐soluble paint shall be kept in accordance with regulations concerned.

P.5	Procedure of Painting

P.5.1	Exterior - Surface of Mortar, Plaster and Concrete

AEP‐ Weather shield Exterior paint

| Coating Process | No. of Coats | Type of Paint | Drying hour |
| --- | --- | --- | --- |
| 1. Surface preparation |  | Dry, clean and free from impurities |  |
| 2. Surface sealing | 1 | Exterior Wall Sealer | As per manufacturers’ specifications |
| 3. Texture base | 2 | texture base putty mechanically sprayed uniformly | As per manufacturers’ specifications |
| 4. Surface finishing |  | Flatten with masonry trowel uniformly surfaced |  |
| 5. First coating | 1 | Weather shield paint | As per manufacturers’ specifications |
| 6. Finish coating | 2 | Weather shield paint | As per manufacturers’ specifications |

Notes:

  - Degree of dryness on the surface to be painted shall be kept under 6% in water content and below pH 9.5

  - Puttying and sanding process shall be allowed to omit depending on the conditions of the surface.

  - Drying time of putty shall be long enough for sanding to proceed.

  - Amount of sealer for surface sealing shall be adjusted with direction of the Employer as it varies with the surface conditions.

P.5.2	Exterior ‐ Iron Products in General OP ‐ Synthetic resin mix paint

OP ‐ Synthetic resin mix paint

| Coating Process | No. of Coats | Type of Paint | Drying hour |
| --- | --- | --- | --- |
| 1. Surface preparation |  | Completely remove rust, moisture, oil and other impurities by sander, cleaner and surface. |  |
| 2. First Coating 24 hours | 1 | Rust proof oil paint | As per manufacture’s specifications |
| 3. Touch‐up |  | Touch‐up rustproof oil paint |  |
| 4. First Coating | 1 | Rustproof oil paint | As per manufacture’s specifications |
| 5. Second coating | 1 | Synthetic resin mix paint | As per manufacture’s specifications |
| 6. Finish coating | 1 | Synthetic resin mix paint | As per manufacture’s specifications |

Note:

- Paint for touch‐up painting shall be the same as used for first coat in process No. 2

P.5.3	Exterior ‐ Wood

OP ‐ Synthetic resin mix paint finish

| Coating Process | No. of Coats | Type of Paint | Drying hour |
| --- | --- | --- | --- |
| 1. Surface preparation |  | Clean and sand to plane surface |  |
| 2. Knot treatment | 1‐2 | Lacquer varnish | As per manufacture’s specifications |
| 3. First coating | 1 | First coat paint of oil mix paint | As per manufacture’s specifications |
| 4. Second Coating | 1 | Oil mix paint | As per manufacture’s specifications |
| 5. Finish coating | 1 | Oil mix paint | As per manufacture’s specifications |

Note:

- Puttying and sanding shall be done after process No.2 when there are cracks, etc. on the surface putty shall be oil‐putty, but drying time shall vary depending on conditions.

P.5.4	Interior ‐ Mortar, board, etc.

Stipple (EP) ‐ Polyvinyl acetate resin emulsion paint finish

| Coating Process | No. of Coats | Type of Paint | Drying hour |
| --- | --- | --- | --- |
| 1. Surface preparation |  | Dry, clean and free from impurities |  |
| 2. Surface sealing | 1 | Sealer for emulsion paint | As per manufacture’s specifications |
| 3. Puttying |  | Putty for emulsion paint |  |
| 4. Grinding |  | Grind with proper grinding tool |  |
| 5. Spot painting |  | Second coating paint of polyvinyl acetate resin emulsion paint |  |
| 6. Second Coating | 2 | Polyvinyl acetate resin emulsion paint | As per manufacture’s specifications |
| 7. Finish Coating | 1 | Polyvinyl acetate resin emulsion paint for stipple‐finish | As per manufacture’s specifications |

Notes:

- Degree of dryness on the surface to be painted shall be kept under 6% in water content and below pH 9.5

- Puttying and sanding process shall be allowed to omit depending on the conditions of the surface.

- Drying time of putty shall be long enough for sanding to proceed.

- Amount of sealer for surface sealing shall be adjusted with direction of the Employer as it varies with the surface conditions.

P.5.5	Interior ‐ Mortar, plaster, concrete, etc.

VP Solvent ‐ Polyvinyl chloride resin paint finish

| Coating Process | No. of Coats | Type of Paint | Drying hour |
| --- | --- | --- | --- |
| 1. Surface preparation |  | Dry, clean and free from impurities |  |
| 2. Surface sealing | 1 | Sealer for emulsion paint | As per manufacture’s specifications |
| 3. Puttying |  | Putty for polyvinyl chloride resin paint |  |
| 4. Grinding |  | Grind with proper grinding tool |  |
| 5. Spot painting |  | Solvent‐polyvinyl chloride resin enamel emulsion paint |  |
| 6. Second Coating | 1 | Solvent‐polyvinyl chloride resin enamel emulsion paint | As per manufacture’s specifications |
| 7. Finish Coating | 2 | Solvent‐polyvinyl chloride resin enamel emulsion paint | As per manufacture’s specifications |

Notes:

- Degree of dryness on the surface to be painted shall be kept under 6% in water content and below pH 9.5

- Puttying and sanding process shall be allowed to omit depending on the conditions of the surface.

- Drying time of putty shall be long enough for sanding to proceed.

- Amount of sealer for surface sealing shall be adjusted with direction of the Employer as it varies with the surface conditions.

P.5.6	Interior ‐ Mortar, plaster, concrete, etc. EP Polyvinyl acetate resin emulsion paint finish

EP Polyvinyl acetate resin emulsion paint finish

| Coating Process | No. of Coats | Type of Paint | Drying hour |
| --- | --- | --- | --- |
| 1. Surface preparation |  | Dry, clean and free from impurities |  |
| 2. Surface sealing | 1 | Sealer for emulsion paint | As per manufacture’s specifications |
| 3. Puttying |  | Putty for emulsion paint |  |
| 4. Grinding |  | Grind with proper grinding tool |  |
| 5. Spot painting |  | Polyvinyl acetate resin emulsion paint |  |
| 6. Second Coating | 1 | Polyvinyl acetate resin emulsion paint | As per manufacture’s specifications |
| 7. Finish Coating | 1 | Polyvinyl acetate resin emulsion paint | As per manufacture’s specifications |

Notes:

- Degree of dryness on the surface to be painted shall be kept under 6% in water content and below pH 9.5

- Puttying and sanding process shall be allowed to omit depending on the conditions of the surface.

- Drying time of putty shall be long enough for sanding to proceed.

- Amount of sealer for surface sealing shall be adjusted with direction of the Employer as it varies with the surface conditions.

P.5.7	Interior ‐ Iron products, steel. OP ‐ Synthetic resin mix paint

OP ‐ Synthetic resin mix paint

| Coating Process | No. of Coats | Type of Paint | Drying hour |
| --- | --- | --- | --- |
| 1. Surface preparation |  | Completely remove rust, moisture, oil and other impurities by sander, cleaner and surface |  |
| 2. First Coating | 1 | Synthetic resin rust‐proof. Red lead‐type, lead compound‐ type | As per manufacture’s specifications |
| 3. Touch‐up |  | Touch‐up rust proof paint |  |
| 4. First Coating | 1 | Synthetic resin rust‐proof paint. Red lead‐type, Lead compound‐ type | As per manufacture’s specifications |
| 5. Second Coating | 1 | Synthetic resin mix paint | As per manufacture’s specifications |
| 6. Finish Coating | 1 | Synthetic resin mix paint | As per manufacture’s specifications |

Notes:

- Paint for touch‐up painting shall be the same as used for first coat in process No.2

- When oil rust‐proof paint is used instead of synthetic resin rust proof, its specification shall conform to No. 5 and No.6.

P.5.8	Floor ‐ Concrete and Mortar EXP ‐ Epoxy resin paint finish

EXP ‐ Epoxy resin paint finish

| Coating Process | No. of Coats | Type of Paint | Drying hour |
| --- | --- | --- | --- |
| 1. Surface treatment |  | Dry, clean and free from impurities |  |
| 2. First coating | 1 | First coating paint for epoxy | As per manufacture’s specifications |
| 3. Finish Coating | 2 | Epoxy resin paint | As per manufacture’s specifications |

Notes:

- Degree of dryness on the surface to be painted shall be kept under 6% in water content and below pH 9.5.

- Amount of paint and number of paint shall be as directed by the Employer as they vary with the conditions of surface and required thickness of coating.

- Painted surface shall be kept out of use for more than 7 days after application of final coat.

- ELECTRICAL INSTALLATIONS

Q.1	General

Q.1.1	The work shall be carried out strictly in accordance with the standard specifications and shall also conform to the requirements of Electricity Rules in force in Male’, Republic of Maldives.

Q.1.2	All materials to be used in the Works shall be of standard make and shall bear the certification marks of local authorities. All materials shall be approved by the Employer before use in the Works.

Q.1.3	Earthing shall invariably be done in the presence of the Employer or his representative.

Q.1.4	All the conduits shall be continuously earthed. Check nuts shall be provided at the point where the conduits enter the Inspection Chamber box and junction box.

Q.1.5	The Contractor shall arrange for the inspection of all Medium Pressure Installation by the Electrical inspector of the local electric supply authority from where the electricity connections has to be obtained and see that they are passed by him/her.

Q.1.6	The Contractor shall be responsible for all necessary permits, approvals, fees, deposits etc., required to complete the Electrical works in accordance with the Contract.

Q.1.7	Scope of work

Q.1.7.1	The works consists of furnishing all tools, plants, labour, materials and equipment and performing the internal electrical Works comprising of:

(a)	Light and power wiring

(b)	Fans and fixtures

(c)	Wires and Cables

d)	Telephone System

(e)	Sub‐ Station Equipments:

(f)	Distribution Fusegear

(g)	Earthing System

(h)	Lightening Protection System

(i)	Fire Alarm System

(j)	Air Conditioning System

(k)	Computer Network Cabling outlet work

Q.1.8	Prequalification

Q.1.8.1	The Electrification Work shall be carried out only by a licensed contractor authorized to undertake such work under the Utility Regulatory Authority.

Q.1.9	Qualification

Q.1.9.1	A licensed Electrical Contractor should have the following qualifications:

(a)	Must have a competent Electrical Engineer registered with Utility Regulatory Authority.

(b)	Must have in its employment an Electrical Consultant having certificate of competency who will exclusively supervise this work.

(c)	Must have necessary tools, plant and instruments.

(d)	Must have adequate experience of similar works.

(e)	If a contractor does not possess the above qualifications the contractor shall be allowed to sublet the Work to a competent Sub‐Contractor provided an application for the prequalification made to the employer for approval. Decision of the Employer in this case shall be binding on the Contractor.

Q.1.10	Rules and Regulations

Q.1.10.1	The installation in general shall be carried out in conformity with the Electricity Rules, 1937 (UK), and the latest edition of the Regulations for the Electrical Equipment of Buildings issued by the Institution of Electrical Engineers, London (I.E.). However, in case of conflict between these Specifications and the I.E. Regulations, these Specifications shall be followed.

Q.1.11	Standards

Q.1.11.1	The latest relevant British Specifications, and I.E. recommendations shall be applicable and be followed for the equipment specified herein.

Q.1.12	Climatic Conditions

Q.1.12.1	All equipment supplied shall withstand, without developing any detect, the following climatic conditions:‐

Maximum Ambient Temperature	= 	113° F or 45° C

Minimum Ambient Temperature	= 	28° F or ‐ 2.2° C

Maximum Humidity			=	98%

Q.1.13	Specifications

Q.1.13.1	The Contractor shall furnish all material and equipment at site, confirming fully to the specifications given herein and to Maldivian regulatory requirements.

Q.1.13.2	It is not the intent of these Specifications to include all details of design and construction of various material and equipment to be supplied under this contract.

Q.1.13.3	The Contractor shall supply and install all material and equipment specified herein and also all installation and small material such as nuts, bolts, washers, shims angles, levelling material, insulation, tape, solder, etc. and all such required for complete installation as intended by the Specifications.

Q.1.13.4	The contractor shall provide for all the required technical and non ‐ technical personnel, skilled and non ‐ skilled labour, construction equipment, transportation etc., as required for the completion of Work in strict accordance the Technical Specifications laid herein‐after.

Q.1.13.5	All material and equipment supplied by the Contractor shall be new and in all respects conforming to the high standard of engineering design and workmanship.

Q.1.13.6	All material and equipment which has to be supplied and installed by the Contractor shall be passed/approved by the Employer, even if the same is exactly in accordance with the Bill of Quantities and Drawings.

Q.1.14	Submittal

Q.1.14.1	The Contractor, after the award of work, shall submit for approval of the Employer all drawings and cuts of equipment, appliances, fixtures and accessories. Cuts, catalogues and drawings shall be clearly marked to indicate the items furnished.

Q.1.15	Approval of Drawings and Data

Q.1.15.1	The Contractor shall provide detailed electrical drawings, wire diagrams, etc. for all electrical switchgear, fusegear and all other systems etc. for the Employer to review and approval. Three sets of equipment drawings shall be provided for obtaining approval.

Q.1.16	Drawings & Data

Q.1.16.1	Three sets of drawings and data for each equipment shall be furnished by the Contractor for the Employer approval before commencement of work. The drawings to be supplied by the Contractor shall be as follows:‐

Electrical Drawings showing:‐

(a)	Single‐Line diagram

(b)	Detailed wiring diagram

(c)	All interconnections

(d)	Relays, their locations, and internal wiring diagrams

(e)	Other electrical devices including meters instruments and their wiring diagram

(f)	Transformer switchgear calculations and drawings

Q.1.17	Shop Drawings

Q.1.17.1	The design drawings do not show conduit routes and depict only the position of various fixtures and outlets. All the planning for the conduit routes shall be carried out, well in advance of the actual execution of work, by the Contractor to the satisfaction of the Employer. For this purpose, the Contractor shall prepare shop drawings and obtain prior approval of the Employer. There prints of each shop drawings shall be submitted for obtaining approval. work.

No piece of work shall be allowed to be executed at site without the availability of these approved shop drawings. These shop drawings shall clearly depict the load balancing chart of each Distribution Board.

Time required for the preparation and approval of shop drawings shall be considered to have been included in the total time allowed for the completion of the work.

Q.1.18	Spare Parts list

Q.1.18.1	A list of spare parts required for one year’s operation of each equipment where deemed necessary together with unit price of each part, shall be supplied by the contractor.

Q.1.19	Guarantee

Q.1.19.1	The Contractor shall furnish written guarantee in triplicate of the manufacturer for successful performance of each equipment. Such guarantee shall be for replacement which may be found defective in material or workmanship.

Q.1.19.2	The guarantee shall cover a minimum period of 12 months effective from the date of completion certificate.

Q.1.20	As‐Built Drawings

Q.1.20.1	The Contractor shall, during the progress of work, keep a careful record of all changes and revisions where the actual installation differs from that shown on shop drawings. These changes and revisions shall be accurately carried out on the shop drawings and submitted to the Employer for approval. After approval these drawings shall become the property of the Owner. These updated and approved shop drawings depicting clearly all changes and revisions made on site shall be called As‐Built Drawings.

Q.1.20.2	Reproducible tracings of all these As‐Built Drawings shall be handed over to the Employer. Final payment will be withheld until the receipt of the approved As‐Built Drawings.

Q.1.21	Test Reports

Q.1.21.1	The Contractor shall be responsible for the submitting the test reports/certificates and get the installation inspected passed by the Utility Regulatory Authority.

Q.2	Conduit And Conduit Accessories

Q.2.1	Conduit Pipe

Q.2.1.1	The conduit for the wiring of lights, socket outlets and other systems shall be made of PVC confirming to BSS 3505/1968 Class‐D.

The conduit shall have following wall thickness and standard weights:

| Pipe Size | Wt/100Rft. | Wall thickness |
| --- | --- | --- |
| 20mm dia | 3.4 Kg | 0.04 to 0.05 |
| 25mm dia | 4.5 Kg | 0.045 to 0.055 |

Q.2.1.2	Steel conduit shall conform to BSS 31/latest. The conduit shall be enameled with good quality non‐ cracking and non‐flaking black paint.

Q.2.2	Conduit Accessories

Q.2.2.1	The use of factory made round PVC junction boxes shall be used and should have nipples to receive PVC pipe with force fit, shall be used for ceiling outlets. The wall type junction box shall also be PVC.

Q.2.2.2	Each junction box shall be provided with one piece cover which shall be fitted on the box with screws.

Q.2.2.3	Conduit accessories such as switch boxes, socket outlet boxes, pull boxes and inspection boxes shall be made of PVC having dust tight covers. All boxes shall have required number of conduit entry holes. All the rectangular or square shaped boxes shall have nipples to receive PVC conduit force fit.

Q.2.2.4	Manufactured smooth bends shall be used where conduit changes direction. Bending of Conduit by heating or otherwise shall be allowed only at special situations with the permission of the Employer. Use of sharp 90 degree bends and tees are prohibited.

Bends shall have enlarged ends to receive the conduit without any reduction in the internal diameter of the PVC pipe.

Q.2.2.5	All accessories e.g. boxes, coupling, bends, solid plugs, bushes, reducers, checknuts etc. shall be equal in quality to the specified conduit.

Q.2.2.6	The drawings do not show conduit routes and all the planning for arranging conduit routes shall be carried out by the Contractor to the satisfaction of the Employer.

Q.2.2.7	The entire conduit system shall be essentially completed before the wiring pulling is taken in hand. Each conduit run shall be tested for continuity and obstructions. All obstructions shall be cleared in an approved manner. Water and moisture that has entered any section of the conduit installation must be dried with suitable swabs to the satisfaction of the Employer.

Q.2.2.8	Adequate expansion joints shall be provided in all conduit runs passing across the expansion joints in the concrete slab of the buildings.

Q.2.2.9	All the free ends of conduit shall be solidly plugged till such time as final and proper terminations are made.

Q.3	Wires, Cables and Cords

Q.3.1	Wires & Cords

Q.3.1.1	The wires & cords for the conduit wiring shall be single core, made of stranded copper conductors, PVC insulated, tested to B.S. 6004, 1975. The voltage grade shall be 300/500 volts or 450/750 V unless otherwise specified on Drawings and Bills of Quantities.

Q.3.1.2	All the wire and cables shall be of the approved standard of Utility Regulatory Authority.

(a)	For light or fan point wiring with 1.5 mm square or as specified in the BOQ.

(b)	For light circuit wiring with 2.5 mm square or as specified in the BOQ.

(c)	For power plug 15A wiring with 4mm square or as specified in the BOQ.

Q.3.2	Installation Instructions

Q.3.2.1	All wiring shall be continuous between terminations and use of connectors or joints is not allowed. Spur and tee connections are strictly prohibited.

Q.3.2.2	Manufacturers’ recommended lubricant shall be allowed to facilitate pulling of wires. Use of any kind of oil and soap is prohibited.

Q.4	Wiring Accessories

Q.4.1	Switches

Q.4.1.1	Indoor switches controlling lights and fans shall be single pole, 5A, one or two way, suitable for 250V,50 Hz. The body of the switches shall be made of moulded plastic, one, two, three or four gang with integral built in moulded plastic face plate.

Q.4.1.2	Weatherproof switches shall conform to British Standards.

Q.4.2	Switch Socket Outlet Units

Q.4.2.1	Switch & socket units shall be single, pole, 3 pin rated 5A. 15A or 20A,  250V, 50 Hz. These shall be moulded plastic type with white integral built‐in face plate. Each socket shall have its control switch by the side of it on a common face plate. Thus, the complete unit specified in BOQ shall be as switch and a socket outlet unit.

Q.4.3	Fans

Q.4.3.1	All fans shall be as per finishing schedule / details drawings and suitable for operation on 200/220 volts, 50 Hz, A.C Supply. All ceilings fans shall have five speed dimmers. The air displacement shall be 10,000 c.f.m for 48” (1219 mm) Sweep and 12,000 c.f.m. for 56” (1423 mm) Sweep at maximum speed. The fan motor shall be capacitor type and bearings shall be groove type to give noiseless and quiet operation. The noise level relative to a frequency of range 1000 Hz should be within the limits of +3 dB.  The blades of the fan shall be of Aluminium (minimum 4mm thick) and shall be approved by the employer.

Q.4.4	Dimmer

Q.4.4.1	The dimmer shall be recessed type as required and shall be approved by the Employer.

Q.4.5	Fan Hook

Q.4.5.1	The fan hook shall be made of 12 dia mild 5/5 steel rod bent to shape of approved design. It should be in the form of a loop about 3‐1/4” (87.5 mm) long and about 2” (50 mm) wide. The rod shall be bent to have at least 8” (200 mm) extension on both sides for teeing to the reinforcement steel of the slab. All ceiling fans shall be of one make only.

The fan hook shall be installed in the RCC slab of the ceiling at the time of pouring concrete.

Q.5	Light Fixtures

Q.5.1	General

Q.5.1.1	The description of light fixtures is given in the Bills of Quantities, and stated on the Drawings/ Finishing schedule, and all relevant materials are described in this Section.

Q.5.1.2	The determination of quality is based on certified photometric data covering the coefficient of utilization, light distribution curves, construction material, shape, finish, operation, etc.

Q.5.1.3	The Contractor shall submit samples of each and every lighting fixture specified for approval of the Employer.

Q.5.1.4	The type of fixtures with manufacturer catalogue references are given in Bill of Quantities and finishing schedule.

Q.5.1.5	The lighting fixtures shall be manufactured by suppliers (manufacturers) as per BOQ / finishing schedules / detail drawings, approved by Employer.

Q.5.2	Incandescent Light Fixture

Q.5.2.1	The glass globes/ shades/ diffusers of the incandescent light fixtures shall be first class quality glass free from any air bubbles or voids. The glass shall generally be of opal white colour unless otherwise specified. The shape of the glass may be spherical, hemispherical, flattened bottom or tablet shaped as required.

Q.5.2.2	Surface mounted fixture shall have stove enamelled sheet steel body. It may also be satin brass or aluminium anodised finish as required. The fixing holes shall match the outlet box. Wall bracket light fixtures shall have back plates with matching holes of the outlet box and decorative finish as required.

Q.5.2.3	All the lighting fixtures shall be suitable for local climatic conditions.

Q.5.3	Fluorescent Light Fixture

Q.5.3.1	All the light fixtures shall have lamps and electronic ballasts of the wattage specified. The fluorescent lamp shall be per finishing schedule / details drawings and the colour shall generally be daylight, cool day light in the order of preference or as mentioned specifically.

Q.5.3.2	The fluorescent lamps shall be products mentioned in finishing schedule / detail drawings having a minimum useful life of 5000 hours.

Q.5.3.3	The ballast shall be totally enclosed electronic type suitable for operation on 220 V, 50 Hz, single phase supply, a wiring diagram, wattage, voltage and current ratings shall be printed on the body of the ballasts. The power loss shall not be more than 10 watts for 36 watts ballast. The ballast shall be noiseless in operation without any whistling sound.

Q.5.3.4	The manufacturer shall be called upon to guarantee a trouble free life of 3 years, effective from the date of completion certificate.

Q.5.3.5	The starters shall have radio‐interference suppressers.

Q.5.3.6	The internal wiring of light fixtures shall be carried out at manufacturer’s factory with heat resistance wires of size not less than 1.5 mm square.

Q.5.3.7	The louvers of light fixtures shall be made of anodized aluminium and/or moulded plastic. The diffusers shall be made of acrylic perspex.

Q.5.3.8	All the lighting fixtures shall be suitable for local climatic conditions.

Q.5.4	Installation Instructions

Q.5.4.1	The light fitting shall be installed according to manufacturers’ recommendations or as approved by the Employer.

Q.5.4.2	Flexible connecting wires from outlet box to the fixture shall be provided by the contractor; connector made of porcelain or thermoplastic material shall be provided and installed in the outlet boxes for connecting flexible wires to the point wires.

Q.5.4.3	Outlet boxes or any openings in the ceilings and walls shall be covered with appropriately fabricated accessories to provide an architectural entity to conceal them.

Q.5.5	Main L.T. Switchboard (Substation Panel)

.5.5.1	The L.T. switchboard shall be indoor type, free standing, free supporting, floor mounted, totally enclosed, sheet clad, dust and suitable for operation on 3 phase 4 wire system, 415 v , 50 Hz, AC supply.

Q.5.5.2	The board shall be suitable for installation back to the wall and capable of front attendance. The switch board shall be designed to suit service conditions and ensure security and safety during operation, inspection, operation, cleaning and maintenance.

Q.5.5.3	The switchboard shall be designed and tested to IEC recommendations. Each panel shall withstand strain of 2000 volts insulation level for one minute power frequency test.

Q.5.5.4	The L.T. switch board shall consist of the following:

STELCO incoming cable.

KWh meters (To be approved and checked by URA)

Outgoing distribution feeders.

Q.5.6	Distribution Feeder Panel

Q.5.6.1	Single line diagram of the L.T. switch board shall be approved by the employer and URA before placing order for the switch board.

Q.5.7	Earthing

Q.5.7.1	The switchboard shall be effectively earth by means of a copper strip of 25mm x 3mm (1” x 1/8”) cross ‐section bolted to connections near the bottom of the switchboard.

Q.5.8	Accessories

Q.5.8.1	Designations labels, lifting lugs, foundation bolts, interconnecting nuts blots, and washers, thimbles, lugs, levelling shims cable glands and/or cable end box for all the sizes of incoming and outgoing cable shall be supplied with the switchboard.

Q.6	Testing

Q.6.1 The following tests shall be conducted on each completed switchboard.

Q.6.1.1	Type Tests

(a)	Temperature rise test

(b)	Mechanical endurance test

Making/Breaking Capacity test

Q.6.1.2	Routing Test High Voltage test

Q.6.2 The Switchboard shall be approved by URA. Preference shall, however, be given to Switchboards fabricated from all components manufactured by only one manufacturer.

Q.7	Installation Instruction

Q.7.1	All labour, equipments, tools and plants required to complete the installation shall be provided by the contractor. The Switchboard shall be fixed firmly on the floor in perfect line, plumb and level position.

Q.7.2	All incoming and outgoing cable connections shall be made from the bottom including Earth connections.

Q.8	Distribution Board

Q.8.1	The distribution boards shall be free standing, cubical type or wall mounting type suitable for recessed mounting. Each distribution board (d.b.) shall be suitable for a tropical climate fully dust and vermin proof and liquid repellent.

Q.9	Telephone System

Q.9.1	General

Q.9.1.1	The design drawings do not show conduit routes and depict only the position of various telephone outlets. All the planning for the conduit routes shall be carried out well in advance of the actual execution of work, by the Contractor to the satisfaction of the Employer. For this purpose, the Contractor shall prepare shop drawings and obtain prior approval of the Employer. Three prints of each shop drawings shall be submitted for obtaining approval before commencement of work.

Q.9.1.2	No piece of work shall be allowed to be executed on site without the availability of these approved shop drawings. Time required for the preparation and approval of shop drawings shall be considered to have been included in the total time allowed for the completion of the work.

Q.9.1.3	The contractor shall furnish and install the type of Telephone outlets approved by Dhiraagu/Ooredoo. All the floor mounted telephone boxes shall be concealed in a PVC box with openable cover for easy access.

Q.9.1.4	Both ends of each set of conductors shall be properly identified with durable tags with the same identifications of both ends, at the outlet and the telephone terminal cabinets to facilitate the installations of the telephone instrument in the future and for trouble shouting purposes. Cable used shall be twisted and shielded 3 cables in the office area and the rest as shown in the drawing.

Q.10	Lift

Q.10.1	Lift shall be installed as per detail drawings and mentioned in BOQ. All the accessories and methods shall be accordance with manufacturers’ specifications.

Brand and details: - Refer detail drawing of lift

Q.10.2	Fixing shall be executed by the qualified personnel of this trade.

Environment, Health and Safety Management Requirement

- Anticipated Environmental Impacts

- Construction Phase

- Impacts on the Physical Environment

Air: Construction of the buildings at HAC may result in some localised dust from machinery and earthworks (excavation for foundations). However, there are few offices and sensitive receptors around the site, who may be affected from the project work.  Standard controls are (i) In all cases equipment needs to be in good condition and well maintained to avoid emitting black smoke. Use of equipment in good condition; (ii) correct engine maintenance; (iii) contract provisions allowing the Employer to ban vehicles emitting black smoke; (iv) effective community liaison and grievance procedures, so that polluting vehicles can be reported, identified and sanctioned; and (v) spray water on approach road to the construction site; (vi) Cover the vehicle transporting construction material; (vii) Cover the work area by barricading to block visuals of working area by placing at least 3m high screen (colored CGI sheet, green net); (viii) keep construction aggregates covered by plastic cover or keep them wet so that dust is not generated; (ix) routinely measure dust level at source and receptors  and (x) consult with the stakeholders including the sensitive receptors around the project affected area and arrange working schedule and mitigation measures in common agreement so as not to impact their regular operation.

Noise: the HAC site is in a quiet location. Background levels will be established at the start of construction. Noise will be generated by all construction vehicles and by some stationary construction equipment such as generators and cement mixers. Standard controls are (i) use of effective silencers on all vehicles and machinery; (ii) contract provisions allowing the Employer to ban machinery causing excess noise; (iii) limits on working hours; and (iv) effective community liaison and grievance procedures, so that excessively noisy vehicles and equipment can be reported, identified and sanctioned. Regular noise level measurement at source and receptor locations when construction work is going on and heavy plant and machinery are in operation. In this regard, consult with the stakeholders including the sensitive receptors around the project affected area under overall consultation program and arrange working schedule and mitigation measures in common agreement so as not to impact their regular operation.

Vibration: the civil works will not involve vibration-inducing activities such as blasting or pile-driving. If any structures close to roads used by construction traffic are considered fragile, a condition survey should be carried out as early as possible. If any problem occurs or is perceived, it can be reported and followed up through the standard measure of an effective community liaison and grievance procedure.

Surface water and groundwater: work on the HAC facility will not affect surface water because this does not exist in the planned location. Appropriate surface drainage will be provided along the various short lengths of paved roads required to service this facility. No impacts on groundwater are anticipated from construction. In all cases, arrangements shall be made so that no sediment shall flow into open drainage or waterbodies prior to keeping them in tank as sediment trap and clean the tank regularly. Drainage around fuel and chemical storage area shall also have collection tank and no surface water flow shall carry the contaminants in surface waterbody and groundwater and the collected oil and chemicals shall be professionally and appropriately disposed following accepted standards.

Soils, erosion and sedimentation: all the construction works are small scale and will take place on essentially flat surfaces. There is little possibility of erosion in the form of gullying. However, if heavy rain falls on openly exposed surfaces such as trenching for foundations, there may be a small overflow of sediment-laden runoff. Any such impact will be very short-term and localized. Mitigation includes sequencing works so as to minimize the area of ground exposed and constructing silt-traps at the lower edge of work sites so as to detain runoff and promote settlement.

Site access: the construction site is accessible from existing paved roads.

Existing services: local consultations and fieldwork have identified existing services within and adjacent to the HAC compound that might be affected by the works. The existence and exact locations of all services will be checked during detailed design through coordination with service providers. Any temporary disruption should be minimized. Prior inform the affected area and provide alternative arrangements for undisrupted services.

Construction materials: the HAC facility construction will have very small requirements for bulk construction materials (primarily sand and gravel) The site is easily accessible on paved roads from the commercial harbor on the west side of the island.

Waste management, pollution and site housekeeping: all construction operations generate a variety of solid and liquid wastes, ranging from inert substances such as rubble and bricks through wood waste from formwork, plastics, metal in various forms, workshop wastes, wastewater from worker accommodation, staff offices and site sanitation, office waste, and kitchen wastes if there are canteens. For the project, the most important category of waste will be used engine oils and filters from vehicle maintenance, because these have the potential to create both soil and water contamination. The physical works are not expected to generate any other wastes in any significant quantities. Waste management, pollution prevention and site housekeeping will be ensured by standard provisions which will require the contractor to develop and implement a site waste management plan. This will cover identification of all waste types; separation of all wastes into different waste streams; collection and storage of wastes; disposal of all wastes by approved methods in the hierarchy re-use, recycle, safe disposal; record keeping; specific requirements in relation bulk fuel storage and vehicle maintenance, including by subcontractors on the site; emergency response in case of spills or soil contamination; and worker awareness and training.

- Impacts on the Biological Environment

No important ecological habitats or environmentally sensitive areas have been identified in the vicinity of the location for the HAC multipurpose facility. However, fruit bats were observed to be roosting in the trees in HAC premises. Hence, proper care shall be taken not to disturb or harass them, and be sensitive during their daily activities, breeding and nursing season.

- Impacts on the Social Environment

Construction traffic and disturbance: construction of the new HAC facilities will be on the existing HAC site and - subject to limits on working hours and control of traffic movements to and from the nearby harbor -, disturbance should be minimal.

Public health: apart from possible road traffic accidents, no impacts on public health are foreseen from the project activities. There may be some interaction between local residents and the contractors’ staff and workers, but the number of workers will be small and Maldives are socially conservative and alcohol-free. Prevention of STD transmission is a public health issue, and will be managed as a specific item in the contractor’s induction health and safety, and environmental safeguards training for all staff and workers.

Occupational health and safety (OHS): all construction work has health and safety risks. The project works will involve working at a height, in confined spaces, with electricity, and with heavy machinery. There will also be the usual minor risks associated with vehicle maintenance and small-scale construction work and in the labor camps. All these risks can be mitigated by standard measures, including (i) using equipment in good condition, and properly maintaining it; (ii) ensuring all equipment operators are fully qualified, and also experienced; (iii) including a full range of health and safety requirements in the tender documents, including the need for at least one site engineer to have health and safety qualifications; first aid equipment and a minimum number of qualified first aiders amongst site workers; risk assessment procedures; and procedures in case of accidents; (iv) a requirement for the contractor to develop and implement a health and safety plan (including community health and safety); (v) ensuring that the contractor provides health insurance for all staff and workers as well as complying with relevant Maldivian regulations on worker health and safety;  (vi) including OHS expertise in the supervision PIC; and (viii) recruitment letter, work permit number.

Land, compensation and resettlement: no land acquisition or resettlement is currently foreseen. Activities will be undertaken on Government-owned land. The current HAC buildings may be temporarily inaccessible at certain phases of the work, but full access will be restored upon completion of the construction. Obstruction to farming plots should be avoided.

Heritage: there is no physical cultural heritage feature at the HAC site.

- Operational phase

Operation of the upgraded HAC facility will not involve any unusual activities beyond the normal requirements for management of laboratory chemicals and the use of small quantities of artificial fertilizers in agricultural trials.

- Construction Contract Environmental Safeguards Clauses

  - SEMP

Within 28 days from the date of the Letter of Acceptance, the Contractor shall submit a detailed Site-Specific Environmental Management Plan (SEMP), including final health and safety plan (OHS Plan) and other plans as required for the Project Manager’s no objection showing how it intends to comply with environmental, health and safety laws and regulations and other specific requirements prescribed in the Contract, addressing all the monitoring and mitigation measures set forth in the final or updated Environmental Management Plan (“EMP”) of the project as well as related Supplementary Information attached in Section 6 - Employer’s Requirements.

The following environmental provisions will be included and operationalized in the Site-Specific Environmental Management Plan (SEMP) to be produced by the successful bidder to provide the required construction-phase mitigation and monitoring measures.

  - Commencement

The Contractor shall not commence any “Physical Works” including site establishment and tree or vegetation clearance until (i) written confirmation is received from the Employer that all project financier clearances have been obtained, (ii) the Contractor has obtained all required environment, health and safety (labor) permits or licenses, (iii) the Contractor has undertaken and reported to the Employer on the findings of all applicable pre-construction environmental survey work and baseline environmental monitoring requirements as set out in the definitive version of the EMP, (iv) the Contractor has incorporated all the applicable requirements of the definitive version of the EMP into their detailed designs; (v) the Contractor has submitted the detailed designs and all environmental, health and safety sub-plans required by the definitive version of the EMP and received written confirmation from the Employer of their approval. Such confirmation of no objection by the Project Manager shall not relieve the Contractor of any of his obligations or responsibilities under the Contract.

  - Staff

  - EHS Officer

The Contractor shall appoint at least one fulltime and appropriately qualified and experienced Environmental safeguards Officer (SEO) and a Health and Safety Officer (“OHS Officer”) to supervise and monitor SEMP and occupational health and safety plan implementation on a day-to-day basis for the duration of “Physical Works” under the contract and also act as a grievance redress mechanism focal point to keep affected persons informed of works and be available to receive, document, and deal with any grievances at the project site level. The EHS skills shall be split across two persons (Environment; Health and Safety) subject to the Employer’s approval.

  - EHS awareness and training

The Contractor shall ensure that all its design and construction staff, as well as the staff of subcontractors and third parties employed by them, regardless of whether they are formally or informally contracted, understand their responsibilities to implement the SEMP with appropriate environment; health and safety (EHS) training being provided by the Contractor including construction-site EHS inductions, emergency procedure drills, monthly refresher trainings, and daily toolbox talks. The Contractor shall ensure all construction staff, including the staff of subcontractors and third-parties employed by them, have attended the necessary specialized, task-specific training and are medically fit to perform their role.

  - Forced and child labor

The Contractor will ensure no forced or child labor or persons under 18 are employed on the project.

  - Labor welfare

The Contractor is to ensure sufficient sanitation and welfare facilities (with all wastewater discharges connected to a sewerage system or bio septic tank with soak pit or self-contained portable bio-toilets) are available at each construction site and at any worker camps or accommodation provided by them or their subcontractors with a ratio of at least 1 toilet per every 6 persons. Open defecation and urination are prohibited. Use of pit latrines is prohibited, and no direct discharge of untreated wastewater to surface water or groundwater is allowed.

The contractor is to ensure any worker camps or accommodation provided by them or their subcontractors complies with all national and state requirements as well as the ILO guidelines on worker accommodation. No such accommodation will be permitted at the worksites or within the airport perimeters, except for the services for any necessary overnight security personnel.

If a local water source is to be used as drinking water supply it must be licensed for abstraction and be subject to adequate treatment to enable WHO drinking water standards to be maintained, the quality of raw and treated water to be tested regularly to confirm drinking water standards are met.

  - Environmental Protection and Control of Pollution

- General

The Contractor shall observe and comply with all National Laws, Government Regulations, Presidential Decrees, and ADB policies.

- Materials

The Contractor will ensure no CFCs, no PCBs and no asbestos-containing materials are used in the project. Any transformers purchased shall be accompanied by a letter from the manufacturer stating that they are guaranteed PCB-free and labelled as PCB-free before installation. Other materials and equipment purchased shall be accompanied by a letter from the supplier or manufacturer saying they are guaranteed asbestos-free.

Sand and gravel will only be obtained from existing approved sources with environmental clearance and all sources of water for construction will have an abstraction license. Records will be kept of the volume of all materials used and their source, including certificates from the suppliers regarding no child labor are used.

- Waste

As part of the SEMP, the Contractor shall establish and implement a waste management plan based on the principles of reduction, re-use, recycling and safe disposal in accordance with the requirements of the Employer and national regulations. Waste shall be collected and separated on site for recycling or disposal recycling through reputable, legitimate, licensed third parties and/or to suitably licensed and engineered waste management facilities. The Contractor shall keep a record of waste volumes and disposal pathways for monthly reporting including the relevant permits and/or waste destruction certificates.

- Pollution of watercourses and streams

The emission of polluting liquids or other waste into drains, water courses, or groundwater shall not be permitted.

No concrete or cement washings from the works or drainage from the Contractor's concrete batching and mixing areas, asphalt (hot mix) plants, or other manufacturing or production facilities shall be allowed to discharge into streams or drains without passing through an adequate system of settling ponds.

Storage of fuels, fueling and maintenance of plant and vehicles, etc. shall take place only on sites and under conditions that that do not allow spilt fuels to be discharged to water bodies. Fuel storage and fueling areas shall be equipped with adequate protective measures and drainage with collection tank to confine and retain accidental spillages. No drainage from fuel store and plant maintenance depots shall be allowed to be discharged without passing through an adequate arrangement of oil traps and separators.

Spill prevention kits (absorbent pads, absorbent kit, etc.) shall be available at storage areas and other at-risk locations within clearly labelled containers, and staff trained to handle spill in case of such events. Workers handling them shall wear all required protective clothing, gloves, goggles and boot among others, and urgently inform the management and safety staff following the emergency protocol.

Washing of vehicles shall only be done in specially designated and equipped areas with drainage around the area and a settling tank..

Adequate sanitary waste control facilities shall be provided in site offices and workers camps, and sewage waste shall be collected regularly and disposed in accordance with relevant environmental legislation.

The Contractor shall accordingly be responsible for the installation, operation and maintenance of a comprehensive surface drainage system to all areas of the Works. The system shall be constructed such that no discharges of oil, cement, silt or other liquid or solid waste matter can enter the streams and water courses at the site or flow off the site; and it shall have all necessary solid waste and sediment traps, settling ponds, oil separators, etc., required to ensure that pollution of the soil and marine environment does not occur. The Contractor shall be responsible for maintaining the system to the satisfaction of the Employer’s Construction Supervisor and all costs of providing the system shall be deemed to be included in the various rates and lump sum items for the works included in the priced Bill of Quantities.

- Air pollution

The Contractor shall take all necessary steps to minimize air pollution resulting from his/her operations.

Except where stipulated in the Specifications for the disposal of natural vegetation and organic materials from clearing operations, the burning of waste materials for disposal, particularly oil and petroleum wastes, rubber, plastics and similar materials will not be permitted. Such waste material will be collected and disposed at locations approved by the Employer and permitted by the government.

During the performance of the work required under the Contract or of any operations appurtenant thereto, whether on the Project Site or elsewhere, the Contractor shall take all steps necessary, and shall furnish all labor, equipment, materials and means, required to reduce dust nuisance from the Works, and to prevent dust originating from his/her operations from damaging crops, orchards, cultivated fields, and dwellings; or causing a nuisance to persons. The Contractor will be held liable for any damage resulting from dust originating from his operations including on Government roads, rights-of-way or elsewhere.

The emission of dust into the atmosphere shall not be permitted during the manufacture, handling and storage and handling of cement and of concrete aggregates, and the Contractor shall use such methods and equipment as are necessary for the prevention, or the collection and disposal, of dust during such operations. All truckloads of loose materials shall be covered during transportation.

Emission outlets shall be fitted with pollution control devices in compliance with relevant current Government emission control legislation.

The cost of spraying water on haul roads, access roads, government roads, aggregate stockpiles, etc.; or of any other methods of reducing the formation of dust; and the cost of furnishing and applying materials to maintain the works areas, adjacent areas, and roads, in a dustless condition, shall be deemed to be included in the various rates and lump sum items for the works included in the priced Bill of Quantities.

- Noise pollution

The Contractor shall take all necessary precautions to minimize the amount of noise and vibrations coming from construction activities.

The Contractor shall ensure that all plant and equipment is properly maintained in good operating condition, and that noisy construction activities shall be effectively sound reduced by means of silencers, mufflers, acoustic linings or shields, acoustic sheds or screens or other means, to avoid disturbance to any nearby noise sensitive receivers including some Government offices. All plant and equipment shall comply with relevant Government legislation covering sound emissions.

Operation of trucks and heavy vehicles and machinery shall be restricted to the hours of 06:00 to 21:00 or as agreed with affected community.

All necessary measures shall be undertaken to protect schools, hospitals and other adjacent noise sensitive receptors, including the use of noise barriers.

- Damage to property, crops, and vegetation

The Contractor shall limit the movement of his/her employees and equipment within the project area and on adjacent land, including access routes approved by the Employer’s Construction Supervisor, so as to minimize damage to natural vegetation, crops and property, and shall endeavor to avoid any damage to land.

The Contractor shall strictly ensure employees and equipment do not enter any sensitive environmental areas that are demarcated as “no-entry” zones.

The Contractor shall preserve existing trees, plants and other vegetation that are to remain within or adjacent to the Works and shall use every precaution necessary to prevent damage or injury thereto. Trees or shrubs shall only be felled or removed where such impinge directly on the permanent works or necessary temporary works areas; and where such is approved by the Employer’s Construction Supervisor.

On completion of the Works all areas disturbed by the Contractor’s construction activities shall be restored by the Contractor to their original condition, or as may be acceptable to the Employer.

The Contractor shall be responsible directly to the Employer for any excessive or unnecessary damage to crops or lands arising from his/her operations, whether within the project area, on lands adjacent thereto, or adjacent to approved access roads: and deductions will be made from the payment due to the Contractor to cover the cost of such excessive or unnecessary damage, as determined by the Employer.

- Chance finds

In case of a chance find of any potential cultural heritage artefact the Contractor will stop work at that location and immediately inform the Employer to determine the appropriate course of action.

  - Health and Safety

- General

The Contractor shall undertake a health and safety risk assessment and as part of the Site-specific Environmental Management Plan (SEMP), shall develop an Occupational Health and Safety (OHS) Plan, incorporating an emergency preparedness and response plan for approval of the Employer. The Contractor shall take all measures and precautions necessary to ensure the health, safety and welfare of all persons entitled to be on the site and will ensure that works are carried out in a safe and efficient manner. The Contractor shall pay particular attention to those aspects of the work with greatest risk such as but not limited to working in excavations, working at a height, scaffolding and ladders, and working with electricity. The precautions shall include those that, in the opinion of the Employer, are reasonable to prevent unauthorized entry upon the site and to protect members of the public from any activities under the control of the contractor.

- Site supervision

The Contractor will ensure that the active construction site in HAC has adequate health and safety supervision to ensure the health and safety of all workers and local communities. This is to include a suitably qualified and experienced Health and Safety Officer who is at the active construction site fulltime.

- Personal Protective Equipment

The Contractor shall ensure that its construction staff, as well as the staff of subcontractors and third parties employed by them, regardless of whether they are formally or informally contracted, are provided with and wear at all times when they are on the construction site. Adequate Personal Protective Equipment (PPE) appropriate to their role shall be used by the workers. This will include but not be limited to safety helmets, gloves, safety boots, goggles and other eye protectors, hearing protectors, and safety harnesses when working at height.

- First aid

The Contractor shall provide and maintain a suitably equipped and accessible first aid station at each of the work sites and camps when work is in progress. First-aid kits shall contain an adequate supply of unexpired sterilized dressings and bandages with other necessary first aid equipment as per national health and safety (labor) laws and regulations and the EMP requirements, including emergency eyewash facilities. First aid kits shall also be carried by all vehicles used by the Contractor and any subcontractors for the Works.

Free access for workers to a qualified nurse and/or doctor must be provided in relation to any work-related incident.

The Contractor shall ensure that at least one qualified First-Aider is present at the work site at all times when work is in progress. Workers gang leader will be provided first aid training. This person shall be responsible for maintaining the first aid kits and initiating any emergency response to incidents involving injury, in line with the approved emergency response plan in the Health and Safety Plan. Contractor shall establish a medical room in the camp with basic emergency facilities served by a medical staff.

- Worksite sanitation and welfare

Adequate sanitation and welfare facilities for all staff and workers shall be provided at each construction site, including toilets and washrooms with hot and cold running water supplies, soap, sufficient volume of drinking water supply meeting WHO drinking water standards, dedicated rest/eating area with table, chairs and fan, first aid box, drinking water and a prayer facility.

- Communicable diseases

The Contractor shall be required to comply with national restrictions for containing the spread of communicable diseases, if any. If any pandemic is declared by WHO, the Contractor shall undertake a pandemic specific health and safety risk assessment and as part of the Health and Safety Plan shall prepare a management plan following national guidance and WHO recommendations. The Contractor shall also carry out communicable disease awareness programs for labor, disseminate information at worksites on the risks of the pandemic and other communicable viral diseases, and provide adequate handwashing facilities and personal protective equipment, as part of health and safety measures for those employed during construction. This will include keeping a few ready-made tents in stock in case of emergency use as a quarantine area.

- Fire

The Contractor shall provide and maintain all necessary temporary fire protection and firefighting facilities during the construction of the Works and shall comply with all national fire safety requirements. These facilities must include in-date, serviced, portable fire extinguishers suitable for fighting the potential hazards on the construction site, camps and in all vehicles used by the Contractor and any subcontractors for the Works.

- Grievance redress mechanism

The Contractor shall ensure that throughout the period of the physical works signage is prominently visible for local communities and workers detailing the contact points and focal persons for the Contractor and Employer Grievance Redress Mechanism. A printed brochure on GRM shall be prepared and distributed to all relevant stakeholders and also uploaded in the project website.

  - Reporting

The Contractor shall establish an operational system for managing and monitoring environmental, health and safety impacts and risks, progress with implementation, and compliance with SEMP/EMP and OHS Plan requirements. The Contractor’s management and environmental safeguards, and health and safety staff will attend monthly progress meetings convened by the Employer at which EHS will be an agenda item, and will prepare monthly reports on SEMP/EMP progress and compliance as a section in the monthly progress reports to the Employer documenting the mitigation measures and monitoring activities carried out, issues encountered including statistics and details of all environmental incidents, near misses and accidents, grievances received, and follow-up actions that were taken (or will be taken) to correct any issues.

The Contractor shall maintain a record of all emissions and spills of liquid, solid and gaseous matter which occur at the site, whether into water courses, streams, on land, or into the air. This record shall be compiled daily and shall include details of date, time and nature of the event, along with details of the remedial and clean-up measures carried out. Copies of these records shall be given to the Employer monthly.

The Contractor shall also maintain a record of any complaints made by any Governmental or Community Organization or by the public, regarding his/her operations and record through project GRM process. This record shall contain the date and time of receipt of the complaint, the name and address of the complainant and the action taken to remedy the situation. Copies of these records shall be submitted to the Employer monthly.

  - Environmental Management Plan

The requirements of this clause and attendant sub-clauses on Environmental Protection and Pollution Control notwithstanding, the Contractor shall observe and comply with all relevant environmental protection and mitigation, monitoring, and reporting requirements in the Environmental Management Plan (EMP) as stipulated in the Particular Conditions of Contract. In the event of any conflict between the foregoing sub-clauses and the environmental protection and mitigation measures and pollution control requirements of the EMP, the EMP shall take precedence.

The Contractor shall prepare and submit to the Employer’s Construction Supervisor a SEMP demonstrating the manner in which the Contractor will comply with the requirements of the foregoing subclauses on Environmental Protection and Pollution Control, the EMP, SEMP, OHS and any other particular environmental mitigation measures as stipulated in the Particular Conditions of Contract or Technical Specifications forming part of the Contract Documents.

The SEMP and OHS Plan shall be submitted within 28 working days of the Contractor receiving the Notice to Proceed with the Works and prior to field mobilization, and shall include the set of management plan listed as part of the EMP (e.g. waste management plan, dust management plan, noise management plan, surface water management plan, emergency management plan, health and safety management plan, or any other management plan as stated in the EMP). Training shall be provided to workers about the appropriate implementation of the SEMP. Contractor’s field  mobilization and construction or rehabilitation works at the sites cannot start until the SEMP is approved by the implementing agency.

  - Payment

Payment for compliance with the EHS provisions of the contract will be made in accordance with the Unit Rates, Lump Sum or Provisional Sum Items included in the Priced Bill of Quantities. Except where otherwise agreed or provided for by the Employer, no other payment will be made for complying with the EHS provisions of the contract. All costs shall be deemed to be included in the prices for the Contractor’s mobilization for construction, and the various rates and lump sum items for the works included in the priced Bill of Quantities.

The Employer may withhold payment amounting to 1% of the interim bills if monthly reports are not submitted within 15 working days of month end and/or in the event that the corrective action plans instructed by the Employer in written notice to correct, which is also requirements in the contractor’s approved SEMP/EMP and OHS Plan  have not been addressed by the Contractor within 30 days of receipt of the letter.

Drawings

Figure 1: Map of current HAC compound and location on Hanimaadhoo island

Figure 2: Drone imagery of the current HAC compound

Figure 3: HAC Zoning Plan (1)

Figure 4: HAC Zoning Plan (2)

Figure 5: Ground Floor Plan

Figure 6: First Floor Plan

Figure 7: Second Floor Plan 
Figure 8: Terrace Floor Plan

Figure 9: Roof Plan

Supplementary Information

Regarding Works to Be Procured

## Worksite environmental management, safety and good practice

The Contractor shall comply with all applicable national environmental, health and safety (labor) laws and regulations and the labor standards of the International Labor Organization (ILO).

The Contractor shall comply with the Employer’s and project financier’s safeguard requirements [ADB’s Safeguard Policy Statement (2009)] and the World Bank Group’s General Environmental, Health and Safety Guidelines and the guidelines on Telecommunications.

## IEE and EMPs

The Contractor shall implement the mitigation measures and carry out the monitoring requirements assigned to them as set forth in the initial environmental examination (“IEE”) prepared by the Employer in accordance with the project financier’s safeguard requirements [ADB’s Safeguard Policy Statement (2009)] and the environmental management plan (“EMP”) and environmental monitoring plan (“EMonP”) and any subsequent updates to them. Mitigation and monitoring plans extracted from the EMP are attached in Annexes A and B to this section. These may be updated and the definitive version of the EMP to be complied with by the Contractor will be that disclosed on the Employer’s and the project financier’s [ADB] website. The Contractor shall also implement the mitigation measures and carry out the monitoring requirements assigned to them as set forth in the environmental management plan (“EMP”) prepared for the HAC facility construction by the Employer in accordance with the Environmental Protection Agency’s requirements and shall comply with any conditions attached by the Agency to the environmental permit for the works. The EMP prepared under the environmental impact assessment conducted in accordance with the national environmental requirements and legal provisions of the Government of Maldives will be attached as Annex C (when available); and the EPA’s conditions to the approval of EMP will be attached as Annex D (when available] In case of any inconsistency between the requirements of the two EMPs, the Contractor will follow the Employer’s instructions.

The IEE includes a template for a Site-Specific Environmental Plan (SEMP), attached as Annex E to this section.

The Contractor shall ensure mitigation measures are implemented at construction sites as well as at any workers camps or accommodation provided by them or their subcontractors. The Contractor shall allocate the budget and staff resources required to ensure that such requirements are met, and implement any corrective actions required by the Employer. The Contractor shall cover the cost of corrective action required in response to non-compliance with the definitive version of the EMPs or property damage caused by the Contractor, subcontractors and third parties employed by them.

Personnel Requirements

Using Form PER - 1 and PER - 2 in Section 4 (Bidding Forms), the Bidder must demonstrate that it has personnel who meet the following requirements:

| No. | Position | Total Work Experience [years] | Experience In Similar Work [years] |
| --- | --- | --- | --- |
| 1 | Project Manager - (Bachelor’s degree in project management/Engineering or related field) - Must have reasonably good experience in engineering projects management. | 7 | Experience in similar works is not mandatory |
| Design team |  |  |  |
| 2 | Architect - (Bachelor’s Degree in Architecture) | 7 | 7 |
| 3 | Civil/ Structural Engineer - (Bachelor’s Degree in Civil/Structural Engineering) | 7 | 7 |
| 4 | Building Services Engineer - (Bachelor’s Degree in electrical/mechanical engineering with good engineering knowledge of building services) | 7 | 7 |
| Construction team |  |  |  |
| 5 | Project Engineer (Bachelor’s Degree in Civil engineering) | 5 | 3 |
| 6 | Site Supervisor (Diploma in related field) | 3 | 2 |
| 7 | Quantity Surveyor (Bachelor’s Degree in Quantity Surveying) | 5 | 3 |
| 8 | Building Services Engineer (Bachelor’s Degree in electrical/mechanical engineering with good engineering knowledge of building services) | 7 | 7 |

Using Form EXP-6 in Section 4 (Bidding Forms), the Bidder must demonstrate that it has EHS personnel who meet the following requirements:

Key Personnel as determined by the EMP and other safeguard management plans

| Item No. | Position/specialization | Relevant academic qualifications | Minimum years of relevant work experience | Minimum time on-site (%FTE) |
| --- | --- | --- | --- | --- |
| 1 | Environment, Health & Safety (EHS) Specialist* | Minimum BSc in environmental management or environmental science MSc in environmental management Certification in construction projects issued by authorized OHS institutions. | 5 years | 50% |
| 2 |  |  |  |  |
| 3 |  |  |  |  |

* This position may be split across two persons each focusing on environmental safeguards and OHS.

Equipment Requirements

Using Form EQU in Section 4 (Bidding Forms), the Bidder must demonstrate that it has the key equipment listed below:

| No. | Equipment Type and Characteristics | Minimum Number Required |
| --- | --- | --- |
| 1 | Concrete Mixer/ Machine | 1 |
| 2 | Vibrator | 1 |
| 3 | Truck/ Dump Truck | 2 |
| 4 | Excavator/ Barge | 1 |

Annex A: Environmental Management Plan from ADB IEE

| Activity / Topic | Impact | Mitigation Measure | Responsibility |
| --- | --- | --- | --- |
| Planning & Design Phase |  |  |  |
| Impact assessment | If no assessment, potential for significant unmitigated negative impacts | Undertake impact assessment in line with ADB policy and legal requirements of Maldives | PMU |
| Environmental Management Plan (EMP) | If no environmental management plan, identified impacts will not be mitigated | Prepare EMP as part of safeguard assessment Update IEE during detailed design phase PMU Environmental Safeguards Specialist to review updated IEE and submit to ADB for clearance Attach approved, updated IEE with EMP in bidding document and contract Include comprehensive safeguard and safety provisions as suggested by EMP in bidding document and contract agreement | PMU |
| Grievance redress | If no system for submitting and solving complaints, subproject may be delayed and government suffers reputational damage | Design accessible and effective grievance redress mechanism following the GRM evisaged in the project PAM | PMU |
| Environmental supervision and monitoring | If unclear or under-resourced arrangements for environmental monitoring & supervision, implementation of EMP may be sub-standard | Prepare fully resourced requirements for implementing Environmental Monitoring Plan as part of safeguards documentation | PMU |
| Construction Phase |  |  |  |
| Air quality | Excessive dust from earthworks and construction traffic | Dust suppression by watering at sensitive locations when required Cover of sSpray water and cover the construction material loads on trucks during transportation Measure dust (PM10 and PM2.6) by using volume sampler installed at work site | Contractors Compliance supervision by PIU/PMU |
| Noise | Disturbance of residents | Ensure all equipment is properly maintained and silencer used Limit working hours near sensitive receptors in consultation with stakeholders Measure noise levels during work operation near sensitive receptors Consult with affected community on resolving the grievance related to noise nuisance | Contractors Compliance supervision by PIU/PMU |
| Vibration | Damage to structures from passage of heavy trucks | Condition survey of any structures considered fragile close to roads used by haul traffic Use seismometer if risk of vibration is significant Repair/compensation for any damage | Contractors Compliance supervision by PIU/PMU |
| Surface water and sedimentation | Mobilization of sediment from earthworks during rainstorms | Minimize duration of exposed, non-hardened earthworks Silt traps, silt barriers, and settling tank at edge of work sites | Contractors Compliance supervision by PIU/PMU |
| Site access | Temporary reduced access to facilities on roads | Main access will remain open Public notices will be provided on planned works On site vehicle and human movement area demarcation with heavy vehicles having reverse alarm | Contractors Compliance supervision by PIU/PMU |
| Public roads and bridges | Damage to public roads by heavy construction vehicles | Condition survey prior to start of operations Monitor road conditions and repair and maintain as appropriate to keep road in a pliable condition at all times Inform local residents about the work and movement of heavy vehicles. Speed control at settlement areas Keep reverse alarm in heavy vehicles and assistant to support driver | Contractors Compliance supervision by PIU/PMU |
|  | Temporary increase in traffic congestion | Provide temporary bypasses to support traffic flow Issue public notices on planned work schedule and use of diversion | Contractors Compliance supervision by PIU/PMU |
| Existing services | Damage or cutting of existing services, potentially including sewer pipes, internet cables and other services | Check location of all public services and utilities before start of any physical works, and integrate measures around existing utilities | PMU / Contractors |
| Construction materials | Supply from unauthorized sources | Ensure all construction materials (aggregates and rock) are sourced from authorized locations and in accordance with the permit conditions | Contractors Compliance supervision by PIU/PMU |
| Waste management & pollution prevention | Contamination of soil and water by liquid and solid wastes from contractor’s activities | Include standard provisions in the tender documents which will require the contractor to develop and implement a waste management plan for construction and camp sites. This will cover identification of all waste types; separation of all wastes into different waste streams; collection and storage of wastes; disposal of all wastes by approved methods in the hierarchy of re-use, recycle, safe disposal; specific requirements in relation to bulk fuel storage and vehicle maintenance, emergency response in case of spills or soil contamination; and organize worker awareness and training. If service is available, arrange routine waste collection under the municipal waste management system. | PIU/PMU Contractor |
| Ecological impacts | Damage to or removal of established woody vegetation to make way for physical activities (site access) or installations (structures) | Detailed vegetation surveys prior to final design decisions which should avoid or minimize damage to existing vegetation Removal and replanting of mature vegetation, where feasible s compensatory plantation | PIU/PMU / Contractors |
| Construction traffic | Dust, delays to public road users, safety hazards from excessive speed and bad driving, road deterioration | (i) cover truck load by tarpaulin; (ii) limit speed and install reverse alarm system; (iii) spray water at locations having dust nuisance; (iv) training provided to driver; (v) limit working hours/days near sensitive receptors in consultation with affected community; (vi) bidding document provisions to ensure compliance; (vii) community awareness including safety training provided to children/students; (viii) place warning and informative signboards at work areas; (ix) establish an effective community liaison and grievance reporting and redress mechanism. | PIU/PMU/Contractors |
| Public health | Importation of transmissible/communicable diseases by construction workers | Handle communicable diseases following the established regulations and procedures. Prevention of STD transmission and manage as (i) a specific item in the residents’ awareness program, and (ii) a specific item in the contractor’s induction on health, safety and environment training requirements for all staff and workers, and work with local health institutions and use of media to generate awareness. | PIU/PMU/Contractors |
| Occupational health and safety | A variety of risks associated with both small-scale construction and operating heavy machinery | Mitigate these risks by following standard measures, including (i) using equipment in good condition, and properly maintained and having safety arrangements; (ii) ensure all equipment operators are fully qualified and experienced; (iii) include a full range of health and safety requirements in the bidding document, including the need for at least one site engineer to have health and safety qualifications; (iv) keep first aid equipment at work site and camps and a minimum number of qualified first aiders amongst site workers; (v) provide basic knowledge to use first aid to gang leader of workers group; (vi) on-site nurse or health worker and basic medical room in camp for handling accidents and trauma cases in the island without a nearby hospital or health centre; (vii) risk assessment procedures and procedures in case of accidents; (viii) a requirement for the contractors to develop and implement a health and safety plan; and (ix) ensure that the contractors provides health insurance for all staff and workers as well as complying with relevant regulations on worker health and safety of the Government of Maldives. | Contractors Reviewed by PIU/PMU |
| Operation Phase |  |  |  |
| Surface and groundwater | Impacts will be beneficial, by design | Inspect and maintain drains | PIU/PMU / Island City Council |
| Waste | Solid waste from expanded HAC will exceed local waste management capacity | Develop and implement an institutional waste management plan maximizing waste separation, adopt re-use and recycling | HAC Supported by PIU/PMU |

Annex B: Environmental Monitoring Plan from IEE Cleared by ADB

| Topic | Parameter to be Monitored | Location | Method | Frequency | Responsibility |
| --- | --- | --- | --- | --- | --- |
| Construction Phase |  |  |  |  |  |
| Air quality | Airborne dust | Earthworks sites Along all haul routes on unpaved roads | Particulate measurement to establish pre-works background levels Use f volume sampler during active operations Use indoor dust meters | Pre-construction baseline under typical range of traffic, wind and rain Daily Use meters if controls (watering) ineffective or complaints received | Contractors Monitoring by PIU/PSC |
|  | Vehicle and equipment emissions | At fixed and mobile equipment, including trucks | Visual inspection for excessive smoke | Daily | Contractors |
| Noise | Ambient noise | At any pre-identified locations and where complaints received | Noise meter app on smartphone Use Decibel Meter to measure noise levels | Prior to start of excavation work Daily, if noise is considered a problem by local residents | Contractors Monitoring by PIU |
| Vibration | Condition of fragile properties adjacent to access roads | At any pre-identified locations | Visual inspection Use seismo-meter if vibration is critical | Prior to start of haul traffic Weekly or as alerted by property owners | Contractors Monitoring by PIU |
| Surface water quality | Turbidity | Drainage outlets from work sites if problem observed visually | Visually; if required, use turbidity tube Use lab testing as required | During all heavy rain events | Contractors Monitoring by PIU |
| Vegetation / habitat (onshore) | Damage or degradation of established woody vegetation | All areas adjacent to haul routes and work sites | Visual inspection, with time-stamped photos | Prior to start of all works Daily | Contractors Monitoring by PIU |
| Fuel & oil storage & handling | Bulk fuel storage facilities and arrangements Refueling procedures Vehicle maintenance procedures, specifically collection & disposal of engine and hydraulic oils and used oil filters | Contractor’s compound / workshop On-site locations where equipment is refueled (e.g. excavators) | Visual inspection, with time-stamped photos | Weekly | Contractors Monitoring by PIU |
| Waste management & site housekeeping | All aspects of waste management & site housekeeping: separation, storage, disposal | Contractor’s compound / workshop Construction sites | Visual inspection, with checklist, time-stamped photos | Weekly | Contractors Monitoring by PIU |
| Labor camp | Facilities Physical condition Sanitation & hygiene | All labor camps | Visual inspection, with checklist, time-stamped photos | Weekly | Contractors Monitoring by PIU |
| Worker safety & health | Use and condition of PPE Safe working practices First aid equipment Fire extinguishers | All active work sites All vehicles including subcontractors | Visual inspection, with checklist, time-stamped photos, periodic stock check | Informally, every day Formally, with checklist, every week | Contractors Monitoring by PIU |
|  | Transmissible diseases | Staff and workers, local community | Health records | Monthly | Contractors, in liaison with local health authorities |
| Community safety & health | Disturbance, accidents, health impacts from construction activities | Next to and near all work sites and access routes | Reports from local health authorities Liaison with local government Complaints through GRM | Continuous | Contractors Risk monitoring by PIU |
| Construction materials | Condition of stockpiles, cover, compliance with operating plans and conditions | Stockpiles used to supply construction aggregates | Visual inspection, with checklist, time-stamped photos | Prior to first use of stockpile Weekly during use | Contractors Monitoring by PIU |
| Construction traffic | Driver qualifications Driver training & awareness Vehicle condition Correct loading and sheeting Driving behavior Assistant to driver as flag-person | Site access tracks Public roads | Check of documents Check of attendance at induction / safety training Vehicle inspection and regular maintenance Visual observation Review of grievances | Before start of work for the subproject Daily | Contractors Monitoring by PIU |
| Public roads | Restricted use, temporary closure | Haul routes, work sites | Record of times and locations Place safety signages and barriers as required | Every time restrictions needed | Contractors Monitoring by PIU |
|  | Condition | Routes used by heavy construction traffic | Visual inspection | Prior to start of haul traffic Weekly | Contractors Monitoring by PIU |
| Community liaison | Existence of community liaison mechanism and plan Effectiveness of liaison and grievance redress mechanism | All communities affected by construction | Check of documentation Regular meetings with designated community contact persons Review of grievances & their resolution | Prior to start of operations Weekly Continuous review of grievances | PIU/PMU Contractors |
| Site clean-up | Condition of all locations and facilities used by contractor | All work sites All site access tracks All roads and bridges used by heavy construction traffic Contractors’ camps, compounds and workshops | Visual inspection with time-stamped photos | Prior to start of operations (condition survey) After site clean-up | Contractors Monitoring by PIU |
| Operation Phase |  |  |  |  |  |
| Drains | Condition and performance | Outfalls and inspection pits All open drainage facilities, e.g. bioswale | Visual inspection with time-stamped photos | After all major rainfall events | Local government |
| Groundwater | Depth to water table Water quality (salinity) at different depths | Representative wells and boreholes | Electrical conductivity (EC) of samples from different depths | Monthly | Local government |

Annex C: EMP from Environmental Assessment from the Government of Maldives

The Environmental Impact Assessment (EIA) screening for the project has already been completed by the Environmental Regulatory Authority (ERA). Based on the outcome of the screening, an EIA will be prepared to address the identified environmental management requirements.

Annex D: EPA EMP Approval Conditions

Once the EIA is reviewed and approved by the ERA, all resulting conditions and requirements must be incorporated into the Site Environmental Management Plan (SEMP). The SEMP shall be developed by the Contractor and submitted for approval before the commencement of site mobilization.

Annex E: Template for Site-Specific Environmental Management Plan

| Topic/ Potential Impact | Mitigation, Management and Enhancement Measures | Means of Verification | Responsibility | Monitoring Procedure |
| --- | --- | --- | --- | --- |
| - Environmental, Social, Health and Safety Management |  |  |  |  |
| 1. Responsibilities and liabilities | Adjust this SEMP to the specific project, define the frequency of the monitoring procedure and share it with PMU and ADB. Identify if further Management Plans have to be prepared. | Final project specific SEMP | Contractor Approval by PIU/PMU | Monitor and record the management of the environmental safeguards, and Occupational Health and Safety (OHS) compliance though visual monitoring, review of work permit and daily work records, record of injuries/accidents, regular consultation with workers etc. |
|  | Ensure that all workers, suppliers and subcontractors are familiar with and comply with the requirements and specifications of this SEMP. | Induction performed and recorded | Contractor Verification by PIU | Document and report the induction event (participants, information shared etc.) through QPR and SEMR |
| 2. ESHS Management | Define all organizational and technical provisions to satisfy and follow up the specifications of the SEMP (e.g. resources, documentation) | Requirements for the environment and health & safety management are defined | Contractor Assured by PIU | Once, prior to finalization of project specific SEMP |
|  | Make sure that the Site Manager/Supervisor has experience in dealing with ESHS management. Give authority for stopping of works if non-conformities are identified. Appoint qualified EHS officer. Communicate with the relevant authorities on the responsibilities of the ESHS staff. | The Site Manager has experience in dealing with environment and health & safety issues in similar projects in the past; Appoint qualified safeguards and OHS officer on site | Contractor Assured by PIU | Once, prior to selection of Site Manager. Weekly inspection reports. |
| 3. Code of Conduct | Establish a Code of Conduct taking into consideration safety rules, substance abuse, environmental sensitivity, communicable diseases, gender issues (sexual harassment), respect for local beliefs and customs, community interactions etc. | Workers Code of Conduct is in place Staff and workers are oriented on strictly following the issues | Contractor Cleared by PIU | Punishable or misconduct behaviors are reported and communicated Check grievance records |
| 4. ESHS Training | Provide Health and Safety (H&S) Induction and Training to the workforce regarding H&S risks and mitigation measures (including indirect workers) tailored to project scope. | Training performed and recorded | Contractor Reviewed by PIU | Check training records |
|  | Awareness raising regarding community health and safety to the local communities through campaigns (e.g. schools) | Awareness raising campaigns performed and recorded | PIU/PMU Contractor | Check information material produced |
| - Protection of the Environment and People |  |  |  |  |
| 5. Protection of adjacent areas | Restrict excavation activities during periods of intense rainfall. Use silt traps to reduce the risk of sediment moving off site. | No excavation during intense rainfall and when site is slippery Use shoring in trench above 2m height | Contractor Monitored and assured by PIU | Site inspection |
|  | Create cutoff ditches to prevent water from entering excavations, if needed. | No surface water entering excavations | Contractor | Random site inspection |
|  | Ensure that means of protection are in place to avoid or minimize adverse effects on vegetation, soils, groundwater and surface water, biodiversity, natural drainage and the water quality in areas within the works area. Construction methods to minimize impacts to the extent possible. | No impacts identified in the adjacent environment | Contractor Monitored by PIU | Regular (e.g. monthly) monitoring of adjacent natural resources. |
|  | Ensure that work site boundaries and limits are in accordance with plans agreed in advance. All construction activities should be carried out within boundaries. | Site boundaries and limits on site Demarcate separate path for use by vehicles and people inside camp | Contractor Assured by PIU | Site inspections |
|  | Ensure no works in wetland areas. | Avoid work in the site in wetland areas | Contractor Assured by PIU | Once during site selection |
| 6. Selection of stockpile sites and access road | Select specific, limited areas for stockpiling and construction of access tracks, and obtain full authorization before construction. | Designated areas selected Stockpile properly drained and covered by tarpaulin | Contractor Assured by PIU | Once during site selection Weekly inspection reports |
| 7. Pollution prevention | Ensure all works carried out minimize pollution risk (e.g. liquid effluents; air emissions; noise and vibration management; vehicle and equipment maintenance and selection; fuel, oil and chemical storage and handling.) including the whole duration of the project. | No pollution, spillage, foul smell identified | Contractor | Weekly inspection reports |
| 8. Effluents | Ensure appropriate containment and storage of construction wastewater, including sanitary water. No untreated effluent is discharged. | only treated wastewater discharge Use settling tank before discharge of effluent | Contractor Monitored by PIU | Regular site inspection |
| 9. Atmospheric emissions and dust | Use vehicles in good condition. Test for emission control where applicable. | Technical Specification Sheet Observation Exhaust testing | Contractor Assured by PIU | Prior to commencement of works and each time new equipment/vehicle is used at the site. Test exhaust twice a year |
|  | Ensure vehicles are switched off when not in use. Sensitize drivers. | Engines switched off | Contractor | Site inspection |
|  | Best practice to ensure minimization of dust emissions (e.g. proper stockpiling, watering etc.) during dry and windy conditions and transportation. | Spray water at required frequency, no dust emissions are observed, no workers’ or residents’ grievances on dust nuisance | Contractor Monitored by PIU | Regular site inspection |
|  | Ensure speed limits on site. Sensitize drivers. | Speed signs installed; residents’ grievances | Contractor Periodic check by PIU | Random site inspection |
| 10. Noise and vibration | Avoid operations and vehicle movements at night. Sensitize drivers. | No work conducted between 10pm and 7 am near sensitive receptors, no grievances received | Contractor Checked by PIU | Random measurement near sensitive receptors, review of filed grievances, interaction with community |
|  | Set traffic speed limits. Sensitize drivers. | Place speed limit signs Training given to drivers Tale action on defaulters | Contractor Assured by PIU | Random site inspection, Review of training records |
|  | Locate stationary equipment (such as power generators) as far as possible from nearby receptors (e.g. worker resting areas, populated areas and environmentally sensitive areas). | Distances between equipment and receptors are maintained Cover the equipment to dampen noise | Contractor Monitored by PIU | Once prior to commencement of works Monitor noise levels in case equipment is used |
| 11. Waste | Identify waste management facilities and waste management contractors. Ensure disposal through waste contractors licensed for treatment/removal/recycling of each of the waste types. | Waste management through licensed contractors, if feasible Waste Management Plan | Contractor | Inspect waste management facilities Proof of contractors’ certifications |
|  | Ensure that all wastes produced are properly collected, segregated, stored, transported and re-used, recycled or disposed of. | Waste collection areas identified, keep waste inventories | Contractor | Random site inspection, Review of waste inventories |
|  | Minimize waste production to the extent possible. | Records of waste production are kept | Contractor | Monitor (e.g. monthly) the amount of waste produced |
|  | Document all waste related operations (type of wastes, quantities produced etc.). | Storage, transport and treatment of waste is documented | Contractor | Document inspection |
|  | Appropriate and safe storage of fuels, construction materials, wastes and any other potentially polluting materials. | Safe storage of materials, Spill remediation equipment in place | Contractor | Random site inspection Weekly site inspection reports |
| 12. Vegetation clearing | Limit vegetation clearing to areas within the site boundary where it is strictly necessary. | Minimal vegetation clearing | Contractor | Random site inspection |
|  | Describe the methods of vegetation clearance. | No use of fires or chemicals on site | Contractor | Site inspection during site clearance |
|  | Avoid clearing mature trees and any protected species. | No mature trees cleared Compensatory plantation record | Contractor | Visual inspection |
|  | Avoid off-road vehicle traffic. Use existing roads. Sensitize drivers | No off-road traffic | Contractor | Random site inspection |
| 13. Biodiversity | Avoid disturbance of natural habitats. | Disturbance of natural habitats avoided | Contractor | Inspection of construction practices. |
| 14. Erosion and sediment transport | Ensure preventive erosion control measures are applied (e.g. plan to retain grass and other vegetation, use of natural contours for roads and drainage networks, create drainage channels). | Preventive erosion control measures in place Sediment trap constructed | Contractor | Random site inspection to check if measures where applied |
|  | Ensure appropriate storage of topsoil. After construction, use topsoil as backfill for restoration of the area according to detailed Site Restoration Plan. | Site Restoration Plan prepared Topsoil stored and re-used | Contractor | Monitor if topsoil is properly stored |
|  | Optimal stockpiling. | Stockpile height limited and free from erosion and contamination Stockpile covered | Contractor | Random site inspection |
| 15. Site rehabilitation | Ensure revegetation and restoration of cleared areas after construction. | Revegetation completed | Contractor/ PIU | One-time inspection after construction |
|  | Ensure that rehabilitated areas do not pose health and safety risks (e.g. holes, ponds). | Reinstatement completed | Contractor/ PIU | One-time inspection after construction |
|  | Reinstate construction working area after construction activities are completed. | Reinstatement completed | Contractor/ PIU | One-time inspection after construction |
|  | Rehabilitate storage sites and access roads, where applicable. | Rehabilitation completed | Contractor Monitoring by PIU | One-time inspection after construction |
| - Health and Safety |  |  |  |  |
| 16. Health and Safety Plan | Develop a Health and Safety Plan. | OHS Plan in place Staff and workers oriented | Contractor Assured by PIU | Monitoring of Contractor’s measures |
| 17. Accident reporting procedure | Ensure all H&S related incidents (e.g. observations, accidents) on site are recorded and followed up effectively. | Incident recording and reporting procedure in place Safeguards and OHS staff of PSC and Contractor, and EA’s staff oriented | Contractor | Check incident/accident records Check orientation record |
| 18. Personal protective equipment | Ensure the provision of Personal Protective Equipment (PPE) for workers (hardhats, masks, safety glasses, safety boots etc. as appropriate). | PPE used by everyone on site | Contractor/ Monitoring by PIU | Random site inspection Contractors PPE stock use and stock log review |
| 19. Emergency scenarios prevention | Ensure immediate cleaning of any spills and remediation of contaminated areas after construction. | Workers trained | Contractor/ PIU | Random site inspection after spill events One-time inspection after construction |
|  | Provide necessary prevention equipment on site in line with applicable regulations to respond to emergency scenarios, e.g. fire, floods, natural hazards etc. | Prevention equipment in place | Contractor Check by PIU | Regular site inspection Regular assessment of emergency management and rescue preparation |
|  | Maintain high standard in housekeeping on site. Construction materials and light equipment should be stored properly. | Visual verification of good housekeeping on-site | Contractor Assured by PIU | Random site inspection |
| 20. First-aid | Ensure minimum first aid provisions on site. (suitably stocked first-aid kits; at least one person trained in first aid at each work site, respectively an adequate number of first-aid helpers; ensure that staff and workers are informed about first-aid arrangements) | Suitable first aid kits on site Ensure the presence of first aid helpers in all shifts | Contractor | Regular monitoring of first aid kits |
| 21. Access to health care and training | Ensure the workforce has access to primary healthcare on site, providing prescriptions and vaccinations. | Healthcare and medical room with medical staff available on site | Contractor | Random site inspection |
|  | In all cases, determine the location and access routes to the nearest functional hospital, medical clinic or health center and establish an evacuation plan for casualties. | Public medical centers in the proximity of the site. | Contractor | Medical centers in the proximity of the site identified once prior the commencement of works |
| 22. Hygiene, accommodation and food | Ensure provision of H&S and hygienic and sanitary facilities at the site, including shaded welfare areas, showers, changing rooms and potable water. Ensure toilets and changing rooms are separated between male and female employees. | Appropriate H&S and sanitary facilities provided at site | Contractor | Random site inspection |
|  | Ensure the provision of adequate space, supply of water, adequate sewage and garbage disposal system, appropriate protection against heat, cold, damp, fire and disease-carrying animals, adequate sanitary and washing facilities, adequate lighting, and basic medical services, in accordance with all applicable health and safety regulations and norms. | Appropriate conditions for workers on site | Contractor | Random site inspection Grievance mechanism log |
|  | Report any occurrence of any communicable diseases amongst the workforce (Covid-19, STDs, Tuberculosis and Hepatitis B and C). Sensitize workers. | Communicable Diseases Register | Contractor | Review of diseases register and disease prevention program if available. |
| - Local labour and relations with local communities |  |  |  |  |
| 23. Labor conditions | Ensure minimum legal labor standards as per the International Labor Organization regulations (child/forced labor, sexual assault, no discrimination, equal opportunities, working hours, minimum wages) are met. | Grievance Mechanism Records, Training recorded | Contractor PIU/PMU | Inspection reports (also from labor authorities), Review of grievance register and training record |
|  | Ensure that all direct and indirect workers have access to and are aware about the Grievance Mechanism where they can raise workplace relevant complaints anonymously. | Grievance Mechanism in place and grievances recorded | Contractor/ PIU | Review of grievance register |
|  | Ensure all workers have the same rights and are treated equally. | Non-discrimination policy in place | Contractor Assured by PIU | Random site inspection Grievance mechanism log |
| 24. Local recruitment | Ensure local communities are preferred for the supply of goods and services to the subproject and subproject personnel, where appropriate. | Local Procurement and Employment Records | Contractor | Review procurement and employment rules and records |
| 25. Transport | Organize transportation for workers where feasible. | Construction vehicle used | Contractor | None |
| 26. Community interaction | Engage / communicate / inform affected communities. Ensure good communication and consultation with local authorities and communities regarding all activities. | Minutes of Meetings Grievance Mechanism | Contractor / PIU | Review of grievance register Minutes of consultation meetings |
|  | Initiate an efficient Grievance Mechanism to allow potentially affected individuals to raise their concerns. | Grievance Mechanism in place, grievances recorded | Contractor / PIU | Review of grievance register |
|  | Engage with the local community and potential affected households to understand their needs and identify the risk of damage to their livelihoods by subproject activities (e.g. exclusion from pasture land, lack of access to water, slope failure risk during excavation). Should impacts be unavoidable, a timely and fair compensation should be given to all affected persons in accordance with project’s Resettlement Framework (RF). | Minutes of Meetings Grievance Mechanism records. Management Plan for Land Acquisition and Compensation if needed | Contractor/ PIU | Review of grievance register and meeting minutes |
| 27. Damage to people and property | Ensure all contractors implement codes of conduct concerning employment and workforce behavior (including but not limited to safety rules, zero tolerance for substance abuse, environmental sensitivity of the area, dangers of sexually transmissible diseases and HIV/AIDS, gender equality and sexual harassment, respect for the beliefs and customs of the populations and community relations in general). | Workers Code of Conduct Grievance Mechanism records | Contractor Assured by PIU | Worker interviews, Review of grievance log |
|  | Ensure that site areas are provided with appropriate security, fencing, signage and lighting. Use hazard notices / signs / barriers to protect children and other vulnerable people from harm and prevent access to non-workers. | H&S planning of construction site approved, items installed | Contractor Assured by PIU | Random site inspection |
| 28. Land acquisition and land take | Engage with the local community to understand local land ownership and land use. Avoid to the extent possible land take of both formal and informal land owners/land users. If land take is inevitable, no forced eviction should take place. Should impacts be unavoidable, a timely and fair compensation should be given to all affected persons in accordance with project’s Resettlement Framework (RF). Owners should be compensated prior to access to land. | Grievance Mechanism Management Plan for Land Acquisition and Compensation if needed | PIU / Contractor | Once during site selection Review of grievance log Follow up of land acquisition/compensation process |
| 29. Traffic management | Ensure safe driving by subproject personnel (e.g. through training/induction). | Driver Training Records as part of Induction training | Contractor | Review of training records |
|  | Target signage and outreach activities to improve public awareness of traffic changes and potential hazards for high-risk sections of public roads, including near the site and contractor’s compounds. | Warning signs Minutes of Meetings Inform public of work schedule till when road will be closed Place signboards informing diversions | Contractor | Inspection if traffic routes, Review of grievance register |
| 30. Fossils/ Archaeological Chance Finds | Establish specific procedures to manage the protection of archaeological and historical sites, chance finds and fossils. Ensure all finds of physical cultural heritage (e.g. graves, old ceramic, old building fragments) are reported immediately to the relevant authority and avoid excavation in the ultimate neighborhood of a chance find, fence the chance find and await instructions from the competent authority. | Training records Records of chance fiends | Contractor PIU/PMU | Site inspection |

Asian Development Bank, 2022. Inclusive Cities. Urban Area Guidelines. Manila; and Commission for Architecture and the Built Environment, 2006. The Principles of Inclusive Design. (They include you.). London

Labor camps are not anticipated since worker numbers will be small and are likely to be sourced from and/or accommodated in existing facilities in the local community.